BEFORE THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO

Docket No. 97I-198T

_______________________________________________________________________

IN THE MATTER OF THE INVESTIGATION INTO U S WEST COMMUNICATIONS, INC.’S COMPLIANCE WITH § 271(C) OF THE TELECOMMUNICATIONS ACT OF 1996

______________________________________________________________________

AFFIDAVIT OF

MICHAEL J. WEIDENBACH

U S WEST COMMUNICATIONS, INC.

November 30, 1999

NOTICE OF CONFIDENTIALITY: A PORTION OF THIS DOCUMENT HAS BEEN FILED UNDER SEAL.

I.	Identification of Affiant	1

II.	Purpose of Affidavit	1

III.	Executive Summary	2

IV.	Checklist Item 1 -- Interconnection	6

Interconnection Overview	6

The Exchange of Traffic between U S WEST and CLECs	7

Interconnection at Any Technically Feasible Point	8

Standard Interconnection Arrangements Provided by U S WEST	9

The Bona Fide Request (BFR) Process	13

Ordering Interconnection	13

Processes Supporting Interconnection	16

U S WEST Provides CLECs with Interconnection at Least “Equal in Quality”	17

U S WEST Network Trunk Design Standards	19

Interconnection Performance Measures	22

Trunk Blocking Measures	23

Timely Trunk Installation	26

Repair Quality	29

Rate Elements for Interconnection	33

Summary	34

V.	Checklist Item 1 -- Collocation	36

Collocation Overview	36

FCC Collocation Rules:	38

First Report and Order	38

“706” Federal Collocation Rules	39

U S WEST’s Compliance with the FCC Collocation Rules	40

U S WEST Provides Physical and Virtual Collocation	41

U S WEST Collocation Policies and Procedures	43

Three Step Collocation Provisioning Process	47

Collocation Process Flows	49

Collocation Performance Measures	49

Collocation Rate Elements	51

Summary	51

VI.	Checklist Item 13 - Reciprocal Compensation	53

Reciprocal Compensation Overview	53

Call Termination	53

Transport Options Provided By U S WEST	55

Direct Trunked Transport	55

Tandem-Switched Transport	56

Call Transit	59

Rate Elements Associated with Reciprocal Compensation	60

Billing Records, Billing and Payment of Reciprocal Compensation	63

U S WEST Processes Supporting Reciprocal Compensation	67

Summary	68

VII	Checklist Item 3 -- Poles, Ducts, Conduit and Rights-of-Way	69

Overview of Access to Poles, Conduits, Ducts, and Rights-of-Way	69

FCC Rules Governing Nondiscriminatory Access to Poles, Ducts, Conduits, and Rights-Of-Way	71

Requirements for the Prima Facie Case for Checklist Item 3	74

Evaluating Requests for Access to Poles, Conduits, Ducts and Rights-of-Way	75

Nondiscriminatory Access to Information	78

Choice of Workforce	78

Rates for Access to Poles, Conduits, Ducts, and Rights-of-Way	79

Processes Supporting Access to Poles, Ducts, Conduits, and Rights-of-Way	79

Summary	80

VIII.	CONCLUSION	82

INDEX OF EXHIBITS	1

# I.	Identification of Affiant

My name is Michael J. Weidenbach.  I am employed by U S WEST Communications (“U S WEST”) as a Manager in the Wholesale Markets Organization.  My business address is 1801 California, Room 2330, Denver, Colorado, 80202.  My qualifications are provided in Exhibit MJW-01.

# II.	Purpose of Affidavit

The purpose of this affidavit is to demonstrate that U S WEST has satisfied the requirements for interconnection and collocation (checklist item #1), pole, conduit, duct, and right-of-way space leasing (checklist item #3) and reciprocal compensation (checklist item #13) as specified in Section 271 of the Telecommunications Act of 1996 (Act) and various Federal Communications Commission (FCC) orders interpreting the Act.  The requirements of checklist items 1, 3 and 13 of Section 271 are met through U S WEST’s Statement of Generally Available Terms and Conditions (SGAT) and the existing interconnection agreements between U S WEST and CLECs in Colorado and the processes and procedures employed by U S WEST to ensure that U S WEST is meeting the requirements of checklist items 1, 3 and 13.  In addition, U S WEST is currently furnishing these checklist items in the quantities that competitors may reasonably demand and at an acceptable level of quality.

# III.	Executive Summary

Checklist item 1 requires U S WEST to make interconnection available to CLECs on a nondiscriminatory basis.  The FCC has defined the term interconnection as “. . . the linking of two networks for the mutual exchange of traffic.  This term does not include the transport and termination of traffic.”  Interconnection provides the means to connect the U S WEST network with the network of a CLEC for the exchange of traffic.

U S WEST provides several alternative arrangements to facilitate the interconnection of a CLEC network with the U S WEST network, including physical collocation, virtual collocation, mid-span meet arrangements, entrance facilities, and hub locations.  These interconnection arrangements exchange a wide variety of traffic types with CLECs, including local, directory assistance, 911/E911, operator services, wireless traffic, intraLATA toll, and switched access.  In accordance with the FCC’s requirements, U S WEST provides interconnection at the line-side of a local switch, the trunk-side of a local switch, the trunk interconnection points for a tandem switch, central office cross-connection points, signal transfer points, and points of access to unbundled network elements.

U S WEST provides both physical and virtual collocation so that CLECs may place their equipment on U S WEST central office floor space for purposes of connecting to the six points of interconnection established by the FCC.  There are four standard varieties of physical collocation -- caged, shared, cageless, and InterConnection Distribution Frame (ICDF) collocation.

Caged Physical -- Caged physical collocation allows the CLEC to place its equipment within U S WEST’s central office while being surrounded by a cage that provides an increased level of security for the CLEC’s equipment.

Shared Physical --  Under Shared Physical Collocation, one CLEC obtains a Caged Physical Collocation arrangement from U S WEST.  CLECs may share the space, in accordance to terms and conditions agreed to between the two CLECs.

Cageless Physical -- Cageless physical collocation allows the CLEC to place its equipment in the U S WEST central office adjacent to U S WEST or other CLEC equipment, but it is not separated from other central office equipment.

ICDF Collocation  -- ICDF collocation is offered for CLECs that do not require transmission equipment to be placed in the U S WEST Central office, but require access to unbundled network elements for the purpose of combining unbundled network elements.

Virtual Collocation -- Virtual collocation allows the CLEC to turn over equipment to U S WEST for engineering, installation, and maintenance on behalf of the CLEC.  This type of arrangement is used, principally, when there is no space for physical collocation.

Standard collocation arrangements are available at U S WEST wire centers.  U S WEST, through the Bona Fide Request (BFR) process will provide collocation at other U S WEST locations, including adjacent collocation and any collocation method used by one incumbent LEC or mandated by the Colorado Commission.

Checklist item 13 requires U S WEST to negotiate interconnection agreements with CLECs that provide for compensation on a mutual and reciprocal basis for the transport and termination of local traffic.  U S WEST satisfies these requirements through the U S WEST SGAT and the various interconnection agreements between U S WEST and CLECs in Colorado.

Checklist item 3 requires U S WEST to provide nondiscriminatory access to poles, conduits, ducts and rights-of-way on rates terms and conditions that are consistent with Section 224 of the Act.  U S WEST and other telecommunications carriers have entered into broad joint-use agreements for the use of poles, ducts, conduits, and rights-of-way, pursuant to the 1978 Pole Attachment Act.  U S WEST has successfully negotiated such joint use agreements for years.  Thus, U S WEST is highly experienced at providing access to poles, conduits and rights-of-way consistent with Section 224 of the Telecommunications Act of 1996.  U S WEST satisfies the requirements of checklist item 3 and has a concrete and specific legal obligation to provide access as referenced in the U S WEST SGAT and the various interconnection agreements between U S WEST and CLECs in Colorado.

U S WEST has procedures in place for providing interconnection, collocation; access to poles, ducts, conduits, and rights-of-way, and reciprocal compensation in conformance with the requirements of the Act and the FCC’s rules.  As a result, the Colorado Commission should therefore find that U S WEST has satisfied the Act’s checklist requirements for interconnection, collocation; access to poles, ducts, conduits and rights-of-way; and for reciprocal compensation.

# IV.	Checklist Item 1 -- Interconnection

## Interconnection Overview

Interconnection provides the means to connect the U S WEST network with the network of a CLEC for the exchange of traffic.  The traffic that is exchanged may be local, toll, or a variety of specialized traffic, such as directory assistance, operator services, or 911.

The FCC has defined the term interconnection as “. . . the linking of two networks for the mutual exchange of traffic.  This term does not include the transport and termination of traffic.”  Interconnection provides the means to connect the U S WEST network with the network of a CLEC for the exchange of calls.

The Act requires U S WEST to provide “interconnection in accordance with the requirements of Sections 251(c)(2) and 252(d)(1) of the Act.”

These requirements are met through the SGAT and the various interconnection agreements between U S WEST and CLECs in Colorado.  These requirements are also satisfied by the specific procedures U S WEST employs to implement interconnection with CLECs.  In the following sections of my testimony, I discuss each requirement in more detail.  I will also discuss the additional requirements established by the FCC.

## The Exchange of Traffic between U S WEST and CLECs

Section 251(c)(2)(A) imposes a duty upon incumbent LECs to provide "interconnection with the [LEC's] network . . . for the transmission and routing of telephone exchange service and exchange access.”

U S WEST’s SGAT and existing Colorado interconnection agreements specify U S WEST’s legal obligation to provide interconnection.  Section 7.0 of the SGAT describes how U S WEST interconnects U S WEST's network with CLECs’ networks to exchange local traffic.  Section 7.0 of the SGAT includes: Interconnection Facility Options, Methods of Interconnection, Exchange of Traffic, Transport and Termination of Local Traffic, Reciprocal Compensation, Interconnection Facility Options, and Ordering.

Section 7.2 of the SGAT defines a wide variety of traffic types that can be exchanged over these arrangements, including local, directory assistance, 911/E911, operator services, wireless traffic, intraLATA toll, switched access traffic, and transit traffic.

Currently, U S WEST is providing interconnection to 13 CLECs in 5 Colorado metropolitan areas, including Denver, Colorado Springs, Fort Collins, Greeley, and Pueblo.  A total of 486 local interconnection trunk groups, with 80,614 trunks in service between U S WEST and CLECs in Colorado, interconnecting 65 wire centers.  The following list identifies the types and number of trunk groups in service in Colorado through September of 1999, including the equivalent number of DS0 trunks in service:

Trunk			Equivalent Trunks

Groups		In Service

Directory Assistance	2	18

E911	95	290

Local	329	71,837

Operator	16	331

Switched Access	8	1,368

Toll	36	6,770

Total	486	80,614

Confidential Exhibit MJW-C1 also provides specific wire center interconnection detail.

## Interconnection at Any Technically Feasible Point

In addition to specifying the purposes for which carriers may request interconnection, Section 251(c)(2)(B) obligates incumbent LECs to provide interconnection within their networks at any "technically feasible point.”  U S WEST satisfies this requirement by providing CLECs with interconnection at the six minimum points of interconnection defined by the FCC.  U S WEST satisfies any other requests for interconnection through the Bona Fide Request Process (BFR).

The minimum points of interconnection are:

1)	Line-side of a local switch;

2)	Trunk-side of a local switch;

3)	Trunk interconnection points for a tandem switch;

4)	Central Office cross-connection points;

5)	Signal Transfer Points; and

- Points of access to Unbundled Elements.

Section 7.1.1 of the SGAT, for example, creates a specific legal obligation to provide interconnection at these points.

## Standard Interconnection Arrangements Provided by U S WEST

Section 251(c)(6) of the Act imposes on incumbent local exchange carriers the duty to provide collocation so that CLECs may place their equipment on U S WEST central office floor space for purposes of interconnection with U S WEST’s network.  In addition to collocation, the FCC has established other methods of technically feasible interconnection or access to U S WEST’s network, such as mid-span meet arrangements.

In accordance with the Act and FCC’s requirements, U S WEST provides several alternative arrangements that facilitate the interconnection of a CLEC network with U S WEST's network, including collocation, mid-span meet arrangements, entrance facilities and hub locations.  Section 7.1.2 of the SGAT specifies the legal obligation to provide these methods of interconnection.

First, U S WEST provides both physical and virtual collocation so that CLECs may place their equipment on U S WEST central office floor space for purposes of connecting to the six points of interconnection established by the FCC.  The collocation section of this affidavit describes in detail the collocation arrangements provided by U S WEST.

Second, U S WEST provides a mid-span meet interconnection arrangement.  In a mid-span meet interconnection arrangement, U S WEST and a CLEC extend facilities from their respective networks to a common agreed upon point where their facilities meet.  Included below is an illustration of a mid-span meet interconnection arrangement.

Third, U S WEST provides an entrance facility interconnection arrangement.  This arrangement extends a U S WEST provided transport system from the CLEC’s central office to the U S WEST serving wire center.  The length of an entrance facility is limited to the boundary of the serving U S WEST wire center.  U S WEST's private line transport service is also available as an alternative to an entrance facility, when a CLEC uses the private line transport service in conjunction with multiple services.  Included below is an illustration of an entrance facility interconnection arrangement.

Fourth, when a CLEC locates its switch outside the U S WEST local calling area, the Hub Location Point of Interface establishes the CLECs Point of Interface within the local calling area. The Hub Location Point of Interface is typically established at the U S WEST Central Office were multiplexing is performed.  Included below is an illustration of a Hub Location Interconnection arrangement.

U S WEST’s SGAT incorporates the above interconnection alternatives.  Specifically, the SGAT states:

The parties will negotiate the facilities arrangement between their networks.  CLEC shall establish a Point of Interconnection in each U S WEST local calling area where it does business.  The following alternatives are negotiable:  (1) a DS1 or DS3 entrance facility; (2) Virtual or Physical Collocation; (3) negotiated Mid-Span Meet POI facilities; or (4) Hub location.

## The Bona Fide Request (BFR) Process

In addition to the standard interconnection arrangements in the SGAT, a CLEC may request a novel use, or some modification to U S WEST’s standard interconnection arrangements.  A CLEC may request additional points of interconnection through the Bona Fide Request (BFR) process.  The SGAT describes the BFR process as follows:

Any request for Interconnection . . . that is not already available as described herein shall be treated as a Bona Fide Request (BFR).  U S WEST shall use the BFR Process to determine the terms and timetable for providing the requested Interconnection . . ., if available, and the technical feasibility of new/different points of Interconnection.  U S WEST will administer the BFR Process in a non-discriminatory manner.

In Colorado, during 1998 and 1999, U S WEST received six BFR requests for interconnection.  One BFR is pending, two BFRs were fulfilled, one CLEC withdrew its BFR and two BFRs were denied.  For those BFRs that were denied, the CLECs were offered a tariffed alternative.

## Ordering Interconnection

As the first step in ordering interconnection, the CLEC representative and the CLEC’s U S WEST Account Manager establish a joint planning meeting.  This meeting precedes initial interconnection orders.  The CLEC and U S WEST, in the joint planning meeting, ascertain interconnection requirements culminating in an interconnection order.  The CLEC provides its best estimate of the traffic distribution to each U S WEST end office and local tandem office.  The service order and due dates for the initial interconnection orders are established.  Subsequent service intervals and due dates are established in accordance with the guidelines for interconnection trunks contained in the Interconnection and Resale Resource Guide, available on U S WEST’s web site.

The SGAT defines the responsibilities of both parties regarding the ordering process, including the joint planning meeting and the Access Service Request (ASR) process.  The ASR provides the required information as defined by the industry Standard Service Ordering Guidelines (ASOG), in order to establish interconnection with U S WEST.  Examples of industry-defined information are Network Channel (NC) and Network Channel Interface (NCI) codes.  These codes define the characteristics of the circuit and it’s connecting points.  The SGAT includes the following interconnection ordering process language:

When ordering LIS, the ordering Party shall specify on the Access Service Request: (ASR)  1) the type and number of Interconnection facilities to terminate at the Point of Interconnection in the Serving Wire Center; 2) the type of interoffice transport, (i.e., Direct Trunked Transport or Tandem Transmission);  3) the number of trunks to be provisioned at an end office or local tandem; and 4) any optional features.  When the ordering Party requests facilities, routing, or optional features different than those determined to be available, the Parties will work cooperatively in determining an acceptable configuration, based on available facilities, equipment and routing plans.

U S WEST and CLECs have the obligation to participate in joint planning meetings at quarterly intervals to establish trunk design and provisioning requirements.  At these meetings, U S WEST and the CLEC provide mutual trunk forecast information to ensure end user call completion between the parties’ networks.  Section 7.2.2.8 of the SGAT defines the responsibilities of both parties regarding the interconnection forecasting process, including: the forecast forms and format, required information, forecast cycle, joint planning meetings, U S WEST Trunk Group Servicing Request (TGSR) process, and the trunk group resizing requirement.

The forecasting process includes joint planning meetings at quarterly intervals to review trunk design and provisioning requirements.  The parties provide each other with forecasts of trunk utilization for each direct trunk group and the amount of traffic destined to each subtending office for tandem-switched traffic.  Each party utilizes the forecast cycle outlined in the SGAT, which stipulates that forecasts be submitted on a quarterly basis.  The forecast will identify trunking requirements for a two-year period.  These forecasts are intended to facilitate the availability of interconnection facilities and identify potential blockage problems before they occur.  To the extent an unexpected trunk blockage occurs, either party can initiate a request for additional trunks.  When appropriate, as previously mentioned, U S WEST notifies the CLEC through the U S WEST Trunk Group Servicing Request (TGSR) process of the need to take action and place orders in accordance with the forecasted trunk requirements.

To assist CLECs with ordering and obtaining interconnection, U S WEST offers Local Interconnection Service (LIS) training and facility tours and provides to CLECs its Interconnection and Resale Resource Guide.  In addition, U S WEST’s Account Teams meet individually with CLEC representatives to ascertain CLEC specific requirements.

## Processes Supporting Interconnection

U S WEST has developed detailed processes that support the ordering, provision, and repair of interconnection for CLECs.  Exhibit MJW-02 and Exhibit MJW-03 contain the end-to-end process flows and task lists, specifically defining the interconnection procedures utilized for the ordering and provision process.  Exhibit MJW-04 and MJW-05 contain the end-to-end process flows and task lists for the repair process of interconnection services.

U S WEST satisfies the requirements for providing interconnection under the Act and FCC rules through the U S WEST SGAT, existing Commission-approved interconnection agreements, and the specific procedures it employs to implement interconnection with CLECs.  U S WEST has established centers that coordinate the fulfillment of “interconnection” service orders.  U S WEST’s “interconnection” centers support multiple states and trained personnel exist to meet CLECs demand for interconnection in Colorado.

## U S WEST Provides CLECs with Interconnection at Least “Equal in Quality”

Section 251(c)(2)(C) requires an incumbent LEC to provide interconnection "at least equal in quality to that provided by the [incumbent LEC] to itself or to any subsidiary, affiliate, or any other party to which the carrier provides interconnection.”

U S WEST is committed to provide CLECs with interconnection that is equal in quality to that which U S WEST provides itself, its subsidiaries and its affiliates.  This commitment is documented in the SGAT, which states:

U S WEST will provide to CLEC interconnection at least equal in quality to that provided to itself, to any subsidiary, affiliate, or any other party to which it provides interconnection.

To satisfy this objective, U S WEST has adopted industry standards for network blockage and transmission quality in designing its interconnection facilities.  Specifically, Section 7.2.2.9.1 of the SGAT provides that: “The Parties will provide designed Interconnection facilities that meet the same technical criteria and service standards, such as probability of blocking in peak hours and transmission standards, in accordance with current industry standards.”

CLECs place orders for Interconnection facilities based on their business plan, internal forecasts, and independent collection of traffic data.  Either party can initiate a request for additional trunks if an unexpected trunk blockage occurs.

U S WEST has implemented procedures to minimize trunk blocking by proactively monitoring interconnection traffic.  At least once per month, U S WEST service assurance employees collect traffic data on all two-way CLEC trunk groups.  The traffic data is analyzed and trunk groups that are blocking beyond incidental levels are identified.

Based upon this analysis, U S WEST issues a TGSR to the CLEC on any trunk group requiring augmentation to relieve blocking.  The CLEC, in response to the TGSR, reviews its traffic data.  If the CLEC agrees that augmentation is appropriate, it issues an augmentation order to U S WEST.  The CLEC must either accept or reject the TGSR within thirty days.  The SGAT includes the following TGSR process:

When appropriate, U S WEST will notify CLEC through the U S WEST Trunk Group Servicing Request (TGSR) process of the need to take action and place orders in accordance with the forecasted trunk requirements. . . .

U S WEST has instituted the detailed TGSR process to monitor current trunk group status, enabling an appropriate response to both inadequate and excess capacity.  This process allows CLECs to minimize blocking problems.

In Colorado, during 1998 and 1999, U S WEST proactively notified CLECs of potential interconnection blocking by issuing 129 TGSRs.  CLECs responded to 28% of the TGSRs by placing orders to augment the trunk groups.  CLECs responded to 6% of the TGSRs by issuing disconnect orders.  However, CLECs responded to 69% of the TGSRs by declining to take corrective action.

## U S WEST Network Trunk Design Standards

Network Trunk Design Standards size the network to operate to specified service objectives.  The industry standard for assessing trunk performance is call blockage.  Call blockage is defined as attempted calls that cannot be further advanced toward the call destination due to equipment shortage or network failure.

As previously mentioned, U S WEST employs identical industry standards for trunk traffic engineering and design for U S WEST and CLEC trunk facilities.  These concepts have evolved through exhaustive traffic studies, probability theory, technological advances, and the use of economic models to determine the most economical trunking network required to provide the desired quality of “service-blocking objective” for all customer demand.  The quality of service-blocking objectives are the product of years of analytical work, studies, and experience by the Bell Operating Companies (BOCs).

The U S WEST network’s design and layout is based upon the application of these principles.  U S WEST’s service-blocking objectives assume that all customers will not use the network at the same time, so economic utilization can be realized by designing the trunking facilities to industry guidelines.  If U S WEST were to design its network so that no call ever blocked, a separate path would be needed between each telephone in the network.  For networks of more than a few telephones, this would become cost prohibitive.  The solution is to provide fewer paths (trunks) that many telephones can access.  This creates a probability that some calls will be blocked if more telephone calls are attempted than trunks are available.  Therefore, as a policy matter, it is necessary to define a service-blocking objective that identifies acceptable blocking levels and balances customer expectations and economic efficiency.

U S WEST’s service-blocking objectives are established to provide high-quality, low-cost trunk facilities as needed.  Based upon industry standards, U S WEST designs direct final-trunk groups utilizing a one-percent blocking criterion and designs tandem-connecting trunk groups utilizing a one-half of a percent criterion.

U S WEST satisfies the service-blocking objective for CLECs by providing facilities and equipment in sufficient quantities to connect a reasonable proportion of the number of expected attempted calls originated by CLEC customers.  CLECs establish the quantity of attempted calls by providing trunk forecast information to U S WEST.

Based on industry studies, trunk groups that are engineered to the one-percent blocking criterion actually experience blocking averages between two to three percent.  When the two-percent blocking criterion is applied, the blocking average increases to about six-percent.  The increased average blocking, attributable to the two-percent engineering criterion, strongly supports the use of the one-percent blocking trunk design criterion, because it achieves an industry accepted balance between customer expectations and economic efficiency.  Variations in the level of busy hour traffic cause the blocking of service beyond the engineering blocking criterion.

As a result of these industry studies, U S WEST considers blocking below a two-percent threshold to be incidental.  Moreover, based upon these industry studies, incidental blockage of this type is expected, given variable traffic patterns that all carriers experience.  This means that on selected days when the number of calls made during the busiest hour of the day is higher than the average, blocking will exceed the blocking criterion.  Similarly, on selected days when the number of calls made during the busiest hour of the day is below the average, blocking will be less than the blocking criterion.  Service variations below two-percent are within the range of acceptable service.  Increasing trunk capacity to lower the actual blocking average below a two-percent threshold would require a change in the service-blocking criterion and would result in inefficient network utilization.

## Interconnection Performance Measures

U S WEST has developed detailed performance measures covering interconnection, including provisioning, repair, and network blocking.  U S WEST interconnection performance measurements track how, and within what time frames, U S WEST provides comparable interconnection to itself.  These measurements ensure that CLECs receive interconnection that is equal in quality.

### Trunk Blocking Measures

As described in the reciprocal compensation section of this affidavit, U S WEST and CLEC end offices connect with other end offices through two types of trunking – direct and tandem routing.  Direct trunks allow one end office to transport traffic directly to another end office over a single interoffice facility.  Tandem switches, in contrast, allow an end office to send traffic destined for one or more end offices on a ‘common trunk’ to a tandem switch.  The tandem switch then sorts this combined traffic, and routes each call to the appropriate terminating end office.

U S WEST measures two types of interconnection trunk blockage, which extend from a CLEC switch to the U S WEST network.  U S WEST measures the blockage on interconnection trunks that connect CLEC end offices with U S WEST tandems and separately measures the blockage on interconnection trunks that connect CLEC end offices with U S WEST end offices.

U S WEST also provides two blockage measures for U S WEST’s Colorado network.  U S WEST measures trunk blockage on trunks that connect U S WEST end offices with U S WEST tandems and U S WEST provides blockage on trunks that extend from one U S WEST end office to another U S WEST end office.

These performance measures allow a comparison of the blockage on direct trunks utilized by CLECs and direct trunks utilized by U S WEST.  The performance measures also allow a similar comparison of blockage experienced on tandem trunks.  The following performance data is contained in Mike William’s testimony:

Percentage Blocking -- Direct Trunking

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 0.57% | 2.12% | 0.10% | 0.05% | 0.45% |
| U S WEST | 0.38% | 0.53% | 0.40% | 0.35% | 0.20% |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 0.33% | 0.56% | 0.48% | 0.54% | 0.55% |
| U S WEST | 0.07% | 0.06% | 0.13% | 0.05% | 0.23% |

Direct trunk blocking for U S WEST and the CLECs has been negligible throughout 1999.  In eight of the nine months, overall CLEC blockage did not exceed 0.6%.  This means that, in eight of the nine most recent months, CLEC customers experienced significantly fewer than six calls blocked out of every 1000.  In February, CLEC blockage exceeded the two-percent threshold.  However, on February 19, 1999, U S WEST issued a TGSR to the CLEC causing this blockage.  On March 23, 1999, the CLEC rearranged their trunk configuration and blocking returned to below the two-percent threshold.  As previously indicated, trunk groups that are engineered to the one-percent blocking criterion typically experience actual blocking averages between two to three percent depending on the variable traffic patterns that all carriers experience.  U S WEST’s data is similar.

Percentage Blocking -- Tandem Trunking

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 4.22% | 0.45% | 0.02% | 1.95% | 0.00% |
| U S WEST | 3.51% | 1.79% | 1.17% | 0.97% | 0.72% |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 0.10% | 0.21% | 1.64% | 2.16% | 1.21% |
| U S WEST | 0.23% | 0.09% | 0.72% | 0.17% | 0.93% |

In seven of the nine months, CLEC blockage did not exceed 2%.  In two months, CLEC blockage exceeded the two-percent threshold.  All the blockage in January was caused by a significant increase in traffic by a CLEC on a single trunk group.  U S WEST issued a TGSR on February 25th.  The CLEC declined to augment the trunk.  The next month blocking returned to below the two-percent threshold.  The majority of the blockage in September was caused by a significant increase in traffic by the same CLEC on a single trunk group.  U S WEST did not issue a TGSR, because the CLEC had issued seven orders to augment the trunk group that was primarily responsible for the blockage.  As previously indicated, trunk groups that are engineered to the one-percent blocking criterion typically experience actual blocking averages between two to three percent depending on the variable traffic patterns that all carriers experience.  U S WEST’s data is almost identical.

As represented by the year to date trunk blockage data, U S WEST provides interconnection that is “at least equal in quality” to that which U S WEST provides to itself.

### Timely Trunk Installation

For interconnection trunks provided to CLECs, U S WEST provides a measure of the installation interval, the percentage of scheduled dates met (‘commitments met’), and for installations that are not completed on time, U S WEST provides the average number of days service is late beyond the original scheduled date.  For each of the above interconnection trunk measures, U S WEST also provides comparable trunk measures for the U S WEST network.  With these new measures, the Commission can directly compare trunk installation for CLECs and U S WEST.  The results for the installation interval measurements are as follows:

Average Installation Intervals – Interoffice Trunking (Days)

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 54.3 | 53.1 | 48.0 | 48.6 | 39.2 |
| U S WEST | 65.2 | 156.1 | 127.1 | 111.0 | 72.6 |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 39.0 | 39.2 | 32.6 | 35.2 | 44.3 |
| U S WEST | 126.4 | 42.1 | 60.8 | 94.7 | 99.5 |

The monthly average installation intervals demonstrate that CLECs experienced significantly shorter trunk installation intervals than did U S WEST in each of the last nine months.  The year to date average interval for installation of U S WEST’s interoffice trunks is more than double the installation interval of interconnection trunks for CLECs.

These results demonstrate that CLECs in Colorado receive trunk installation that is “at least equal in quality” to that which U S WEST provides to itself.

As the preceding data indicates, U S WEST also meets its trunk installation commitments for CLECs more frequently than it meets installation commitments for itself.  The “commitments met” data are as follows:

Installation Commitments Met – Interoffice Trunking (Percentage)

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 94.4% | 100.0% | 96.6% | 88.4% | 95.4% |
| U S WEST | 11.3% | 3.6% | 4.5% | 17.4% | 7.2% |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 93.0% | 94.7% | 86.1% | 78.7% | 92.3% |
| U S WEST | 10.8% | 20.0% | 38.6% | 56.6% | 23.8% |

In each of the nine most recent months for which data is available, U S WEST met its commitments to CLECs at a materially higher percentage than within the U S WEST network.  In six of these nine months, U S WEST met in excess of 90% of its trunk installation commitments for CLECs.  In 8 of these 9 months it met in excess of 86% of its commitments.  Again, these results demonstrate that CLECs in Colorado receive trunk installation that is “at least equal in quality” to that which U S WEST provides to itself.

When U S WEST fails to install a trunk within the committed installation interval, U S WEST records the additional time it takes to get the trunk installed for the CLEC.  This performance measure is called “delayed days.”  Similar data is tracked for delays experienced in U S WEST’s own network.  The following table compares the average number of days a trunk installation is delayed beyond the commitment date:

Delayed Days – Interoffice Trunking (Days)

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 5.5 | 0.0 | 26.6 | 9.0 | 9.4 |
| U S WEST | 44.1 | 58.4 | 109.6 | 105.8 | 33.8 |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 36.9 | 39.0 | 9.6 | 16.0 | 16.3 |
| U S WEST | 98.3 | 24.1 | 39.1 | 36.7 | 57.9 |

In five of the most recent nine months, CLECs’ experienced delays less than 10 days.  More importantly, in eight of the nine months, the average number of delay days that U S WEST experienced for its own trunk installations was significantly longer than the average delay experienced by CLECs.   Year to date, U S WEST has experienced delays four times longer than that which CLECs experience for trunk installation.  Thus, while U S WEST has installed interconnection trunks for CLECs on time a high percentage of the time, when a delay occurs, the delay has been comparatively short.

This data again shows that CLECs in Colorado receive trunk installation that is “at least equal in quality” to that which U S WEST provides to itself.

### Repair Quality

U S WEST provides additional performance measures for trunk repair, including Installation Trouble Reports, All Troubles Cleared within 4 Hours, Mean time to Restore, and Repair Repeat Report Rate.

Installation trouble reports are a means of assessing installation quality.  By measuring the percentage of newly installed interconnection trunks orders that generate a trouble report within 30 days of installation completion, an indication of installation quality can be obtained.  U S WEST does not provide a comparison at this time.  The process for issuing internal trunk installation trouble reports and the associated performance measurement are under development.

The performance measurement data on installation trouble reports indicates that a very low percentage of interconnection trunk orders generate trouble tickets.  The results for the CLEC new service installations without trouble reports measurement results are as follows:

New Service Installations Without Trouble Reports (Percentage)

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 97.8% | 100.0% | 98.4% | 97.8% | 96.5% |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 97.1% | 91.6% | 98.3% | 95.1% | 97.1% |

The speed with which U S WEST clears trouble reports can also be used to evaluate the repair process.  U S WEST measures the percentage of CLEC interconnection trunk trouble reports that are cleared in under four hours.  Unlike the previous measure that addresses newly installed trunks, this measure addresses trouble reports on all CLEC interconnection trunks, regardless of how recently the trunks were installed.  U S WEST provides a comparable measure for trouble reports on trunks within the U S WEST network.  Because this particular measure was developed recently, data is only available for the months of July, August and September 1999:

All Troubles Cleared within Four Hours (Percentage)

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 37.5% | 44.2% | 58.6% | 56.0% | 38.9% |
| U S WEST | NA | NA | NA | NA | NA |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 38.5% | 51.9% | 57.5% | 50.0% | 48.7% |
| U S WEST | NA | 7.14% | 3.2% | 23.1% | 8.6% |

These results demonstrate that U S WEST clears CLEC troubles on interconnection trunks within four hours approximately 50% of the time.  For the three months when comparable data exists for the U S WEST network, U S WEST cleared troubles within four hours for CLECs much more often than it did for itself.  These results demonstrate that U S WEST provides CLECs with interconnection trunk repair that is “at least equal to” the trunk repair it provides to itself.

This data does show, however, that approximately 50% of the time it takes longer than four hours to clear troubles on CLEC interconnection trunks.  To capture the CLECs overall interconnection trunk repair experience, U S WEST measures the average time it takes to restore a CLEC interconnection trunk.  A similar measure is available for the repair of trunks within the U S WEST network, although, results were not available for this new measure until July 1999.  A comparison of the mean time to restore data is provided below:

Mean Time to Restore (Hours:Minutes)

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 17:25 | 11:58 | 11:01 | 4:55 | 10:59 |
| U S WEST | NA | NA | NA | NA | NA |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 10:52 | 11:16 | 5:29 | 8:03 | 10:17 |
| U S WEST | NA | 216:43 | 933:29 | 156:22 | 586:18 |

These results demonstrate that U S WEST has cleared CLEC trouble reports on interconnection trunks in approximately 11 hours or less in each of the last eight months.  On average it takes approximately 10 hours to repair a problem.  Thus, CLECs can count on U S WEST repairing their interconnection trunks on the same day that the trouble is reported.  For the three months where comparable data exists, U S WEST cleared troubles for CLEC interconnection trunks much more quickly than on trunks within U S WEST’s network.  These results also demonstrate that U S WEST provides interconnection repair to CLECs that is “at least equal to” the quality the repair it provides to itself.

U S WEST also measures the number of troubles that generate an additional trouble ticket within 30 days. A similar measure exists for trunk repair within the U S WEST network, although results were not available for this new measure until July 1999.  A comparison of the repair repeat report rate measurements is provided below:

Repair Repeat Report Rate (Percentage)

|  | Jan-99 | Feb-99 | Mar-99 | Apr-99 | May-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 25.0% | 44.2% | 27.6% | 16.0% | 36.1% |
| U S WEST | NA | NA | NA | NA | NA |

|  | June-99 | July-99 | Aug-99 | Sept-99 | YTD-99 |
| --- | --- | --- | --- | --- | --- |
| CLECs | 38.5% | 44.4% | 40.0% | 29.2% | 34.5% |
| U S WEST | NA | 78.6% | 58.1% | 61.5% | 63.8% |

These results demonstrate U S WEST experiences a higher percentage of repeat reports than CLECs experience in the interconnection trunks.

All of this performance data demonstrates that U S WEST provides interconnection trunking to CLECs in a timely manner; that CLECs obtain interconnection trunks in quantities that generally provide CLECs with similar levels of trunk blockage; that the quality of interconnection trunks is high, as demonstrated by the relatively low number of trouble reports generated by CLECs; and that the repair of interconnection trunks, when required, is also performed in a timely manner and at a high level of quality.

## Rate Elements for Interconnection

Section 251(c)(2)(D) requires that incumbent LECs provide interconnection "on rates, terms, and conditions that are just, reasonable, and nondiscriminatory.”  Section 252(d)(1) requires rates for interconnection and unbundled network elements to be based on "cost” (determined without reference to a rate-of-return or other rate-based proceeding), and allow for a “reasonable profit”.

U S WEST offers interconnection at rates determined by this Commission in its cost docket, which are reflected in the proposed SGAT.  The rates that apply to interconnection include: Entrance Facilities, Direct Trunked Transport, Tandem Switched Transport, Multiplexing, End Office Call Termination, Expedite Charge (LIS Trunks), and Transit Traffic.  The development of those rates are described in the affidavit of Mr. Paul McDaniel.

## Summary

I have reviewed each of the requirements of the Act and the FCC rules with regard to interconnection.  U S WEST satisfies these requirements and has a concrete and specific legal obligation to provide interconnection as referenced in the U S WEST SGAT and the various interconnection agreements between U S WEST and CLECs in Colorado.  The requirements for providing interconnection are satisfied and supported by the specific procedures U S WEST employs to implement interconnection with CLECs.  U S WEST has established centers that coordinate the fulfillment of “interconnection” service orders.  U S WEST’s “interconnection” centers support multiple states and trained personnel exist to meet CLECs demand for interconnection in Colorado.  U S WEST stands ready to provide interconnection to CLECs in Colorado in accordance with these requirements pursuant to the terms of the U S WEST SGAT.

U S WEST has presented prima facie evidence that the procedures it has in place for providing interconnection to CLECs ensure that the requirements of the Act and the FCC rules for interconnection are and will be satisfied, now and in the reasonably foreseeable future, for competitors entering into the Colorado telecommunications market.  U S WEST’s processes, procedures, and capabilities for interconnection ensure that an efficient competitor is afforded a reasonable opportunity to compete.

Based on this evidence, the Colorado Commission should find that U S WEST has satisfied the requirements of Checklist Item 1, Interconnection, of Section 271(c)(2)(B)(i) of the Telecommunications Act of 1996.

# V.	Checklist Item 1 -- Collocation

## Collocation Overview

Collocation is the term used to describe the ability of a CLEC to put its equipment in the incumbent LEC’s wire center building and join the CLEC’s equipment to the incumbent LEC’s equipment (interconnection).  In other words, collocation allows for the placement of equipment necessary for interconnection.  Collocation also facilitates CLEC access to unbundled network elements and ancillary and finished services.

Described more specifically, collocation allows a CLEC to place cables into a U S WEST central office and to terminate those cables on transmission equipment owned by the CLEC.  The CLEC’s transmission equipment can be interconnected to the U S WEST network through connections referred to as “Interconnection Tie Pairs” (ITP).  The ITP provide for interconnection with a wide variety of network services, including trunk-side ports on end office and tandem switches, unbundled loops, and other private line facilities.

Section 251(c)(6) of the Act requires that incumbent local exchange carriers provide to CLECs physical collocation and when necessary virtual collocation.  That Section reads:

The duty to provide, on rates, terms, and conditions that are just, reasonable, and nondiscriminatory, for physical collocation of equipment necessary for interconnection or access to unbundled network elements at the premises of the local exchange carriers.  Except that the carrier may provide for virtual collocation if the local exchange carrier demonstrates to the State commission that physical collocation is not practical for technical reasons or because of space limitations.

A number of federal rules govern the provision of collocation, including rules adopted in its 1st Report and Order, CC Docket 96-325 and recently adopted  “706” federal rules, FCC Docket 98-147 (3/99).

Collocation is not a specific checklist item requirement.  However, checklist item one requires that U S WEST permit interconnection at “any technically feasible point” within U S WEST’s network.  U S WEST facilitates interconnection through collocation.  As such, collocation is a means to demonstrate compliance with the interconnection requirements of the checklist.  Collocation also facilitates CLEC access to unbundled network elements or ancillary services.

In Colorado, U S WEST is providing 186 units of physical collocation and 13 units of virtual collocation to 15 CLECs in 43 central office buildings under existing collocation agreements.  Confidential Exhibits MJW-C2 and MJW-C3 provide specific wire center detail and collocation activity, respectively.

## FCC Collocation Rules:

### First Report and Order

In its 1st Report and Order, CC Docket 96-325, the FCC concluded that it should “adopt explicit national rules to implement the collocation requirement of the 1996 Act.”  They further found that “. . . specific rules defining minimum requirements for nondiscriminatory collocation arrangements will remove barriers to entry by potential competitors and speed the development of competition.”

The “national rules”, established by the FCC in this docket, were codified in their rules, § 51.323, and are summarized as follows:

Incumbent LECs shall provide physical and virtual collocation.

Collocated equipment must generally be used to obtain interconnection or to access unbundled network elements.  Collocation of switching equipment is not required.

Points of interface (POI) provisions are established and at least two such POIs are required at each incumbent LEC premises where the incumbent LEC has at least two entry points.  Fiber, copper, coax, and microwave facilities must be accommodated, consistent with state commission-approved interconnection agreements.

Establishes provisions for nondiscriminatory allocation of space for the collocation of equipment.  Space shall be made available on a first-come, first-served basis.  Incumbent LECs are not required to lease or construct additional space to provide for additional space if existing space is exhausted.  Incumbent LECs shall take into account collocation demand when forecasting growth of facilities.  Incumbent LECs may retain space for their own future use, but not on more favorable terms than those of other providers also wishing to reserve space for future growth.  Space reserved for future growth shall be relinquished to a virtual collocation request, before denying such a request on the grounds that insufficient space exits.  Subject to space constraints and the approval of state commissions, incumbent LECs may impose reasonable restrictions on the warehousing of unused space by other collocating providers.

Establishes provisions for interconnection between collocating carriers and between collocating carriers and unbundled network elements.  Generally, incumbent LECs shall provide the connection between collocation spaces and between collocation spaces and unbundled network elements.

Incumbent LECs may require reasonable security measures to separate collocation  installations from incumbent LEC facilities.

Incumbent LECs shall permit a collocating provider to subcontract the construction of its physical collocation arrangements with contractors approved by the incumbent LEC.

### “706” Federal Collocation Rules

In the FCC’s Order, CC Docket No. 98-147, Deployment of Wireline Service Offering Advanced Telecommunications Capability, the FCC required incumbent LECs to make new collocation arrangements, including cageless shared, and adjacent collocation available to competing carriers.  Under these new rules, CLECs are permitted to locate all equipment necessary for interconnection, whether or not such equipment has a switching function.  The following is a summary of the new collocation rules:

Incumbent LECs must make available to requesting competitive LECs shared cage and cageless collocation arrangements.  Moreover, when collocation is exhausted at a particular LEC location, incumbent LECs must permit collocation in adjacent controlled environmental vaults or similar structures to the extent technically feasible.

A collocation method used by one incumbent LEC or mandated by a state commission is presumptively technically feasible for any other incumbent LEC.

Incumbent LECs may adopt reasonable security measures to protect their central office equipment.

Incumbent LECs may not require competitive LEC equipment to meet more stringent safety requirements than those the incumbent LEC imposes on its own equipment.

Incumbent LECs must permit competitors to collocate all equipment used for interconnection and/or access to unbundled network elements (UNEs), even if it includes a "switching" or enhanced services function, and incumbent LECs cannot require that the switching or enhanced services functionality of equipment be disengaged.

Incumbent LECs must permit a competitive LEC to tour the entire central office in which that competitive LEC has been denied collocation space.  Incumbent LECs must provide a list of all offices in which there is no more space.  Incumbent LECs must remove obsolete, unused equipment, in order to facilitate the creation of additional collocation space within a central office.

## U S WEST’s Compliance with the FCC Collocation Rules

U S WEST’s legal obligation to provide collocation is established by the Colorado SGAT and the various interconnection agreements between U S WEST and CLECs in Colorado.  Section 8 of the SGAT includes the collocation terms and conditions, rate elements, descriptions and arrangements, and the ordering process offered by U S WEST.

### U S WEST Provides Physical and Virtual Collocation

U S WEST offers two types of collocation, physical and virtual collocation.  U S WEST provides four standard arrangements of physical collocation: caged, cageless, shared and InterConnection Distribution Frame (ICDF) collocation.

Caged physical collocation allows the CLEC to place its equipment within U S WEST’s central office while being surrounded by a cage that provides an increased level of security for the CLEC’s equipment.

Cageless physical collocation allows the CLEC to place its equipment in the U S WEST central office adjacent to U S WEST or other CLEC equipment, but it is not separated from other central office equipment.

Shared Physical Collocation allows two or more CLECs to share a single collocation enclosure.  Under Shared Physical Collocation, one CLEC obtains a Caged Physical Collocation arrangement from U S WEST.  CLECs may share the space, in according to the terms and conditions agreed upon by the two CLECs.  U S WEST will prorate the charge for site conditioning and preparation undertaken by U S WEST to construct the shared collocation cage or condition the space for collocation.

ICDF collocation is offered for CLECs that do not require transmission equipment to be placed in the U S WEST Central office, but require access to unbundled network elements for the purpose of combining unbundled network elements.

Virtual collocation allows the CLEC to turn over equipment to U S WEST for engineering, installation, and maintenance on behalf of the CLEC.  This type of arrangement is used, principally, when there is no space for physical collocation.

Physical and virtual collocations are relatively similar, but differ in a few significant ways.  First, under a physical collocation arrangement, the CLEC engineers, installs, and maintains its own equipment in the collocation space.  Under virtual collocation, however, the CLEC’s equipment is turned over to U S WEST for installation and maintenance.

Quite simply, the difference between physical and virtual collocation is dependent on who installs and maintains the CLEC’s transmission equipment in the collocation space provided by U S WEST.  If the CLEC installs and maintains the transmission equipment, then it constitutes a physical collocation.  If U S WEST installs and maintains the CLEC’s transmission equipment, then it is a virtual collocation arrangement.

Standard collocation arrangements are available at U S WEST wire centers.  U S WEST, through the Bona Fide Request (BFR) process will provide collocation at other U S WEST locations, including adjacent collocation and any collocation method used by one incumbent LEC or mandated by the Colorado Commission.

### U S WEST Collocation Policies and Procedures

U S WEST has implemented policies and procedures that comply with all of the FCC’s currently effective rules.  U S WEST has also recently revised a number of its policies and procedures included in the SGAT, to comply with the FCC 706 collocation rules.  For example, Section 8.0 of U S WEST’s SGAT explicitly allows for collocation consistent with the FCC’s collocation rules.

Both virtual and physical collocation are available to CLECs in Colorado.  Section 8.1.1 of the SGAT includes the standard collocation descriptions and arrangements offered by U S WEST.  The arrangements include virtual, caged physical, cageless physical, and ICDF collocation.  Specifically, Section 8.1.1.3 and 8.1.1.4 of the SGAT obligates U S WEST to provide physical cageless and shared collocation.  Standard collocation arrangements are available at U S WEST wire centers.  U S WEST, through the Bona Fide Request (BFR) process will provide collocation at other U S WEST locations, including adjacent collocation and any collocation method used by another incumbent LEC or mandated by the Colorado Commission.

U S WEST does not require CLEC collocated equipment to meet more stringent safety requirements than those U S WEST imposes on its own equipment.  Sections 8.2.2.5 and 8.2.3.12 of the SGAT only require that a CLECs collocated equipment comply with the Bellcore Network Equipment System (NEBS) generic equipment requirements TR-NWT-000063, U S WEST wire center environmental and transmission standards, and any statutory (local, state or federal) and/or regulatory requirements in effect at the time of equipment installation or that subsequently become effective.  This is expressly permitted by the FCC.

Consistent with the FCC rules, U S WEST allows CLECs to collocate equipment that is necessary for interconnection or access to unbundled network elements (UNEs), regardless of whether such equipment includes a switching functionality, provides enhanced services capabilities, or offers other functionality’s.  Section 8.2.1.2 of the SGAT contains only one limitation on the equipment that cannot be collocated -- CLECs may not collocate equipment that is not necessary for either access to UNEs or for interconnection, such as equipment used exclusively for switching or for enhanced services.  The FCC expressly authorizes this limitation.  U S WEST will permit collocation of any equipment required by law, unless U S WEST first proves to the Commission that the equipment will not be actually used by a CLEC for the purpose of obtaining interconnection or access to unbundled network elements.

Section 8.2.1.9 of the SGAT states that upon request by a CLEC, U S WEST will submit to the CLEC a report containing: available collocation space in a particular U S WEST premises; the number of collocators; any modifications in the use of the space since the last report; and measures that U S WEST is taking to make additional space available for collocation.

Similarly, Section 8.2.1.11 of the SGAT states that, upon request by a CLEC, U S WEST shall allow a CLEC’s representatives to tour the entire wire center premises escorted by U S WEST personnel, within ten days of the denial of collocation space.  Such tours shall be without charge to the CLEC.  If, after the tour of the premises, U S WEST and the CLEC disagree about whether space limitations at the wire center make collocation impractical, U S WEST and the CLEC may present their arguments to the Commission.  Again, these principals comply with FCC rules.

U S WEST maintains a publicly available document, posted for viewing on the Internet, indicating all premises that are full, and updates this document within ten days of the date at which it learns that a premises is out of physical space for collocation.  The internet address is:

http://www.uswest.com/carrier/bulletins/collocation-bulletins/colosum599.html.

If U S WEST denies a request for collocation due to lack of space, Section 8.2.1.14 of the SGAT states that a CLEC may request that U S WEST remove obsolete, unused equipment, in order to facilitate the creation of additional collocation space within a central office.  Again, this is consistent with FCC rules.  U S WEST also proactively reviews central office space for obsolete or unused equipment prior to collocation denial.

Consistent with the FCC rules, Section 8.2.1.23 of the SGAT states that U S WEST shall provide, at the request of CLEC, the connection between the CLEC’s equipment in its collocated spaces to the collocated equipment of another CLEC located in the same U S WEST Wire Center.  Alternatively, a CLEC utilizing an approved vendor, may construct its own connection, using copper or optical fiber equipment, between the CLEC's equipment and that of another CLEC.  A CLEC may place its own connecting facilities outside of the actual physical Collocation space, subject only to reasonable safety limitations.

U S WEST provides CLECs with the same network connections as U S WEST uses to provision services to its own retail customers.  CLECs and U S WEST share the same distribution frames for similar types and speeds of equipment, where technically feasible and space permitting.  CLEC terminations are placed on U S WEST cross-connect frames using standard engineering principles.  CLEC terminations share frame space with U S WEST terminations without a requirement to utilize an intermediate device, such as a SPOT (Single Point of Termination) frame.  This provides a clear and logical demarcation point for U S WEST and the CLEC.  If a CLEC disagrees with the cross-connect frame selected by U S WEST, the CLEC may request a tour of the U S WEST wire center and may request use of an alternative frame through the BFR process.

## Three Step Collocation Provisioning Process

After the CLEC requests collocation, U S WEST performs the following three steps in the collocation provisioning process:

Feasibility Study – First, U S WEST provides the CLEC with a study of the feasibility of providing collocation pursuant to the CLEC’s request at a particular site.  In accordance with the terms of the SGAT, U S WEST will provide CLECs with a feasibility study within seven calendar days of the CLEC’s initial collocation request for virtual collocation, and ten calendar days for physical collocation.  If the CLEC’s first choice for collocation is not available (e.g., caged physical), the study will determine the feasibility of the CLEC’s second choice (e.g., cageless physical), as described by the CLEC in its initial request.

Quote Preparation – Second, if the CLEC’s collocation request is found to be feasible, U S WEST provides the CLEC with a quotation of the charges associated with the specific request within 25 calendar days of the completion of the feasibility study.

Installation – Third, upon payment of the CLEC of 50% of the non-recurring charges for the collocation installation and, in the case of virtual collocation, receipt of the CLEC’s collocated equipment, U S WEST will install the collocation arrangement within 90 days.

The collocation process described above is included in the SGAT.  These provisions demonstrate U S WEST’s legal obligation to provide collocation.  Specifically, Section 8.4. of the SGAT includes the specifics concerning the collocation ordering process and intervals.

U S WEST offers collocation on a first-come, first-served basis.  If a request for collocation is denied due to lack of space in a U S WEST central office, that CLEC will be offered a number of collocation alternatives.  Alternative collocation options include: (1) a lesser amount of space that is determined to be available in relation to the original request; (2) a CLEC will be offered cageless physical collocation (bay at a time); or (3) virtual collocation as an alternative to physical collocation.  As previously mentioned, a CLEC may also request space reclamation such as: the grooming or the moving of working circuits from working equipment to other equipment for the purpose of providing additional collocation space or conditioning or reconditioning of space for the placement of transmission equipment.

## Collocation Process Flows

U S WEST satisfies the Act and the FCC requirements for providing collocation through the U S WEST SGAT and the specific procedures U S WEST employs to implement collocation with CLECs.  Exhibit MJW-06 contains detailed process flows defining the collocation procedures and Exhibit MJW-07 contains the task list associated with the process flows.

These processes are fully operational.  U S WEST centers that coordinate the fulfillment of “collocation” requests support multiple states and trained personnel are in place to meet demand for collocation in Colorado.

## Collocation Performance Measures

U S WEST tracks a number of performance indicators for collocation to establish that it offers collocation such that an efficient competitor has a meaningful opportunity to compete.  These collocation measures correspond with each of the three steps in the collocation process.  Specifically, U S WEST measures the average time it takes to provide CLECs with feasibility studies, quotes, and installations.  It also tracks the percentage of feasibility studies, quotes, and installations that are completed on schedule.

At the beginning of 1999, U S WEST had established an objective of completing feasibility studies within a fifteen day period.  In its 706 decision (which became effective on June 1999) the FCC required U S WEST and other LECs to provide feasibility studies in ten days.  Consistent with the FCC’s decision, Section 8.4.3.1 of the SGAT requires U S WEST to perform feasibility studies within ten calendar days from date of receipt of the collocation request.

Significant volumes of CLEC collocations orders in Colorado and throughout U S WEST’s fourteen state region have contributed to delays in fulfilling U S WEST’s objectives for feasibility studies, quotations, and installations.  Nevertheless, in Colorado, U S WEST has provided 186 units of physical collocation and 13 units of virtual collocation to 15 CLECs.  The sheer volumes of collocations already provisioned and operational demonstrates that U S WEST provides CLECs in Colorado with collocation as required by the Act.

The testimony of Michael Williams further describes U S WEST’s performance measures.

## Collocation Rate Elements

The rate elements for collocation are described in Section 8.3 of the SGAT and the collocation rates are included in Exhibit A.  The development of those rates has been discussed in the Direct Testimony of Mr. Paul McDaniel.

## Summary

I have reviewed the requirements of the Act, the FCC rules and the SGAT with regard to collocation.  U S WEST has satisfied these requirements and has a concrete and specific legal obligation to provide collocation as referenced in the U S WEST SGAT and the various interconnection agreements between U S WEST and the CLECs in Colorado.  The FCC requirements for providing collocation are satisfied and supported by the specific procedures U S WEST employs to implement collocation with CLECs.  U S WEST’s centers that coordinate the fulfillment of collocation requests support multiple states and trained personnel meet CLECs’ demand for collocation in Colorado.  U S WEST stands ready to provide collocation to CLECs in Colorado in accordance with these requirements pursuant to the terms of the U S WEST SGAT.

U S WEST has presented evidence that the procedures it has in place for providing collocation to CLECs ensure that the requirements of the Act and the FCC rules are and will be satisfied, now and in the foreseeable future, for competitors entering into the local Colorado telecommunications market.  U S WEST’s collocation processes, procedures and capabilities ensure that an efficient competitor is afforded a reasonable opportunity to compete.

Based on this evidence, the Colorado Commission should find that U S WEST has satisfied the collocation requirements of Checklist Item 1 of Section 271(c)(2)(B)(i) of the Telecommunications Act of 1996.

# VI.	Checklist Item 13 - Reciprocal Compensation

## Reciprocal Compensation Overview

When two carriers collaborate to complete a local call, the originating carrier is compensated by its end user, and the terminating carrier is entitled to reciprocal compensation pursuant to section 251(b)(5) of the Act.  Reciprocal compensation is the payment between U S WEST and CLECs for the transport and termination of local traffic between their respective networks.

Call transport is the transport of traffic originating at one carrier’s network and terminating at another carrier’s end office switch.  Under a reciprocal compensation arrangement, the originating carrier pays the terminating carrier for the use of the terminating carrier’s end office switch, transport facilities, and tandem switch facilities.

The FCC defines "termination," for purposes of section 251(b)(5), as the switching of traffic that is subject to section 251(b)(5) at the terminating carrier's end office switch (or equivalent facility) and delivery of that traffic from that switch to the called party's premises.

## Call Termination

Call Termination charges apply to local traffic and involve the terminating carrier’s end office switching.  The Act and FCC orders recognize that carriers make significant investments in switches.  Reciprocal compensation arrangements compensate the terminating carrier for the use of those facilities on a usage sensitive basis.  Call Termination is illustrated below.

The only rate element for Call Termination is a per minute of use charge for the use of the end office terminating switch.  This rate is reflected in Exhibit A of the SGAT.

## Transport Options Provided By U S WEST

U S WEST provides direct trunked transport and tandem-switched transport to CLECs.  Wherever possible, U S WEST offers two-way trunking.  The Parties may purchase transport services from each other, a third party, or from a third party that has leased a Private Line Transport Service facility from U S WEST.

### Direct Trunked Transport

Direct trunked transport is an uninterrupted path between two end offices.  Direct trunks link a U S WEST end office to another U S WEST end office or a CLEC end office.  The transport facility originates at the point of interconnection between the U S WEST network and the CLEC network and terminates at the U S WEST switch; it provides a direct path between the CLEC switch and the U S WEST end office for the exchange of local traffic.  Direct Trunked Transport is available between the serving wire center of the point of interconnection and the terminating party’s local tandem or end office switches.  The rates are listed in Exhibit A of the SGAT.  Included below is an illustration of a U S WEST’s Local Interconnection Service (LIS), configured with Direct Trunked Transport.

### Tandem-Switched Transport

Tandem-switched transport links two or more end offices through a tandem switch.  By way of example, tandem-switched transport connects a CLEC’s switch to a U S WEST wire center through an intermediate U S WEST tandem switch.  The dedicated transport facility originates at the point of interconnection between the U S WEST network and the CLEC network and terminates at the U S WEST tandem switch.  Tandem trunks connect the tandem switch to each end office switch in the local calling area.  These trunks are considered common trunks because the trunks are not dedicated to the CLEC’s use, but instead are used “in common” by many carriers, including U S WEST, independent local exchange carriers, and CLECs.  The combination of switching at the tandem switch and the common trunks is the “tandem switched transport” which allows the CLEC access to every central office connected to the local tandem switch for the exchange of local traffic.  LIS ordered to a local tandem will be provided as Direct Trunked Transport between the serving Wire Center of CLEC's POI and the local tandem.  Tandem transmission rates, as specified in Exhibit A of the SGAT, apply to the transport provided from the local tandem to U S WEST's end office.  Included below is an illustration of a U S WEST’s Local Interconnection Service (LIS), configured with Tandem Switched Transport.

The mix of end office switching and tandem switching is important to ensure the most efficient and cost effective form of transport between the two networks.  Direct trunks to U S WEST end offices are required when the traffic between a U S WEST end office and the CLEC switch exceeds or will soon exceed the busy hour equivalent of approximately one DS1 (i.e., 24 voice grade trunks).

U S WEST has incorporated this requirement in the SGAT:

Based on forecasted or actual traffic at CLEC’s busy hour in centum call seconds (CCS), where there is a DS1’s worth of traffic (512 CCS) between CLEC's switch and a U S WEST end office, CLEC will order a dedicated (i.e., direct) trunk group from the CLEC POI directly to the U S WEST end office. To the extent that CLEC has established a Collocation arrangement at a U S WEST end office location, and has available capacity, CLEC shall provide two-way direct trunk facilities, when required, from that end office to CLEC's switch.  In all other cases, the direct facility may be provisioned by U S WEST or CLEC or a third party.  If both CLEC and U S WEST desire to provision the facility and cannot otherwise agree, the Parties may agree to resolve the dispute through the submission of competitive bids.

Direct traffic involves two switches and a single path.  Tandem-switched traffic involves at least three switches and two paths.  The tandem-switched route is more costly; therefore, the direct trunked route is favored to manage building traffic.

As previously mentioned U S WEST accommodates two-way trunks wherever possible.  However, separate trunk groups will be established based on billing, signaling, and network requirements.  For example, the SGAT states:

Two-way trunk groups will be established wherever possible.  Exceptions to this provision will be based on billing, signaling, and network requirements.  However, either Party may elect to provision its own one-way trunks for delivery of traffic to be terminated on the other Party’s network.

Separate trunk groups will be established based on billing, signaling, and network requirements.  For example, (1) billing requirements – Jointly Provided Switched Access vs. Exchange Service  (EAS/Local) traffic, (2) signaling requirements - MF vs. SS7, and (3) network requirements - directory assistance traffic to Operator Services tandems. . .

### Call Transit

Call Transit is available for two CLECs who do not interconnect with each other to exchange traffic.  The CLECs can exchange traffic by transmitting calls across U S WEST’s network.  U S WEST will accept traffic originated by a CLEC or ILEC and will terminate it at a point of interconnection with another CLEC or ILEC.  U S WEST will provide this transit service through local switches.  The applicable tandem switching and tandem transmission rates contained in Exhibit A of the SGAT apply to the originating CLEC.  Call transit is illustrated in the drawing below.

## Rate Elements Associated with Reciprocal Compensation

Direct-trunked transport has two rate elements:  (1) A fixed, per month charge, and (2) a fixed per mile charge.  Both fixed charges vary with the length of the dedicated facility provided (based on airline mileage), as measured from the serving wire center to the tandem or end office.  Direct-trunked transport levies different charges for the DS1 and DS3 transmission levels.  The SGAT states:

When DTT is provided to a local tandem for Exchange Service (EAS/local traffic), or to an access tandem for Exchange Access (IntraLATA Toll), or Jointly Provided Switched Access traffic, the applicable DTT rate elements apply between the Serving Wire Center and the tandem.  Additional rate elements for delivery of traffic to the terminating end office are Tandem Switching and Tandem Transmission. . .

Direct trunk charges are adjusted when the trunks are established as two-way trunks.  For example, if U S WEST provides direct trunks to the CLEC, the charges will be lowered to account for the portion of the trunk used by U S WEST to transport U S WEST’s originating traffic to the CLEC.  If U S WEST uses the trunk 50% of the time, then the fixed rates are reduced by 50%.  In this arrangement the CLEC pays only for U S WEST transport facilities used by the CLEC to deliver its originating traffic to the U S WEST end office switch.  The reverse is true if the CLEC provides the direct trunks to U S WEST.  The SGAT, for example, includes the following two-way trunk group compensation language:

If the Parties elect to establish LIS two-way DTT trunks, for reciprocal exchange of Exchange Service (EAS/Local) traffic, the cost of the LIS two-way DTT facilities shall be shared among the Parties by reducing the LIS two-way DTT rate element charges as follows:

The provider of the LIS two-way DTT facility will initially share the cost of the LIS two-way DTT facility by assuming an initial relative use factor of 50% for a minimum of one quarter.  The nominal charge to the other Party for the use of the DTT facility, as described in Exhibit A, shall be reduced by this initial relative use factor.  Payments by the other party will be according to this initial relative use factor for a minimum of one quarter.  The initial relative use factor will continue for both bill reduction and payments until the Parties agree to a new factor, based upon actual minutes of use data for non-ISP traffic to substantiate a change in that factor.  Once negotiation of new factor is finalized, the bill reductions and payments will apply going forward, for a minimum of one quarter.  By agreeing to this interim solution, the parties do not waive their position that traffic delivered to Enhanced Service Providers is interstate in nature.

Tandem-switched transport has two rate elements: A transport element and a switching element.  Both the transport element and the tandem switching element are fixed, per minute of use rates.  The SGAT states:

For traffic delivered through a U S WEST or CLEC local tandem switch (as defined in this SGAT), the tandem switching rate and the tandem transmission rate in Exhibit A shall apply per minute in addition to the end office call termination rate. . .

U S WEST’s reciprocal compensation arrangements are in accordance with section 252(d)(2).

U S WEST has a concrete legal obligation to pay reciprocal compensation.  Section 7.3.1 of the SGAT states: “The Reciprocal Compensation Provisions of this SGAT shall apply to the exchange of Exchange Service (EAS/local) traffic between CLEC’s network and U S WEST’s network.”

Symmetrical compensation arrangements obligate U S WEST to pay a CLEC for transport and termination of traffic originated by U S WEST at the same rate that U S WEST charges to transport and terminate traffic originated by the CLEC.  The reciprocal compensation rates in the SGAT and the existing interconnection agreements are symmetrical.  The SGAT states:

The per minute of use call termination rates as described in Exhibit A of this SGAT will apply reciprocally for Exchange Service (EAS/Local) traffic terminated at a U S WEST or CLEC end office.

The reciprocal compensation rates are included in Exhibit A of this SGAT.  The development of these rates is described in the testimony of Mr. Paul McDaniel.

## Billing Records, Billing and Payment of Reciprocal Compensation

U S WEST will record, bill, and pay in accordance with the SGAT and the various interconnection agreements in Colorado.

Certain types of calls or types of interconnection require the exchange of billing records between U S WEST and the CLEC, including, for example, alternate billed and toll free service calls.  All call types must be routed between the networks, accounted for, and settled between CLECs and U S WEST.  Certain calls will be handled via the respective operator service platforms.

U S WEST uses a CroSS7 Platform system that collects and formats call data from the Signaling System Seven (SS7) Network to determine requirements for billing and reporting.  Where possible and appropriate, existing accounting and settlement systems will be used to exchange records and bill.

The Act requires reciprocal compensation payments for local traffic to be made in a timely fashion.  U S WEST has established a concrete and legal obligation to pay reciprocal compensation.  The SGAT states:

Amounts payable under this SGAT are due and payable within thirty (30) calendar days after the date of invoice.

For some time a dispute has existed concerning whether Information Service Provider (ISP)-bound traffic is subject to the reciprocal compensation obligations established by the Act.

On February 28, 1999, the FCC issued a ruling in which it concluded:  (1) that ISP-bound traffic, on a jurisdictional basis, is largely interstate and (2) that such jurisdictional nature notwithstanding, state commissions presently possess the authority under Section 252 of the Act to determine the inter-carrier compensation mechanism for such traffic.  The FCC states:

As noted, section 251(b)(5) of the Act and our rules promulgated pursuant to that provision concern inter-carrier compensation for interconnected local telecommunications traffic.  We conclude in this Declaratory Ruling, however, that ISP-bound traffic is non-local interstate traffic.  Thus, the reciprocal compensation requirements of section 251(b)(5) of the Act and Section 51, Subpart H (Reciprocal Compensation for Transport and Termination of Local Telecommunications Traffic) of the Commission's rules do not govern inter-carrier compensation for this traffic.  As discussed, supra, in the absence a federal rule, state commissions have the authority under section 252 of the Act to determine inter-carrier compensation for ISP-bound traffic.

In Docket No. 98F-299T, the Colorado Commission considered whether the existing interconnection agreement between ICG and U S WEST required these parties to pay reciprocal compensation for ISP bound traffic.  The Colorado Commission concluded that the existing interconnection agreement between ICG and U S WEST required these parties to pay reciprocal compensation for ISP-bound traffic.  In reaching this conclusion, the Commission emphasized that it was not adopting as a general rule, the notion that reciprocal compensation was the appropriate payment mechanism for ISP-bound traffic.

The issue concerning reciprocal compensation for ISP-traffic may be raised by USWC in negotiations with ICG on a new agreement.  Absent agreement of the parties, USWC may raise this issue in future arbitration proceedings before the Commission.

U S WEST will pay ICG as ordered by the Colorado Commission, subject to adjustments excluding toll traffic and internet protocol voice traffic from billed amounts.  However, as evidenced by the language of U S WEST’s SGAT, U S WEST intends to pursue this issue in future interconnection negotiations.

In an order regarding a 271 application in Louisiana, the FCC declared that the payment by a RBOC of reciprocal compensation for ISP-bound traffic is not a 271 requirement:

At this time, we do not conclude that BellSouth is failing to make required reciprocal compensation payments in Louisiana on a timely basis.  The general issue of a LEC’s obligation to pay reciprocal compensation for traffic delivered to ISPs is pending in a number of proceedings.  Neither this commission nor the Louisiana Commission have reached a final determination on this matter.  We do not, at this time, consider BellSouth’s unwillingness to pay reciprocal compensation for traffic that is delivered to ISPs located within the same local calling area as the originating BellSouth end user in assessing whether BellSouth satisfies this checklist item.  Any future grant of in region interLATA authority under Section 271 will be conditioned on compliance with the forthcoming decisions related to Internet traffic in Louisiana.

The SGAT includes language exempting traffic originated to and terminated by enhanced service providers (defined elsewhere as ISP traffic) from the reciprocal compensation arrangements of the SGAT.  The SGAT states:

As set forth above, the Parties agree that reciprocal compensation only applies to EAS/Local Traffic and further agree that the FCC has determined that traffic originated by either Party (the “Originating Party”) and delivered to the other Party, which in turn delivers the traffic to the enhanced service provider (the “Delivering Party”) is interstate in nature.  Consequently, the Delivering Party must identify which, if any, of this traffic is EAS/Local Traffic.  The Originating Party will only pay reciprocal compensation for the traffic the Delivering Party has substantiated to be EAS/Local Traffic.  In the absence of such substantiation, such traffic shall be presumed to be interstate.

Such a provision is consistent with current law, as expressed by the FCC and the Colorado Commission.  The SGAT is consistent with footnote 10 of the Colorado Commission’s that allows U S WEST to negotiate reciprocal compensation arrangements relating to ISP-bound traffic.

U S WEST has an SGAT that obligates it to pay reciprocal compensation.  U S WEST is a party to interconnection agreements that obligate it to pay reciprocal compensation.  U S WEST is paying reciprocal compensation.  U S WEST will pay reciprocal compensation for ISP-bound traffic pursuant to orders by the Colorado Commission.  U S WEST has met its obligation to pay reciprocal compensation pursuant to the Act, FCC, and Colorado Commission Orders.

## U S WEST Processes Supporting Reciprocal Compensation

U S WEST has developed detailed processes that support reciprocal compensation billing and payments to CLECs.  Exhibit MJW-08 contains the detailed process flows defining the billing and payments procedures and MJW-09 contains the task list associated with the process flows.

A CLEC or U S WEST may request an audit of reciprocal compensation billing.  Section 18.0 of the SGAT defines the terms and conditions of the audit process.  The party requesting the audit may review the non-requesting party’s records, books, and documents.  The SGAT includes the following audit provisions:

[An audit is] the comprehensive review of: Data used in the billing process for services performed, including reciprocal compensation, and facilities provided under this SGAT; and

Data relevant to provisioning and maintenance for services performed or facilities provided by either of the Parties for itself or others that are similar to the services performed or facilities provided under this SGAT for Interconnection or access to unbundled loops, ancillary and finished services.

## Summary

The Act requires U S WEST to include reciprocal compensation arrangements for call transport and call termination in its interconnection agreements.  U S WEST satisfies these requirements through the U S WEST SGAT and the various interconnection agreement with CLECs in Colorado.

U S WEST’s processes, procedures, and capabilities for reciprocal compensation ensure that an efficient competitor is afforded a reasonable opportunity to compete.  Thus, this Commission should find that U S WEST has satisfied the requirements of Checklist Items 13, reciprocal compensation.

# VII	Checklist Item 3 -- Poles, Ducts, Conduit and Rights-of-Way

## Overview of Access to Poles, Conduits, Ducts, and Rights-of-Way

Poles are used to support cable, equipment, facilities, apparatuses or appurtenances that are used or useful in providing telecommunications services (“attachments”).  U S WEST provides attachments to poles that are owned in full or in part by U S WEST.

“Ducts” or conduits are enclosed reinforced passages capable of housing communications cables.  Some ducts or conduit controlled by U S WEST may be located within buildings owned by third parties.  Access to such ducts or conduit is made available to other carriers, to the extent permissible, under existing rights-of-way permits and easements.  U S WEST permits an attaching party to interconnect its ducts in the manholes of U S WEST.  This allows the attaching party to conveniently enter and exit U S WEST’s conduit system.

All of U S WEST's poles, cables, conduits, and ducts are located either in public rights-of-way, such as streets, alleys, bridges or dedicated utility easements, or on property owned by private or public entities.  U S WEST's authority to have its poles and conduit systems on public rights-of-way is subject to state and local ordinances and laws, zoning regulations, or other permissions or authorities granted by government entities.  On private or public property (other than public rights-of-way), U S WEST obtains an easement or license from the owner to place and maintain its poles and conduit systems.  Sometimes, easements or licenses from adjoining property owners are necessary even to occupy public rights-of-way.  U S WEST shares use of poles, conduit systems, and easements with an electric utility under the terms of joint use or joint ownership agreements.  In some instances, U S WEST may have poles, ducts, or conduits on private property without any right (or an incomplete right) to grant access to third parties.

U S WEST's ability to maintain its poles and conduit systems is subject to the terms, conditions, and limitations of these various laws and agreements, and so U S WEST must condition an attaching party's access to its poles and conduit systems on those same terms and conditions.  If U S WEST loses its right to have a pole, duct, conduit, or right-of-way at any particular location, it obviously cannot provide an attaching party with a right to use such structure.  The Federal Act does not require the controlling LECs to acquire grants of authority on behalf of other carriers.  U S WEST has incorporated the above approach in the SGAT.

U S WEST satisfies Section 271(c)(2)(B)(iii) of the Act if it provides "[n]ondiscriminatory access to the poles, ducts, conduits, and rights-of-way owned or controlled by the [BOC] at just and reasonable rates in accordance with the requirements of section 224."

## FCC Rules Governing Nondiscriminatory Access to Poles, Ducts, Conduits, and Rights-Of-Way

A number of federal rules govern the access to poles, ducts, conduits, and rights-of-way, including rules adopted in the FCC’s 1st Report and Order, CC Docket No. 96-325, Report and Order, CS Docket 98-20, and the recent FCC Order on Reconsideration, FCC 99-266.

The FCC’s 1st Report and Order adopted general rules and guidelines designed to give parties flexibility to reach agreements on access to poles, ducts, conduits, and rights-of-way, without the need for regulatory intervention.  The FCC’s 1st Report and Order also provides for a dispute resolution mechanism when negotiations fail and establishes requirements concerning modifications to pole attachments and the allocation of the cost of such modifications.

In the FCC’s 1st Report and Order, the FCC determined that “the reasonableness of particular conditions of access imposed by a utility should be resolved on a case-specific basis.  The FCC found that the large number of variables present with respect to poles and conduit nationwide prevented it from creating a comprehensive set of rules.  Instead, the FCC adopted several general rules, supplemented by guidelines and presumptions that are intended to facilitate the negotiation and mutual performance of fair, pro-competitive access arrangements.  The five general rules of general applicability established FCC concerning access to poles, ducts, conduits, and rights-of-way are listed below:

- In evaluating a request for access, a utility may continue to rely on such codes as the National Electrical Safety Code (NESC) to prescribe standards with respect to capacity, safety, reliability, and general engineering principles.

- Federal requirements, such as those imposed by the Federal Energy Regulatory Commission (FERC) and the Occupational Safety and Health Administration (OSHA), will continue to apply to utilities to the extent such requirements affect requests for attachments to utility facilities under Section 224(f)(1).

- The FCC considers state and local requirements affecting pole attachments.

- Where access is mandated, the rates, terms, and conditions of access must be uniformly applied to all telecommunications carriers and cable operators that have or seek access.  Except as specifically provided herein, U S WEST must charge all parties an attachment rate that does not exceed the maximum amount permitted by the FCC formula.  Other terms and conditions also must be applied on a nondiscriminatory basis.

- U S WEST may not favor itself over other parties with respect to the provision of telecommunications or video programming services.

The recent FCC Order on Reconsideration, FCC Order 99-266, clarifies the FCC’s 1st Report and Order.  Summarized below are examples of the requirements:

U S WEST is required to take all reasonable steps to accommodate access and to explore potential accommodations in good faith with the party seeking access.

U S WEST may require that individuals who perform attachments and related activities meet utility standards for the performance of such work, but U S WEST may not dictate the identity of the workers who will perform the work itself.

Regarding the manner and timing of notice that must be provided to third-party attachers to ensure reasonable opportunity to add to or modify an attachment.  U S WEST, absent a private agreement establishing notification procedures, must provide written notification to parties holding attachments on facilities to be modified at least 60 days prior to the commencement of physical modifications.  In emergency situations in which a 60-day notice would be impractical, U S WEST is required to provide notice as soon as reasonably practicable.

In the case of facility modifications initiated by third-party attachers that create excess capacity.  U S WEST is not obligated to use any later-earned revenues from that capacity to compensate the parties who paid for the modification, even in the case in which U S WEST did not share in the cost of the modification.

U S WEST must respond to written requests for access within 45 days.  If access is not granted within 45 days of the request, U S WEST must confirm the denial in writing by the 45th day.

U S WEST satisfies these requirements of checklist item 3 and has a concrete and specific legal obligation to provide access as referenced in the U S WEST SGAT and the various interconnection agreements between U S WEST and CLECs in Colorado.

## Requirements for the Prima Facie Case for Checklist Item 3

U S WEST must establish a prima facie case of nondiscriminatory access to poles, ducts, conduits, and rights-of-way.  The FCC in its BellSouth Louisiana II Order specified four elements for establishing a prima facie case for checklist item 3.  The four requirements for a prima facie case are listed below:

- Evaluating facility requests pursuant to Section 224 of the Act and the Local Competition Order.

- Granting competitors nondiscriminatory access to information on facilities availability.

- Permitting competitors to use non-[RBOC] workers, and

- Complying with state and federal rates.

Also, as part of the prima facie case, the FCC requires compliance with the provisions of the Local Competition Order on poles, ducts, conduits, and rights-of-way.

## Evaluating Requests for Access to Poles, Conduits, Ducts and Rights-of-Way

The FCC outlines three specific elements for the evaluation of facility requests.  U S WEST satisfies these requirements and includes these requirements in the SGAT.  Specifically,

U S WEST must not discriminate in favor of themselves;

U S WEST must take reasonable steps to accommodate access; and

Denials must be in writing within 45-days and specify the reasons access was denied.

When a CLEC inquires about access to poles, ducts, conduits, and rights-of-way, the CLEC completes the “General Information” pages of the U S WEST access request form.  The information provided by the CLEC on these pages includes a drawing of the proposed route and a general description of the facility to be placed.  Upon completion and submission of the General Information pages by the CLEC, they are provided to the U S WEST Account Manager.

U S WEST follows three steps when evaluating a CLEC request for access to poles, ducts, conduits, and rights-of-way:

Record Verification – First, the U S WEST Account Manager receives the access request form, and the request is forwarded to U S WEST engineering.  On a first-come, first-served basis, U S WEST engineering then gathers all available records applicable to the CLEC request.  For example, engineering may research records to determine (1) if the requested route exists, (2) the recorded distances, (3) structure ownership, and (4) the number of manholes/poles along the route.  Upon completion of the record verification process, U S WEST prepares a quote for the field verification work and submits it to the CLEC.  The record verification work and quote for field verification is completed within 10 business days.

Field Verification – Second, upon request and payment of estimated field verification costs, U S WEST will perform field verification of space along the requested route.  This information will be given to the CLEC for approval.  The field verification for duct lease requests includes the identification of all conduits and ducts that enter each wall of each manhole.  The field verification also allows for the identification of any make-ready work.  The field verification required for pole attachments will include the physical inspection of all poles along the requested route for available space to attach and the identification of clearance requirements and any required modifications or make ready work that is necessary to add additional attachments.  U S WEST completes the field verification within 35 days.

Written Response and a Estimated Cost Quote For Any Required Modifications/Make Ready Work – Third, upon completion of the field verification process, U S WEST provides a response to the CLEC concerning duct/pole availability and estimated cost quote for any required modifications/make ready work.  Upon acceptance of the quote, U S WEST and the CLEC will enter into an appropriate contract.  The combined 45-day timeline for evaluating facilities requests complies with the FCC’s requirements.

The process for evaluating a CLEC request for access to poles, ducts, conduits, and rights-of-way described above is included in the SGAT.  These provisions demonstrate U S WEST’s legal obligation to provide access to poles, ducts, conduits, and rights-of-way.  Specifically, Section 10.8.4 of the SGAT includes the specifics concerning the ordering process.

## Nondiscriminatory Access to Information

U S WEST has committed to provide access to information within a reasonable time through the bona fide request process or various interconnection agreements in Colorado.  This process is substantially the same as the process approved in the FCC BellSouth Louisiana II Order.

## Choice of Workforce

The FCC also requires U S WEST to allow CLECs to use workers of their choice to perform make-ready or other work necessary for the attaching of their facilities if those workers have the same qualifications and training as U S WEST's own workers.  The SGAT Section 10.8.2.17 states: “CLEC may use individual workers of its choice to perform any work necessary for the attaching of its facilities so long as such workers have the same qualifications and training as U S WEST’s workers.  CLEC may use any contractor approved by U S WEST to perform Make-Ready Work.”  In fact, CLECs have used their contractors to perform make-ready work.

## Rates for Access to Poles, Conduits, Ducts, and Rights-of-Way

Checklist item 3 requires rates to be consistent with Section 224 and with state commission rates and that the rates be uniformly applied.  U S WEST satisfies this element by its rates in the SGAT, which are expressly consistent with Section 224, FCC rules and the rates approved by the Colorado Commission.

The rate elements associated with providing access to poles, ducts, conduits, and rights-of-way allow U S WEST to recover its costs -- both recurring and nonrecurring -- associated with providing pole and conduit space.  U S WEST quotes to the attaching party the following fees: Inquiry Fee, Field Verification Fee, required Make Ready/Modification Work and Annual Usage.

## Processes Supporting Access to Poles, Ducts, Conduits, and Rights-of-Way

U S WEST developed detailed processes to support the ordering of access to poles, ducts, conduits, and rights-of-way for CLECs.  Exhibit MJW-10 contains the end to end process flows, specifically defining the access to poles, ducts, conduits, and rights-of-way procedure and MJW-11 contains the task list associated with the process flows.

During 1997, 1998 and 1999, six CLECs placed six requests for access to poles and five CLECs placed six requests for duct within the State of Colorado.  Five CLEC requests resulted in access to U S WEST poles, one request for access to poles was declined by the CLEC after U S WEST completed the records verification and the field survey procedures.  Four CLEC requests resulted in access to U S WEST ducts, one request for access to ducts was declined by the CLEC after U S WEST completed the records verification and the field survey procedures, one request is in progress.  To the best of my knowledge, no CLEC has requested access to rights of way.

## Summary

U S WEST satisfies the requirements of checklist item 3 and has a concrete and specific legal obligation to provide access as referenced in the U S WEST SGAT and the various interconnection agreements between U S WEST and CLECs in Colorado.  U S WEST has presented prima facie evidence that the procedures it has in place for providing access to poles, conduit, duct, and rights-of-way ensure that the requirements of the Act and the FCC rules for providing access to poles, conduit, duct, and rights-of-way are and will be satisfied, now and in the reasonably foreseeable future, for competitors entering into the Colorado telecommunications market.  Moreover, U S WEST’s processes, procedures, and capabilities for the provision of access to poles, conduit, duct, and rights-of-way ensure that an efficient competitor is afforded a reasonable opportunity to compete.  Based on this evidence the Colorado Commission should find that U S WEST has satisfied the requirements of Checklist Item 3, access to poles, duct, and rights-of-way, of Section 271(c)(2)(B)(iii) of the Telecommunications Act of 1996.

# VIII.	CONCLUSION

For the reasons described in my affidavit, the Colorado Commission should find that U S WEST has satisfied the Act’s checklist requirements for interconnection and collocation; access to poles, ducts, conduits, and rights-of-way; and for reciprocal compensation.

BEFORE THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO

Docket No. 97I-198T

_______________________________________________________________________

IN THE MATTER OF THE INVESTIGATION INTO U S WEST COMMUNICATIONS, INC.’S COMPLIANCE WITH § 271(C) OF THE TELECOMMUNICATIONS ACT OF 1996

______________________________________________________________________

EXHIBITS OF

MICHAEL J. WEIDENBACH

U S WEST COMMUNICATIONS, INC.

November 30, 1999

NOTICE OF CONFIDENTIALITY: THE PORTIONS OF THIS DOCUMENT IDENTIFIED BELOW, HAVE BEEN FILED UNDER SEAL.

MJW-C1  Confidential Exhibit of Interconnection LIS Trunks

MJW-C2  Confidential Exhibit of Interconnection Collocation

MJW-C3  Confidential Exhibit of Collocation Activity

# INDEX OF EXHIBITS

DESCRIPTION					EXHIBIT

Qualifications of Mr. Michael J. Weidenbach		MJW-01

Interconnection Provisioning Process Flow		MJW-02

Interconnection Provisioning Process Task List		MJW-03

Interconnection Repair Process Flow		MJW-04

Interconnection Repair Process Task List		MJW-05

Collocation Process Flow		MJW-06

Collocation Process Task List		MJW-07

Reciprocal Compensation Billing Process Flow		MJW-08

Reciprocal Compensation Billing Process Task List		MJW-09

Poles, Ducts, and Rights of Way Process Flow		MJW-10

Poles, Ducts, and Rights of Way Process Task List		MJW-11

Confidential Exhibit of Interconnection LIS Trunks		MJW-C1

Confidential Exhibit of Interconnection Collocation		MJW-C2

Confidential Exhibit of Collocation Activity		MJW-C3

47 C.F.R. 51.5.

47 C.F.R. 51.5.

See the SGAT, Section 17.0, Bona fide Request Process.

First Interconnection Order at Para. 212.

First Interconnection Order at Para. 553.

First Interconnection Order at Para. 551 through 553.

See the SGAT, Section 7.1.2, Methods of Interconnection.

See the SGAT, Section 17.0, Bona fide Request Process.

See the SGAT, Section 7.4.1, Ordering Interconnection.

See the SGAT, Section 7.1.1.1, Interconnection Quality.

See the SGAT, Section 7.2.2.8.11, U S WEST’s Trunk Group Service Request Process.

The average blocking ratio to which a group of services is engineered or administered.

Bellcore, SR-TAP-000191, Issue 2, Trunk Traffic Engineering Concepts and Applications.

Direct Final Trunks – A last-choice trunk group that receives overflow traffic, and for which there is no alternate route (only route).

Tandem-connecting – A category of trunk groups that interconnect end-offices with tandems.

Id., Page 4.

Busy Hour – A sixty consecutive minute interval with the highest levels of load, used in traffic engineering.

Bellcore, SR-TAP-000191, Issue 2, Trunk Traffic Engineering Concepts and Applications, Page 4.

For blocking above the two-percent level, see the TGSR process, described in this testimony.

As previously discussed, CLECs responded to 69% of the TGSRs by declining to take corrective action.

The testimony of Michael Williams provides descriptions, data and analysis for all of U S WEST’s performance measurements.

See SGAT Section 7.3 and SGAT Exhibit A, which describe the rates for interconnection.

See the SGAT, Section 8.2.4, Transmission Facility Access to Collocation Space.

See the SGAT, Section 9.1.6, Interconnection Tie Pair.

Telecommunications Act of 1996, Section 251(c)(6).

Standard collocation arrangements are available at U S WEST wire centers.  U S WEST, through the Bona Fide Request (BFR) process will provide collocation at other U S WEST locations, including adjacent collocation and any collocation method used by one incumbent LEC or mandated by the Colorado Commission.

FCC 1st Report and Order, CC Docket No. 96-325, Paragraphs 555-617, § 51.323.

FCC 1st Report and Order, CC Docket No. 96-325, Paragraph 558.

? 	FCC Press Release, March 18, 1999, Report No. CC 99-6.

See the FCC’s Order, CC Docket No. 98-147, Para. 36.

See the FCC’s Order, CC Docket No. 98-147, Para. 30.

See the SGAT, Section 8.2.1.13, Out of Space Document.

See the SGAT, Section 8.2.1.24 – 8.2.1.26, CLEC Connections to the U S WEST Network.

SGAT, Section 8.4.2.1.

SGAT at Section 8.4.3.1.

SGAT at Sections 8.4.2.1 and 8.4.3.1.

SGAT at Sections 8.4.2.2 and 8.4.3.1.

See the SGAT, Section 8.0, Collocation.

See the SGAT, Section 8.2.3.2, Space Allocation.

See First Report and Order, Para. 1040.

See the SGAT, Section 7.3.2, Direct Trunked Transport.

See the SGAT, Section 7.3.4.2, Tandem-Switched Transport.

See the SGAT, Section 7.2.2.1.2.1, Two-way Trunk Groups.

See the SGAT, Section 7.2.2.1.2.2, Private Line Transport.

See the SGAT, Section 7.2.2.1.3, Terms and Conditions.

See the SGAT, Section 7.2.2.9.2, Terms and Conditions, Two way trunk groups..

See the SGAT, Section 7.2.2.9.3, Terms and Conditions, separate group requirements.

See the SGAT, Section 7.3.7, Transit Traffic and associated Rate Elements in Exhibit A.

See the SGAT, Section 7.3.2.1.2 and Exhibit A, Direct Trunked Transport and associated Rate Elements.

See the SGAT, Section 7.3.2.2 (a), Two-way trunk group compensation.

See the SGAT, Section 7.3.4.2.1, Tandem Switched Transport and associated Rate Elements in Exhibit A.

See the SGAT, Section 7.3.4.1.1, End office Call Termination and associated Rate Elements in Exhibit A.

See Section 251(b)(5) of the Act.

See the SGAT, Section 5.4.1, Payment.

In the Matter of Implementation of the Local Competition Provisions in the Telecommunications Act of 1996, Intercarrier Compensation for ISP-Bound Traffic, CC Docket Nos. 96-98 and 96-68, ¶28, fn87 (FCC February 25, 1999).

See Colorado Docket No. 98F – 299T, Footnote No. 10.

See FCC 98-271, CC Docket No. 98-121, FCC BellSouth Louisiana II, Para. 5.

See the SGAT, Section 7.3.4.1.3,Traffic originated by and terminated to enhanced service providers.

See the SGAT, Section 7.3, Reciprocal Compensation.

See the SGAT, Section 18.1, Audit process.

First Report and Order, Para. 1179.

See the SGAT, Section 10.8.2.8, Access to Poles, Terms and Conditions, Ducts, Conduits, and Rights-of-Way.

47 U.S.C. § 271(c)(2)(B)(iii).  As originally enacted, section 224 was intended to address obstacles that cable operators encountered in obtaining access to poles, ducts, conduits or rights-of-way owned or controlled by utilities.  The 1996 Act amended section 224 in several important respects to ensure that telecommunications carriers as well as cable operators have access to poles, ducts, conduits, or rights-of-way owned or controlled by utility companies, including LECs.

Local Competition Order, Para. 1122.

First Report and Order, Para. 1143.

The FCC stated that it would “monitor the effect of this approach and proposed more specific rules at a later date if reasonably necessary to facilitate access and the development of competition in telecommunications and cable services.” First Report and Order, Para. 1143.

First Report and Order, Para. 1151 - 1158.

The FCC states in the Local Competition Order: “. . . state and local requirements affecting attachments are entitled to deference even if the state has not sought to preempt federal regulations under section 224(c).  The 1996 Act increased significantly the Commission's role with respect to attachments by creating federal access rights and obligations, which for decades had been the subject of state and local regulation.  Such regulations often relate to matters of local concern that are within the knowledge of local authorities and are not addressed by standard codes such the NESC.  We do not believe that regulations of this sort necessarily conflict with the scheme established in this Order.  More specifically, we see nothing in the statute or in the record that compels us to preempt such local regulations as a matter of course.  Regulated entities and other interested parties are familiar with existing state and local requirements and have adopted operating procedures and practices in reliance on those requirements.  We believe it would be unduly disruptive to invalidate summarily all such local requirements.”  First Report and Order, Para. 1154.

See the SGAT, Section 10.8.

FCC BellSouth Louisiana II Order, Para. 174.

See the SGAT, Section 10.8.2, Terms and Conditions.

See the SGAT, Section 10.8.2.10, CLEC requests for modifications or additional capacity.

See the SGAT, Section 10.8.4.5, Written Notification and Denial of Access.

See the SGAT Section 10.8.4.1 and 10.8.4.2, Inquiry.

See the SGAT, Section 10.8.4.2, Access to Poles, Terms and Conditions, Ducts, Conduits, and Rights-of-Way, Facility Locations.

See the SGAT, Section 10.8.4., Access to Poles, Ducts, Conduits, and Rights-of-Way, Ordering.

See the First Report and Order, Para. 1224.

See the SGAT, Section 10.8.4, Ordering access to Poles, Ducts, Conduits, and Rights-of-Way.

See the SGAT, Section 10.8.2.4.

FCC BellSouth Louisiana II Order, Para. 180.

FCC BellSouth Louisiana II Order, Para. 181.

See the SGAT, Section 10.8.3, Rate Elements.

See the SGAT, Section 10.8.3, Access to Poles, Ducts, Conduits, and Rights-of-Way, Rate Elements descriptions.