BEFORE THE PUBLIC UTILITIES COMMISSION

OF THE STATE OF COLORADO

DOCKET NO. 07S-521E

RE:  IN THE MATTER OF ADVICE NO. 1495 - PUBLIC SERVICE COMPANY OF COLORADO REVISION TO THE INTERRUPTIBLE SERVICE OPTION CREDIT (ISOC) TARIFF - ELECTRIC.

ANSWER TESTIMONY AND EXHIBITS OF LARRY Y. SHIAO

STAFF OF THE COLORADO PUBLIC UTILITIES COMMISSION

NOTICE OF CONFIDENTIALITY:  A PORTION OF THE EXHIBITS OF DR. LARRY SHIAO IS HIGHLY CONFIDENTIAL AND HAS BEEN FILED UNDER SEAL AND SERVED IN ACCORDANCE WITH THE RULES OF CONFIDENTIALITY 4 CCR 723-1100(c)

March 24, 2008

TABLE OF CONTENTS

i.	IDENTIFICATION OF WITNESS AND PURPOSE OF TESTIMONY	1

II.	PUBLIC SERVICE’S Interruptible service optional credit program (ISOC) AND THE CHANGES THAT IT IS PROPOSING	2

III.	Monthly Credit Rate (MCR)	7

A.	The Avoided CAPACITY Cost derivation	7

B.	THE CAPACITY AVAILABILITY FACTOR (Ca)	10

C.	THE AVOIDED ENERGY RATE	14

D.	CONCLUSION REGARDING THE MCR	15

IV.	THE MONTHLY CREDIT AND ITS RELATIONSHIP TO INTERRUPTIBLE DEMAND	16

V.	INCENTIVE	20

Vi.	MISCELLANEOUS Issues	22

Vii.	Recommendations	24

EXHIBITS LYS-1 THROUGH LYS-8

# i.	IDENTIFICATION OF WITNESS AND PURPOSE OF TESTIMONY

Q.	PLEASE STATE YOUR NAME AND BUSINESS ADDRESS.

A.	My name is Larry Shiao.  My address is 1560 Broadway, Suite 250, Denver, Colorado, 80202.

Q.	BY WHOM ARE YOU EMPLOYED AND IN WHAT CAPACITY?

A.	I am employed by the Colorado Public Utilities Commission (Commission or PUC) as a Professional Engineer.

Q.	HAVE YOU PREPARED A STATEMENT OF YOUR EXPERIENCE AND QUALIFICATIONS?

A.	Yes.  A summary of my qualifications, experience and duties is attached as Appendix A to this testimony.

Q.	ARE YOU GENERALLY FAMILIAR WITH THE FILINGS IN DOCKET NO. 07S-521E?

A.	Yes.

Q.	WHAT IS THE PURPOSE OF YOUR TESTIMONY?

A.	Public Service Company of Colorado (Public Service) has proposed a number of modifications to the Interruptible Service Optional Credit (ISOC) program.  My testimony addresses Staff’s concerns with the following aspects: (1) monthly credit rate derivation, (2) the monthly credit and its relationship to Interruptible Demand, (3) the requested financial incentive, and (4) a few miscellaneous issues.

Q.	PLEASE SUMMARIZE YOUR RECOMMENDATIONS.

A.	Staff recommends that the Commission order Public Service to:

1) Use seasonal capacity to compute the avoided capacity costs;

2) Use average MW to determine the Interruptible Demand;

3) Correct for errors in the equation that is used to calculate the monthly credit rates;

4) Use the current capacity availability factors to calculate the monthly credit rate;

5) Conduct additional studies to reflect the actual power system operation to evaluate the capacity availability factors;

6) Use all available interruptible hours absent a reasonable explanation for not doing so;

7) Address the cost-benefit analysis within the next six months.

# II.	PUBLIC SERVICE’S Interruptible service optional credit program (ISOC) AND THE CHANGES THAT IT IS PROPOSING

Q.	Please describe the PRINCIPLES BEHIND PUBLIC SERVICE’S ISOC program.

A.	Public Service offers an ISOC program that consists of three elements: 1) capacity interruption, 2) contingency interruption, and 3) economic interruption.  Public Service’s ISOC program allows program participants less than 10-minutes notice of an impending interruption or one hour or eight hours of notice.  Attracting interruptible load to the less than 10-minute notice option is valuable to Public Service because such load counts toward Public Service’s operating reserves.

Public Service can call a capacity interruption at any time upon determining that its generation or transmission capacity is insufficient to meet its load obligation.  Public Service can call a contingency interruption at any time when the Company believes that it will be unable to meet its disturbance control standard (DCS) criteria mandated by the Western Electricity Coordinating Council (WECC); however, a contingency interruption is only applicable to the ISOC program participants that selected the less than 10-minute notice option.  Public Service can call an economic interruption upon determining that an interruption will lower overall system costs when compared to overall system costs without an interruption.  Public Service pays a monthly bill credit to the program participants.  This credit is based on (1) a rate equation that calculates avoided capacity cost and avoided energy cost and takes into account the options selected by the program participant and (2) the program participant’s interruptible load.  The program participant receives its credit irrespective of whether Public Service fails to call sufficient interruptions to use up the interruptible hours that are subscribed to the ISOC program.

Q.	Why does Public Service offer the ISOC program?

A.	The ISOC program is a demand side management program that is designed to reduce load during the peak demand periods.  The program does not necessarily reduce overall energy consumption.  The capacity and contingency interruption elements of the ISOC program provide additional capacity resources to support and enhance the reliability of the Public Service power system operations without forcing the construction of another peaking plant or incurring fuel costs.  These types of interruptions represent the real value of the ISOC program.  The economic interruptions, on the other hand, are designed to lower the overall cost of providing energy to meet the energy demand of Public Service’s power system.  As compared to the value of a capacity or contingency interruption, economic interruptions represent a substantially lower value.  The ISOC program is beneficial to Public Service’s power system; however, the program needs to be designed correctly and be implemented properly in order to realize the expected benefit for all rate payers.

Q.	why has Public Service proposed to change the existing ISOC program?

A.	Public Service appears to have proposed changes to the ISOC program in an attempt to attract additional load to the program.  Public Service hopes to expand the program from its present subscription of 120 MW to 234 MW by 2020.  Public Service also proposes to attract additional load by introducing additional options, such as permitting use of an ISOC program participant’s energy management system (EMS) to effect an interruption, that it perceives as reducing barriers to participation.  Finally, Public Service believes that it should be entitled to a financial incentive if it can market and administer the program effectively.

Q.	is Staff concerned about Public Service’s proposed expansion of the ISOC program?

A.	No.  The revisions to the ISOC program proposed by Public Service should provide a net benefit to non-participants.  Currently, Public Service’s interruptible load program participants have a combined contract interruptible load (CIL) of about 120 MW.  Approximately 107 MW of the interruptible load is contributed by the less than 10-minute notice option participants.  The remaining interruptible load, about 13 MW, is the total CIL for the one-hour notice program participants.  Public Service witness Scott Brockett discussed Public Service’s plan for an interruptible load of about 243 MW in 2020.  This would be an increase of about 123 MW from the current interruptible load of 120 MW (See Scott Brockett Direct Testimony, page 16, line 23).  One can only speculate which options the new ISOC participants will select.  Likewise, it is a supposition that Public Service will be successful in its marketing of the program to achieve an increase of 123 MW by 2020.  Staff will continue to monitor the level of benefit to non-participants that is achieved from the anticipated attraction of interruptible load, including monitoring the level of the monthly credits paid to the ISOC program participants to achieve such load attraction.

Q.	What changes does PUBLIC SERVICE propose?

A.	Public Service’s proposed changes are:

(1) Reducing the eligible interruptible customer peak demand to 300 kW,

(2) Elimination of the 200-hour option,

(3) Elimination of the 8-hour notice option,

(4) Allowing participants to select from several more options,

a) Limiting interruptible hours to no more than 4 hours in 24 hours period;

b) No minimum 4-hours requirements;

c) Managing interruptions for participants, who select less than 10 minutes notice, by direct control of customer EMS,

(5) Allowing greater flexibility within the ISOC program (e.g., the use of a customer’s EMS to effect interruptions,

(6) Updating the monthly credit rates,

(7) Setting the ISOC credit to 80 percent of total avoided costs,

(8) Proposing that it retain about 12.5% of total ISOC credits as an incentive, and

(9) Miscellaneous changes to the Schedule ISOC tariff sheets.

Q.	Does Staff agree with Public Service’s proposed changes?

A.	Staff agrees with some of the proposed changes but not all.  Staff agrees with changes that will make the ISOC program more efficient and that will provide benefit to ratepayers at reasonable costs.  In general, Staff agrees with the proposed changes that will reduce the minimum load requirement to 300 kW and eliminating options not selected by current ISOC participants and unnecessary tariff language.

Staff has concerns with the changes that Public Service proposes to make to the monthly credit rate (MCR) equation, including the avoided cost derivation, and the proposed EMS option.

Staff strongly objects to Public Service’s proposed incentive.  In the following sections I will discuss Staff’s concerns and reasons why Staff does not believe Public Service is entitled to an incentive.

# III.	Monthly Credit Rate (MCR)

- Please describe the monthly credit rate (MCR) CALCULATION?

A.	The MCR equation is used to calculate the monthly credit rate Public Service is willing to pay to ISOC program participants.  While Public Service and Staff propose different equations, both parties use the same essential components.  The equation consists of two basic components: (1) the avoided capacity rate and (2) the avoided energy rate.  The avoided capacity rate component is the estimated, derived avoided capacity cost multiplied by a capacity availability factor (Ca).  The avoided energy rate is computed using a generic heat rate, a generic delivered gas price adjusted by the “energy rate paid by customers” and the variable operating and maintenance (VOM) rate to develop a energy rate in $/kWh.  The “energy rate paid by customers” is the sum of the base energy rate with general rate schedule adjustment (GRSA), the electric commodity rate (ECA) and the renewable energy standard adjustment (RESA).

## The Avoided CAPACITY Cost derivation

Q.	What ARE THE elements of Public Service’s PROPOSED avoided CAPACITY cost DERIVATION?

A.	Public Service has assumed that the interruption of a customer’s load within a short time after it gives notice would provide its electric system with characteristics similar to that of a combustion turbine (CT).  In general, a CT is a generation unit typically used to provide capacity to meet operating reserve and peaking demand requirements.  It is normally operated less than 400 hours or 5% of time in a given year.

Public Service assumes that the interruptible load would be able to (1) replace a CT for the purpose of maintaining the operating reserve; (2) defer the Company’s expenses of either constructing or purchasing a CT; and (3) reduce the Company’s costs of providing operating reserve.  In this filing, Public Service argues that the estimated costs of constructing a conventional frame CT, with a nameplate capacity of 160 MW, is a reasonable proxy for calculating the avoided capacity costs associated with the 1-hour notice option of the ISOC program.  For customers that select the less than 10 minutes notice option, Public Service proposes to use a different type of CT, a GE LMS100 with a nameplate capacity of 100 MW, as its proxy for calculating the avoided capacity costs.  Then for both notice options, Public Service uses the respective derived avoided capacity costs as the base cost to determine the monthly credit rate that it will pay to the ISOC program participants.

Q.	Does Staff HAVE ANY concernS with THE ESTIMATED CT CONSTRUCTION COSTS USED IN Public Service’s ANALYSIS OF the avoided CAPACITY costs?

A.	No.

Q.	CAN STAFF ADOPT THE OTHER ASSUMPTIONS CONTAINED IN PUBLIC SERVICE’s AVOIDED CAPACITY COST DERIVATION?

A.	No.

Q.	WITH WHICH ASSUMPTIONS DOES STAFF DISAGREE?

A.	Staff believes that in estimating the avoided capacity costs, Public Service should consider seasonal differences in the capacity rating of a combustion turbine.  While Public Service did use a seasonal ratio in its calculations, Public Service used only the summer capacity rating for its modeled CTs.  Staff believes that a more proper calculation would use the ISO-adjusted seasonally differentiated capacity ratings.  For example, the GE LMS100 CT has an estimated summer capacity of about 77 MW; however, the same GE LMS100 CT has an estimated winter capacity of about 91.6 MW.  Public Service’s method fails to accurately capture this approximately 20% seasonal differential in capacity in the calculation of avoided capacity cost.

Q.	WHAT AVOIDED CAPACITY COST CALCULATION DOES STAFF PROPOSE?

A.	Staff proposes the avoided capacity cost calculation set forth on Exhibit LYS-1.  Exhibit LYS-1 presents a comparison of Public Service’s existing and proposed avoided capacity cost ($/kW) calculations alongside Staff’s proposed avoided cost calculations.  These avoided capacity costs are presented for both the less than 10 minutes notice option (GE LM6000 CT for Public Service existing and GE LMS100 CT for Public Service proposed and Staff proposed) and the one-hour notice option (frame CT), respectively.  Staff’s recommendations eliminate the notification factor (Nf) and the seasonal ratio multiplier (115% or 90%) in the equation that Public Service proposes to use to calculate the monthly credit rate.

## THE CAPACITY AVAILABILITY FACTOR (Ca)

Q.	IS STAFF PRESENTING A NEW METHOD TO DETERMINE THE CAPACITY AVAILABILITY FACTOR?

A.	No.  Staff has been unable to develop a method to recommend to the Commission; however, through its investigation into the new method proposed by Public Service, Staff has determined that Public Service’s new method has significant flaws.  These flaws are described below.  Staff believes that the nature of these flaws should result in Commission rejection of the method proposed by Public Service.  Staff also recommends that the Commission reject the “4 hours in any 24-hour period” option.

Q.	how did Public Service derive THE capacity availability factor?

A.	Public Service uses a production cost simulation model called PROSYM and the forecasted load shapes and generation for 2007-2011 to develop marginal cost data for the 2007-2011 period.  Public Service also uses a separate PROSYM module to derive the loss of load probability (LOLP) using the same forecasted generation mix data and load shapes for 2007-2011.  Public Service then combines the marginal cost data and the LOLP values to derive a set of marginal and reliability cost data that it applies to the five-year historical period 2002-2006.

Q.	HAS STAFF IDENTIFIED ANY PrOBLEMS WITH THE PROSYM INPUTS USED IN PUBLIC SERVICE’S PROPOSED METHOD TO CALCULATE THE CAPACITY AVAILABILITY FACTOR?

A.	Yes.  First, Staff could not verify the capacity of the CT and the interruptible load used in the PROSYM study.  Public Service witness Alan Taylor described that he made a request to the Company to remove the interruptible load program from the base case; however, Staff could not verify the interruptible load in MW used in the PROSYM study.  Public Service’s discovery response on this subject shows that the interruptible load used in the PROSYM was a sum of “CFI Arc furnace and other interruptible load – including the Saver’s Switch load”.  (See Exhibit LYS–2)  The data in the Attachment to CPUC 12-3, however, shows that the total size of the CT used in the PROSYM for 2007 was about 115.29 MW including the Saver’s Switch load, which is about 4.7 MW less than the existing CIL amount of 120 MW.

Second, Public Service did not use the correct CT in the PROSYM study.  The CT used in the PROSYM study to derive capacity availability factors is an “appropriately sized” frame CT and not the GE LMS100 CT.  Public Service should have used the GE LMS100 CT because the vast majority of the interruptible load, about 90%, is contributed by the less than 10-minute notice option participants.  Staff raises this issue because the GE LMS100 CT has a much lower heat rate than a frame CT.  While Staff does not know whether a different type of CT in the PROSYM study will make a material difference to the resulted capacity availability factors, Staff believes in any event that the use of the “appropriately sized” frame CT was in error.

Q.	HAS STAFF IDENTIFIED ANY ADDITIONAL PrOBLEMS WITH THE PROSYM ANALYSIS USED IN PUBLIC SERVICE’S PROPOSED METHOD TO DERIVE THE CAPACITY AVAILABILITY FACTOR?

A.	Yes.  Public Service has introduced a lot of uncertainty into the derivation of the capacity availability factor.  Public Service uses forecasted load shapes from the five-year period 2007-2011 to derive marginal costs and LOLP values and then mixes this data with historical load shapes from the five-year period 2002-2006 to derive the various capacity availability factors it proposes.  Such mixing of data sets can produce skewed results, especially when the analysis focuses only on system demand the top 40, 80 or 160 hours in a year.

Q.	IS IT APPROPRIATE TO RELY ON MARGINAL COST DATA TO DEVELOP A CAPACITY AVAILABILITY FACTOR FOR PUBLIC SERVICE’S INTERRUPTIBLE PROGRAM?

A.	No.  Staff believes that Public Service’s complex and data intensive approach is inappropriately biased toward high demand hours because on-peak hours will always have a higher marginal cost.  An inappropriate bias exists because the capacity or contingency interruptions called by Public Service have not historically occurred during the highest demand hours of the year.  Examining actual interruptible events in the last three years, 2005, 2006, 2007, it is undisputed that neither a capacity interruption nor a contingency interruption was called on the peak day, which day has the highest marginal cost during the course of a year.  Moreover, Exhibit LYS-3 demonstrates that capacity or contingency interruptions have occurred randomly throughout the year.  Importantly,, the marginal and reliability costs developed by Public Service witness Taylor show a marginal cost of zero on certain dates of actual interruptions (for example February 18, 2006 and October 27, 2006).  The existence of a marginal cost of zero on these dates is not surprising as it reflects the fact that, during the shoulder and off peak months, Public Service’s power system has excess capacity available and has no need to run a CT.  Marginal costs, therefore, should not be a factor in determining the capacity availability factor.

Q.	DOES STAFF HAVE A SEPARATE BASIS FOR RECOMMENDING REJECTION OF THE “4 HOURS IN ANY 24-HOUR PERIOD” OPTION?

A.	Public Service’s proposed option to limit interruption to no more than 4 hours in any 24-hour period will limit the ability to call capacity and contingency interruptions as necessary.  It also reduces the value of the ISOC program because, if Public Service must end a capacity interruption as to certain ISOC program participants prematurely, then Public Service will either need to schedule rolling black outs or import expensive power.  Given that Public Service has frequently called capacity interruptions for longer than 4 hours, Staff views the no more than 4 hours in any 24-hour period as such a significant constraint that it should not be offered.

Q.	WHAT IS STAFF’S RECOMMENDATION REGARDING THE CAPACITY AVAILABILITY FACTOR?

A.	Staff is troubled that Public Service is attempting to derive the capacity availability factor using marginal cost analysis when the historical interruption data demonstrates no correlation to marginal costs and is therefore not representative of Public Service’s actual power system operations.  Therefore, Staff recommends that Public Service continue to use the capacity availability structure and factors set forth in the existing Schedule ISOC tariff as the best available derivation of the appropriate capacity availability factors.  Staff recognizes that Public Service would want to update these factors from time to time to reflect the actual power system operations and would likely recognize the need for such updates.

## THE AVOIDED ENERGY RATE

Q.	Is Staff concerned about the equation to compute the avoided energy rate?

A.	Staff is concerned that a load factor of 50% used by Public Service to convert the avoided energy cost in $/kWh into capacity cost in $/kW is unreasonably high for a CT.  Staff examined the monthly generation of CTs either owned by Public Service or under contract with Public service for the last thee years (2005, 2006, and 2007).  The data obtained from this examination is set forth on Exhibit LYS-4.  The results show that the weighted average load factor for CTs that can generate electricity on less than 10 minutes notice is 3.11%.  CTs that need more than 10-minute notice had an average weighted load factor of 13.65%.  The highest load factor for a CT was about 25%.  The historical load factors are better inputs into the avoided energy rate component of the monthly credit rate equation and should therefore be used.  However, because of the minimal impact on the total monthly credit rate to be paid to an ISOC customer, Staff recommends rounding the load factors up a little.  Staff recommends using a load factor of 5% for the less than 10-minute notice customers (based on the GE LMS100 CT) and a load factor of 15% for the one hour notice customers (based on the frame CT).

Another concern is the generic heat rate used in the calculation of the avoided energy cost.  Public Service proposes to use a generic heat rate of 10450 Btu/kWh for both the frame and the GE LMS100 CT.  In response to Staff’s discovery, Public Service indicated that the estimated heat rate for GE LMS100 CT is about 8862 Btu/kWh.  Staff believes that this heat rate should be used to calculate the avoided energy costs.

In summary, because the avoided energy, which is the benefit to non-participants, is very small as compared to the avoided capacity costs, the suggested changes will have insignificant impact to the monthly credit rate.  However, even though small, Staff recommends Public Service use the correct data to calculate the avoided energy costs.

## CONCLUSION REGARDING THE MCR

Q.	PLEASE SUMMARIZE YOUR DISCUSSION ON THE MCR

A.	Staff has put forth a number of suggested modifications to the MCR equation as well as some criticisms that might lay the groundwork for future additional modifications to the inputs to the MCR equation.  As a result of the modifications proposed here, Staff has developed the MCR equation that it believes should be used in the Schedule ISOC tariff.  Exhibit LYS-5 presents Staff’s proposed MCR equation alongside the equation presently in effect and the equation proposed by Public Service in this proceeding.

# IV.	THE MONTHLY CREDIT AND ITS RELATIONSHIP TO INTERRUPTIBLE DEMAND

Q.	How much credit did Public Service pay to ISOC PROGRAM PARTICIPANTS IN 2006 and 2007?

A.	Exhibit LYS-6 shows the actual percentage of each ISOC program participant’s bill attributable to its monthly interruptible credit for 2006 and 2007.  The percentages range from a low of about 3% to a high of about 33%.  As expected, the less than 10-minute notice ISOC program participants generally had a higher credit ratio than the one-hour notice participants.

Q.	HOW DOES PUBLIC SERVICE CALCULATE THE MONTHLY CREDIT?

A.	As stated in the tariff, the Monthly Credit is calculated by multiplying the monthly credit rate (MCR) by the lesser of the customer’s Contract Interruptible Load or the actual Interruptible Demand during the billing month.  Typically, the lesser amount is the Interruptible Demand.

Q.	HOW INFLUENTIAL IS THE INTERRUPTIBLE DEMAND COMPONENT TO THE AMOUNT OF THE BILL CREDIT PAID TO THE ISOC PROGRAM PARTICIPANTS?

A.	This component carries is very influential because it is probably the most individualized component of the monthly credit that is paid to ISOC program participants.  The actual Interruptible Demand carries a lot of weight because the program participant’s load curve forms the basis for this component.  Presently, the monthly credit paid out pursuant to the Schedule ISOC tariff looks only to the maximum 15-minute point in each peak hour to determine the actual Interruptible Demand.  By setting the Interruptible Demand in this manner, the ISOC program from a demand response perspective, values program participants with flat and fluctuating load curves equally even though a program participant with a flat load curve is much more valuable to the ISOC program.  It is because the ISOC tariff values customers with flat and fluctuating load curves equally that there is such a wide range in the percentage of the participant’s bill attributable to the monthly interruptible credit.

Q.	HOW DOES PUBLIC SERVICE PROPOSE TO DERIVE THE INTERRUPTIBLE DEMAND?

A.	In this filing, Public Service proposes to use the maximum daily one-hour integrated kW demands occurring between the hours of 12:00 noon to 8:00 pm Monday through Friday, excluding federal holidays.  By averaging the four data points obtained during the hour from the demand meter, Public Service is deemphasizing the program participant’s peak demand within each hour.  The proposed modification to the method for deriving Interruptible Demand (and, similarly, Contract Interruptible Load) is an improvement of the existing method, albeit a very modest improvement.

Q.	DOES PUBLIC SERVICE’S PROPOSED MODIFICATION TO THE METHOD BY WHICH IT DERIVES INTERRUPTIBLE DEMAND MAKE THE CALCULATION COMPORT WITH THE PUBLIC INTEREST?

A.	No.

Q.	WHY DOES STAFF BELIEVE ADDITIONAL CHANGES TO THE DERIVATION OF INTERRUPTIBLE DEMAND SHOULD BE MADE TO COMPORT WITH THE PUBLIC INTEREST?

A.	Staff has begun examining the load curves of the ISOC program participants to try to assess the relationship between the demand associated with the derivation of the Interruptible Demand and the actual demand.  Highly Confidential Exhibit LYS-7 shows the demand variation of six customers on July 24, 2006, a date when Public Service called a capacity interruption.  For customers with a very high load factor, the 15-minute demand meters provide adequate information about the demand to derive an actual Interruptible Demand that is reasonably representative of the customer’s actual loads.  However, for customers that have a low load factor and large demand swings in a relatively short period of time, the 15-minute demand meter cannot capture the load variation accurately enough.  If one then uses incorrect demand information, it is not possible to derive an actual Interruptible Demand that reasonably represents the customer’s actual loads.  For such customers, the actual Interruptible Demand component of the Monthly Credit is likely to be higher that the actual demand that can be relied upon in the event of an interruption.  Similarly, the ISOC program participant could be paid a Monthly Credit that significantly exceeds the value of that program participant to a demand response program like ISOC.

Q.	WHAT ADDITIONAL CHANGES TO THE DERIVATION OF THE INTERRUPTIBLE DEMAND COMPONENT SHOULD PUBLIC SERVICE MAKE?

A.	Staff recognizes that there are numerous ways to derive a reasonable interruptible demand.  Such methods could include performing a probability analysis to determine how much demand a program participant is likely to have when an interruption event commences or assessing a penalty if the customer has less load than predicted by the actual Interruptible Demand component of the monthly credit.  At this time and based on the information reviewed to date, Staff recommends deriving actual Interruptible Demand on the basis of the average kW for the billing month, which average kW is the total energy consumed in a month divided by the total number of hours in the month.

In conjunction with this proposal, Staff has examined the effect of this proposal on the current ISOC program participants.  The results of Staff’s analysis are depicted on Exhibit LYS-8.  To arrive at the percentages, Staff averaged the 2006 and 2007 data for each of the four months presented.  Further, for ease of presentation Staff has used each ISOC program participants Contract Interruptible Load as a proxy for its actual Interruptible Demand.

Staff believes that this is a fair method to determine how much demand reduction an ISOC program participant can be expected to actually provide in the event an interruption is called.

Q.	IN CONJUNCTION WITH YOUR CRITICISM OF THE INTERRUPTIBLE DEMAND COMPONENT OF THE MONTHLY CREDIT, DOES STAFF HAVE ANY RELATED RECOMMENDATIONS?

A.	Yes.  It appears that the 15-minute demand meter may not be able to record peak demand in an effective manner for purposes of the ISOC program bill credits.  While the existing 15-minute demand meter may provide good data for billing purposes, it limits Public Service’s ability to accurately record the ISOC program participant’s load when an interruption event occurs.  Staff suggests that Public Service explore the possibility of replacing all 15-minute demand meters for ISOC program participants that select the less than 10-minute notice option to a meter that will have the capability to accurately record demand in less than 10-minute intervals and will provide “granularity” of demand information for Public Service to manage the ISOC program effectively.

# V.	INCENTIVE

Q.	What is the incentive proposed by Public Service?

A.	Public Service proposes to retain 12.5 % of the ISOC credit as an incentive if it can successfully market the ISOC program.

Q.	Did Public Service propose any criteria to obtain the incentive of 12.5 %?

A.	No.  Public Service does not propose a method or criteria to calculate the proposed incentive of 12.5% of the ISOC monthly credit.  Furthermore, Public Service does not propose any way to determine or measure the success of the ISOC program.

Q.	Is the 12.5% INCENTIVE to PUBLIC SERVICE fair and reasonable to rate payers?

A.	Staff is very concerned about this self serving assumption.  Rate payer funds, not shareholder funds, are used to provide the credits paid to the ISOC program participants.  The payment to the ISOC program participants is based on the estimated costs of the ISOC program.  There should not be any excess funding of this program.  Any artificial increase in the evaluation of ISOC credit payment is wrong and will increase the DSMCA rate paid by ratepayers.

Q.	WHAT OTHER REASONS EXIST FOR STAFF’S OPPOSITION TO THE INCENTIVE PROPOSED BY PUBLIC SERVICE?

A.	Staff believes that Public Service is mistakenly considering the ISOC as another DSM program that would reduce the peak demand permanently.  Unlike DSM programs such as the energy efficiency program, which may cause a long-term load reduction in customer’s demand, the ISOC program is utilized when the power system is under stress or in an unexpected contingency situation.  Staff agrees that DSM programs that produce permanent reduction in demand should have some mechanism or provision to provide incentive to Public Service in return for promoting efforts to reduce load and in turn revenue.  However, the interruptible load program is not a program that reduces load permanently.  The ISOC program essentially causes program participants to use power at a different time period or during a period when the power system is not under stress or under a capacity shortage situation.  As compensation for the occasional and infrequent inconvenience, Public Service is offering to pay a monthly credit, using funds collected from ratepayers, to commercial and industrial customers that choose to participant in the ISOC program.  The customer’s peak load and energy consumption will remain unchanged unless customers change existing equipment.

Q.	DOES Public Service receive any benefit FROM OFFERING THE ISOC PROGRAM EVEN IN THE ABSENCE OF AN INCENTIVE?

A.	Yes.  Public Service will benefit by reducing its operating reserve requirements and by having additional flexibility to operate its power system.  By leveling power usage, Public Service increases its load factor and system efficiency.  In addition, it will cost Public Service less to serve the ISOC program participants’ load because most of the load will shift to light load or off peak periods.  Further, Public Service will either not have to serve the load during peak periods or, if a buy-through is elected, will be able to charge the ISOC program participant for the highest cost block of electricity consumed in each buy-through hour.

# Vi.	MISCELLANEOUS Issues

Q.	what other issues cause concern to Staff?

A.	The Commission, in Decision No. C07-0559, ordered the ISOC workshop participants to address the following issues: (1) optimizing the use of available interruptible hours, and (2) the cost and benefit of the ISOC program.

Q.	PLEASE DISCUSS THE OPTIMIZATION OF AVAILABLE INTERRUPTIBLE HOURS.

A.	Public Service proposes that if the Company uses at least 80 % of the ISOC program participant’s potential annual hours of interruption then such usage of hours will be automatically deemed prudent and entitle Public Service, all other things being equal, to fully recover the monthly credit payments through the DSMCA it will not require the Company to justify its action.  Staff does not believe this is a reasonable expectation.  Staff believes that Public Service should make every effort to use all available interruptible hours within the year.  An 80% utilization of interruptible hours without further justification is an unacceptably low target.  Staff, however, does not demand perfection; Staff believes that Public Service should however be required to provide a justification to the Commission each year that it does not use up all available interruptible hours.  The Commission would then determine whether Public Service should be entitled to full recovery of the monthly credit payments on the facts presented.

Q.	PLEASE DISCUSS PUBLIC SERVICE’S TESTIMONY REGARDING THE COST AND BENEFIT OF THE ISOC PROGRAM.

A.	Public Service did not “evaluate the usefulness and purpose of preparing a cost-benefit analysis relating to the ISOC program and methodology to be used in preparing such an analysis” as ordered by the Commission in Docket No. 06S-642E.  In Public Service’s 2005 ISOC cost and benefit analysis report, Public Service stated that “intermittent generation resources will play a principal role in the valuation process for the ISOC program as they have a direct impact on the system reserve dynamic.”  Public Service has acknowledged that the addition of wind resources have highlighted the need to have resources that can respond to load drops in 10 minute and even one hour time frames.  But it is unclear how much of these load drops can be reliably covered by ISOC subscriptions.  Staff recommends that Public Service conduct a study within the next six months to evaluate and quantify the benefit contributed by the ISOC program to the operation of the Company’s power system and determine correctly the responsiveness of this demand to a request for reduction.

# Vii.	Recommendations

Q.	Please summarize your recommendations

a.	Staff recommends the Commission order Public Service to:

1) Use seasonal capacity to compute the avoided capacity costs;

2) Use average MW to determine the Interruptible Demand;

3) Correct for errors in the equation that is used to calculate the monthly credit rates;

4) Use the current capacity availability factors to calculate the monthly credit rate;

5) Conduct additional studies to reflect the actual power system operation to evaluate the capacity availability factors;

6) Use all available interruptible hours absent a reasonable explanation for not doing so;

7)  Address the cost-benefit analysis within the next six months.

Q.	DOES THIS CONCLUDE YOUR TESTIMONY AT THIS TIME?

A.	Yes.

The estimated avoided costs data was from Table 39, Cost and Performance Characteristics of New Central Station Electricity Generating Technologies in 2007 annual report published by Energy Information Administration.

In the event the Commission rejects Staff’s recommendation and retains the no more than 4 hours in any 24-hour period option, the Schedule ISOC tariff sections entitled Capacity Interruptions and Contingency Interruptions both need to be amended to include the following language (probably as the second sentence):  “The Company’s ability to call a Capacity [Contingency] Interruption shall not exceed 4 hours in any 24-hour period if Customer has elected that option.”