Experimental Residential Price Response Pilot Program

March 2008 Update to the 2007 Final Report

Reason for and Impact of March 2008 Update – The Company discovered one of the data files used in the 2007 Final Report contained some erroneous information.  The data from September 2006 – July 2007 was provided in kW for analysis. During the data transfer, half of the file became unusable. The replacement data was sent in kWh by mistake. Following our filing of the original report, the error was identified and the corrected data file yielded different results.  All information in this updated report has been modified to reflect the new results.  The changes do not materially affect the report.  The fundamental conclusions remain the same.  The cost/benefit results are the same in that in all cases the pilot still failed the Utility Cost Test, the Rate Impact Test and the Total Resource Cost test.  As an example of the impact, from a percentage perspective, the best response was still the reduction in usage for the CTOU Critical Peak Period enabled by a programmable thermostat.  However, the percent reduction dropped from the 61.25% reported in the 2007 Final Report to 54.22% in this updated report.  All other such changes may be found in the body of this updated report.

In the original report, we stated it is unclear how much of the energy savings are attributable to pricing signals versus other aspects of the program (such as customer education) given the off-peak savings achieved.  Using the new data, the energy reduced during off-peak periods on event days and non-event summer weekdays is less significant.  We now conclude that the majority of the energy savings can be attributed to customers’ response to pricing signals.

# Docket No. 04A-566E

The Company proposed to test the feasibility and effectiveness of dynamic pricing through the Experimental Residential Price Response Pilot Program (Pilot).  In its proposal, the Company presented a four-phased approach to the Pilot, which includes the Test Phase, Enrollment and Baseline Phase, Price Signal Phase, and Evaluation Phase.  The following report summarizes the Company’s results of the Pilot.

## EXECUTIVE SUMMARY

The objective of the CO Residential Price Response Pilot was to design price response options that encourage residential customers to consume less energy during peak conditions.  Also, to provide customers more control over their energy bills by reducing consumption during peak periods when electricity is more expensive.

The Pilot was designed to measure customer response only.  It was not designed to test technology, meaning that the technology used in the pilot is not anticipated for a scale launch of the pilot.  The cost of this equipment is substantial and does not pass the cost benefit analyses provided. The pilot was structured on the assumption that participants would not pay the incremental cost of the meters or enabling technologies.  Participation in the Pilot was voluntary.

The evaluation of the Pilot includes quantifying participant response to Pilot prices.  The evaluation includes measurement of demand and energy reductions during critical peak and on-peak periods; seasonal and annual energy change in on-peak and off-peak usage; and demand reduction during monthly system peak hours (avoided peak capacity requirements).  As participation in the Pilot was voluntary, we also identified possible self-selection bias.

The Pilot included three price options: Time of Use (RTOU), Critical Peak (RCPP), and a combination rate Critical Peak/Time of Use (RCTOU).  RTOU treatment participants pay a higher on-peak price for any energy consumed during on-peak periods and a lower off-peak price for energy consumed at all other times.  RCPP treatment participants pay a critical peak price for energy consumed during the peak periods on up to 10 summer days (called Critical Peak days) and a lower off-peak price for energy consumed at all other times.  RCTOU treatment participants pay a higher on-peak price for energy consumed during on-peak periods (though less than the RTOU option), a critical peak price for energy used during peak periods on up to ten critical peak days, and a lower off-peak price for energy consumed at all other times.

On-peak and critical peak periods occur during summer weekdays (except holidays) between 2 and 8 p.m.  Summer is defined as the months of June, July, August and September.  Off-peak prices were in effect all other hours of the year including non-peak weekday hours, holidays, weekends, and all hours during the winter months.  RCPP and RCTOU participants were notified of the 10 critical peak events by 4 p.m. the day before.

# KEY FINDINGS

Six Price Response Options were evaluated; (1) TOU, (2) CPP with no technology deployed to enable the customer response, (3) CPP with customer response enabled with a Control Switch on the Customer’s AC, (4) CTOU with no customer response enablement, (5) CTOU with customer response enabled with the AC Control Switch, and (6) CTOU enabled with a Programmable Thermostat.

Seven key findings have emerged from the Pilot:

- Customer response for all six options was statistically significant. For all options usage was reduced and load leveling occurred.  From a percentage perspective, the best response was a 54.22% reduction in usage for the CTOU Critical Peak Period enabled by a programmable thermostat.

- A primary purpose of most Customer Response studies is to measure the level of response to price signals.  The standard method to calculate these responses is “own price elasticities” [i.e., how much a customer will change their usage as the result of them comparing the previous rate to the new rate for the rate period in question].  These elasticities are then used to predict customer response when deployed.  The study yielded price elasticities, which were used to update the required Participant Test, Rate Impact Test, Utility Cost Test, Total Resource Cost Test, cost effectiveness tests.

- In addition to price signals educating our customers also created reductions or shifts in customer usage.  This is evidenced by some reductions in off-peak usage even though the annual off-peak rate was below the standard rate, which would infer an increase in off-peak usage in every case.  These reductions in off-peak usage ranged from –0.08% for CPP to –2.95% for TOU.  Although the amount of reduction is not significant, any reduction when rates go down would be a net effect of the expected usage increases from the rate reduction and off-sets general conservation reductions instituted by the customer across the board.

- Given the level of customer response, the meter technology that was used for the pilot is too expensive to make the offerings cost-effective.

- The Pilot confirmed that customers will “self-select” based on their own economics.  Pilot results reflect the combined impact of customer selection and usage reductions and shifts.

- Positive results were obtained from the enabling technologies, i.e. the AC Control Switch and the Programmable Thermostat improved critical peak response for both CPP and CTOU 6% and 18% respectfully.  In like manner the thermostat improved CTOU response by 8%.

- The Pilot successfully met its objective to provide PSCO usable information as to how customers will respond to price signals and education.  Therefore no additional customer response measurements or additional market testing is required.

The pilot was conducted with the understanding that meter costs are not covered in the charge to the participants, but are treated as utility project costs, to be recovered through the Colorado DSMCA.

# RESULTS ANALYSIS

Per the requirements of the Settlement Agreement, which authorized this pilot, this final report includes the following results analysis for all six pilot service options:

- Measure Customer Acceptance – PSCO and the PUC identified two components for measuring customer acceptance:

  - Identifying the percent of customers that were willing to volunteer for the pilot, or “the initial adoption rate”

  - Identifying the number of those customers who adopted the offering who subsequently left the pilot, or the “attrition rate”.

Total sample size was 307,000 customers.  4,400 volunteered to participate in the pilot.  Response rate = 1.4%  (See Attachment A for more detail.)

3,700 were enrolled in the Pilot.  Final number of participates at Pilot conclusion on July 14, 2007 was 2742.  Attrition rate equals 26%.

- Measure Effectiveness for the Summer Peak – PSCO and the PUC defined two ways to measure the effectiveness for the Summer peak:

  - The amount of usage reduction, or the reduction in On-Peak quantities

  - Improvement in Consumption Efficiency, or the quantity that shifts to Off-Peak Usage Periods thus creating load leveling

See ATTACHMENT B for result details.  The table shows that for all six options usage was reduced and consumption efficiency improved for both Critical Peak and On-peak Periods.

For the Critical Peak [Critical Peak Days] portion of the CPP offering the reduction was 32% for customers without central AC and 38% for customers with central AC.  Adding the AC Control Switch increased the usage reduction by 7% to 45%.

For the Critical Peak Days and On-Peak portions of the CTOU offering there was a significant difference with and without AC.  For Critical Peak Days without AC the reduction was 15%, with AC it was 29% and when the AC Control Switch was introduced, the reduction for Critical Peak days was similar to that for CPP, or another 18% jump to 47%.  For On-Peak, AC participants reduced by 8%.  For On-Peak both the Switch and the Thermostat caused about the same additional reduction in usage of about 2%.

For Off-Peak, with the Off-Peak rate about 20% below the Standard Tariff rate, as mentioned previously, small reductions occurred for TOU and CPP customers without central AC. In all other cases, increases occurred, ranging from .59% to 8.69%.  There was no clear pattern between AC and no AC nor with or without enabling technologies.  This lack of pattern is probably attributable to the small usage reductions occurring from the impact of education interacting with the small usage increases occurring in response to the rate decrease.

- Create a Price Elasticity Index

See ATTACHMENT C for the Price Elasticity Index.  This index of elasticities was used to run the tests for cost effectiveness.

# CONCLUSIONS

Customer response for all six options was statistically significant. For all options usage was reduced and load leveling occurred. The Pilot successfully met its objective to provide PSCO usable information as to how customers will respond to price signals and education.  Therefore no additional customer response measurements or additional market testing is required.  However, given available meter technologies, no option was cost effective with the advanced metering technology deployed for the pilot (which was the only technology available at the time that could support the pilot parameters such that customer response rates could be identified).  Until cost effective metering options can be identified and the technology tested, no option should be deployed.

The Price Signal Phase was in effect for part of two summers.  Peak (both critical peak and on-peak) periods occurred between July 15, 2006 and September 30, 2006 and between June 1, 2007 and July 14, 2007.