Decision No. C11-0251

BEFORE THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO

DOCKET NO. 11A-055R

IN THE MATTER OF THE APPLICATION OF THE REGIONAL TRANSPORTATION DISTRICT FOR AUTHORITY TO CONSTRUCT AND OPERATE AN AT-GRADE CROSSING AT RTD'S WEST CORRIDOR AND ZUNI STREET IN THE CITY AND COUNTY OF DENVER, STATE OF COLORADO.

commission order deeming application complete and granting application

Mailed Date:  March 10, 2011

Adopted Date: March 9, 2011

## by the commission

### Statement

- This matter comes before the Commission for consideration of an application (Application) filed by the Regional Transportation District (RTD) on January 20, 2011, seeking authority to modify an existing at-grade highway-rail crossing of Zuni Street with the proposed RTD West Corridor including installation of two new tracks, and installation of active warning devices consisting of flashing lights, gates, northbound cantilever flashing lights, northern and southern median, bells, and mast mounted signage, National Inventory No. 244705R, in the City and County of Denver, State of Colorado.

- Notice of the Application was provided by the Commission to all interested parties, including adjacent property owners pursuant to § 40-6-108(2), C.R.S., on January 27, 2011.

- No interventions were filed in this matter.

- The Commission has reviewed the record in this matter and deems that the Application is complete within the meaning of § 40-6-109.5, C.R.S.

- Now being fully advised in the matter, we grant the Application.

### Findings of Fact

- The Commission gave notice to all interested parties, including the adjacent property owners.  No intervention was received opposing the Application.

- RTD proposes to modify an existing at-grade highway-rail crossing of Zuni Street with the proposed RTD West Corridor in Denver, Colorado.  RTD’s Application states that the crossing currently has no National Inventory Number.  However, a review of the Federal Railroad Administration indicates that Crossing No. 244705R exists at the location at which RTD seeks to construct its track and make its improvements.

- RTD proposes to install two tracks and active warning devices at the Zuni Street crossing consisting of mast mounted signs, gates, flashing lights, northbound cantilever flashing light signals, northern and southern medians, and bells.  Exhibit B-1 shows that the R15-2 “2 Tracks” sign will be posted.  However, Exhibit D does not show the R15-2 in conjunction with the R15-1 crossbucks signs.  We will require RTD to post the R15-2 “2 Tracks” signs at this crossing.

- In reviewing RTD’s proposed signing and striping, RTD shows the addition of a mast mounted R15-8 “Look” sign.  Exhibit B-1 shows the R15-1 crossbuck sign, R15-2 “2 Tracks” sign, in addition to a blank out sign that would illuminate warning motorists a second train is approaching the crossing, and a sign warning motorists of high speed trains.  These signs are in addition to the mast mounted flashing lights and gate arm assembly attached to the post.  Additionally, RTD has proposed the use of this sign at one other crossing on the West Corridor, the Decatur Street crossing, for which application has been made to the Commission to date.  The Commission disallowed the posting of this sign at Decatur Street by Decision No. C11-0225 on March 2, 2011 in Docket No. 11A-053R.

- Our review of the Manual on Uniform Traffic Control Devices shows the R15-8 “Look” sign as an optional sign and not a required sign.  Given the amount of information that is already being given to drivers at the crossing with the required R15-1 and R15-2 regulatory signs, and the blank out and high speed train signs, the additional sign is not necessary and may overwhelm drivers.  RTD shall not be allowed to post the R15-8 “Look” signs at this crossing.

- RTD states that the current average daily traffic volume at the proposed Zuni Street crossing is 3,270 vehicles per day (VPD) at a 35-mile per hour (MPH) speed limit with projected growth estimated at 3,600 VPD in five years.  Rail operations through the crossing are anticipated to consist of approximately 294 light rail vehicle movements at a maximum speed of 25 MPH when operation of the West Corridor commences in 2013 with no further projections for growth in the future at this time.

- RTD estimates the cost for the modified Zuni Street crossing at $167,577 for the civil engineering work and $390,860 for the signal work.  RTD will be paying for 100 percent of the cost of the construction.

- RTD proposes to start construction in March 2011 and complete construction by December 2012 with revenue operations commencing sometime in 2013.

- RTD will be required to inform the Commission in writing that the crossing work is complete and operational within ten days of completion.  The Commission will expect this letter by December 31, 2012.  However, the Commission does understand this letter may be provided earlier or later than this date depending on changes or delays to the construction schedule.

- We will require RTD to update the crossing inventory form for this crossing.  The Commission will expect this information to be filed with completion of the crossing work by December 31, 2012.

- Once construction of the new crossing is complete, RTD will maintain, at its expense, all track, appurtenances, and warning devices.  The City and County of Denver will be required to maintain the roadway approaches to the end of tie of the crossing, advance warning signs, and pavement markings.

### Conclusions

- The Commission has jurisdiction in this matter under §§ 40-4-106(2)(a) and (3)(a), C.R.S.

- No intervenor that filed a petition to intervene or other pleading contests or opposes the Application.

- Because the Application is unopposed, the Commission finds that it will determine this matter upon the record, without a formal hearing under § 40-6-109(5), C.R.S., and Rule 1403, Commission Rules of Practice and Procedure, 4 Code of Colorado Regulations 723-1.

- We find that good cause exists to grant the Application consistent with the above discussion in paragraphs 7 through 16.

## ORDER

### The Commission Orders That:

- The application (Application) filed by the Regional Transportation District (RTD) on January 20, 2011, seeking authority to modify an existing at-grade highway-rail crossing of Zuni Street with the proposed RTD West Corridor including installation of two new tracks, and installation of active warning devices consisting of flashing lights, gates, northbound cantilever flashing lights, northern and southern median, bells, and mast mounted signage, National Inventory No. 244705R, in the City and County of Denver, State of Colorado is deemed complete within the meaning of § 40-6-109.5, C.R.S.

- The Application is granted.

- RTD is authorized and ordered to proceed with the construction of the crossing upgrades at the existing at-grade crossing of Zuni Street and the proposed West Corridor including construction of a two-track crossing and installation of active warning devices consisting of flashing lights, gates, northbound cantilever flashing lights, northern and southern medians, bells, and mast mounted signage in the City and County of Denver, Colorado.

- The City and County of Denver shall maintain the roadway approaches up to the end of tie, pavement markings, and advance warning signs at the crossing at its expense pursuant to Rule 4 Code of Colorado Regulations (CCR) 723-7-7211(c), Rules Regulating Railroads, Rail Fixed Guideways, Transportation by Rail, and Rail Crossing.

- RTD shall post R15-2 “2 Tracks” signs at the Zuni Street crossing.

- RTD shall not post R15-8 “Look” signs at the Zuni Street crossing.

- RTD shall maintain its new track, rails, ties appurtenances, crossing surface, and active warning equipment at its expense pursuant to Rule 4 CCR 723-7-7211(a).

- RTD shall inform the Commission in writing that the crossing is installed and operational within ten days of completion.  The Commission will expect this letter by December 31, 2012.  However, the Commission understands this letter may be provided earlier or later than this date depending on changes or delays to the construction schedule.

- RTD shall be required to file a copy of the crossing inventory form showing the crossing updates by the end of construction on December 31, 2012.

- The 20-day period provided for in § 40-6-114, C.R.S., within which to file applications for rehearing, reargument, or reconsideration, begins on the first day following the effective date of this Order.

- The Commission retains jurisdiction to enter further orders as necessary.

- This Order is effective on its Mailed Date.

### ADOPTED IN COMMISSIONERS’ WEEKLY MEETING
March 9, 2011.

| (S E A L) ATTEST: A TRUE COPY Doug Dean, Director | - THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO RONALD J. BINZ ________________________________ JAMES K. TARPEY ________________________________ - MATT BAKER ________________________________ - Commissioners |
| --- | --- |