Questions for CDOT for Delta County Road 1600

- There are a number of differences between traffic signal controller values shown in the original preemption calculations and the Affidavit preemption calculations.  These values include:

Item							Original	Affidavit

Preempt delay time (sec)				5		0

Controller response time to preempt (sec)		1		0

Min. green time during ROW transfer (sec)		10		0

Ped. clearance time during ROW transfer (sec)	23		0

Veh. Yellow change time				5 (4 earlier)	0

Veh. Red clearance time				1 (2 earlier)	0

Clear storage distance (feet)				45		56

Minimum track clearance distance (feet)		120		50

Multiplier for max APT due to train handling	1.5		1

Portion of CSD to clear (feet)				45		56

- Please provide an explanation of when these values changed.

- Please provide an explanation of why these values changed.

- Please state if the original values were initially used at the crossing and then changed to the new values at a later date, if values different from the original values were used and then changed to the new values at a later date, or if the new values were used all along.

- There are a number of differences between traffic signal controller values shown in the original preemption calculations and the Affidavit preemption calculations.  These values include:

Item							Original	Affidavit

Design vehicle						WB-50		WB-67

Design vehicle length (feet)				55		73.5

Time for DV to accelerate through DVCD		18		15.20

Proportion of non-interaction gate descent time	0.3		0.22

- Why was the design vehicle changed?

- Did the change to the design vehicle perpetuate throughout the design of the intersection/crossing/traffic signal or was the original design vehicle length used in design of the intersection/crossing/traffic signal?

- Figure 2 of the Texas DOT Preemption Form Instructions does not contain information for a WB-67.  Please explain how you determined the time for the design vehicle to accelerate through the design vehicle clearance distance.

- Figure 5 of the Texas DOT Preemption Form Instructions does not contain information specific for a WB-67.  Please explain how you determined the proportion of non-interaction gate descent time.

Questions for UPRR for Delta County Road 1600

- There are a number of differences between the values provided by the railroad used in the calculations for the original preemption calculations and the Affidavit preemption calculations.  These values include:

Item							Original	Affidavit

Clearance time (sec)					9		0

Advance preemption time (sec)			0		49

Duration of flashing lights (sec)			4		3

Full gate descent time (sec)				10		12

- Please provide an explanation of when these values changed.

- Please provide an explanation of why these values changed.

- Please state what times were used in designing and constructing the crossing signal equipment at the subject crossing

- Have any changes been made to what was originally designed and constructed at the crossing?

- Please state what values were originally used at the crossing, and what values currently exist at the crossing.