# Decision No. C15-0402

BEFORE THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO

PROCEEDING NO. 15V-0137R

PETITION OF THE REGIONAL TRANSPORTATION DISTRICT FOR WAIVER OF, OR VARIANCE FROM, 4 CCR 723-7 7325(G)(III) CONCERNING THE SIDE CLEARANCE OF FOUR SIGNALS IN THE CITY OF AURORA, ADAMS COUNTY, STATE OF COLORADO.

commission decision 
GRANTING Variance

Mailed Date:  	April 30, 2015

Adopted Date:  	April 29, 2015

## BY THE COMMISSION

### Statement, Findings, and Conclusions

- This matter comes before the Commission for consideration of a petition filed on March 6, 2015, by the Regional Transportation District (RTD).  In this petition, RTD requests a permanent partial waiver of, or variance from the 8’-6” minimum side clearance standard stated in the Rules Regulating Railroads, Rail Fixed Guideways, Transportation by Rail, and Rail Crossings, 4 Code of Colorado Regulations (CCR) 723-7-7325(g)(III) to a 6’-6” minimum clearance for three wayside signals and 6’-0” for one wayside signal located in the City of Aurora, Adams County, State of Colorado.

- The Commission noticed this petition to all interested persons, firms, and corporations pursuant to § 40-6-108(2), C.R.S., on March 13, 2015.

- No interventions were filed in this matter.

- No petition to intervene in opposition or otherwise participate in this proceeding has been filed.  This proceeding is therefore uncontested.  Pursuant to § 40-6-109(5), C.R.S., this matter may be determined without a hearing.

- Pursuant to Rule 1003 of the Rules of Practice and Procedure, 4 CCR 723-1, the Commission may grant waivers or variances from tariffs, Commission rules, and substantive requirements contained in Commission decisions and orders for good cause.  In making a determination, the Commission may take into account, but is not limited to, considerations of hardship, equity, or more effective implementation of overall policy on an individual basis.

- Rule 7325(g)(III) requires that generally, side clearances for signals and signal poles higher than 3-’0” above top of rail shall, for the portions thereof that are higher than 4’-0” above top of rail and less than 16-’0” above top of rail, keep such portions 8’-6” from the center of the track.

- There are four locations along the East Corridor that RTD is requesting a reduced side clearance due to various construction constraints.  At Sable Signal 6N, there is a ballast wall with fence located 8’-6” from the centerline of track and RTD does not have rights to mount the signal on the back side of the wall.  RTD requests a reduced side clearance at this location of 
6’-6”.  At Sable Signal 2S, the signal is located in between two tracks where there is insufficient clearance to meet the 8’-6”.  RTD requests a reduced side clearance at this location of 6’-6”.  At Chambers Signal 2N, the signal is also located in between two tracks where there is insufficient clearance to meet the 8’-6”.  RTD requests a reduced side clearance at this location of 6’-0”.  At Chambers Signal 6N, there is a wall located 8’-6” from the centerline of track and RTD does not have rights to mount the signal on the back side of the wall.  RTD requests a reduced side clearance at this location of 6’-6”.  These four signals provide train operators information needed for safe operations within the interlocking system and the four locations above provide the most ideal locations to communicate this information to train operators.

- RTD requests a permanent variance of the side clearance for the above four locations to the minimum side clearances listed.  RTD states that it believes that the 
8’-6” clearance was originally enacted in the days where railroad employees performed some of their duties on the outside of railcars, and that no passengers or employees will be allowed to ride on the outside of the RTD vehicles at these locations.  Additionally, RTD provides information showing that the signal locations are outside of the sway of the operating commuter rail vehicles operating at a speed of 79 miles per hour (MPH).  Finally, at the Chambers Signal 2N where the minimum side clearance is requested, the maximum speed of the commuter rail vehicles will be ten MPH, which will reduce the sway of the operating vehicle even further.

- The Commission finds that RTD has shown good cause to grant a permanent variance of Rule 7325(g)(III) to allow reduced minimum side clearances from the centerline of track as discussed above along the East Corridor in Aurora, Colorado.  Because no passengers or RTD personnel will be allowed to ride on the exterior of the vehicles and because the dynamic envelope of the vehicles will be less than the requested reduced minimum side clearances, there is not a public safety issue with this request.

## ORDER

### The Commission Orders That:

- The Regional Transportation District is granted a permanent variance of Rule 7325(g)(III) of the Commission’s Rules Regulating Railroads, Rail Fixed Guideways, Transportation by Rail, and Rail Crossings, 4 Code of Colorado Regulations 723-7 to a 
6’-6” minimum clearance at Sable Signal 6N, Sable Signal 2S, and Chambers Signal 6N, and 
6’-0” for Chambers Signal 2N located in the City of Aurora, Adams County, State of Colorado.  This permanent variance shall be for the life of the commuter rail maintenance facility.

- The 20-day time period provided by § 40-6-114(1), C.R.S., to file an application for rehearing, reargument, or reconsideration shall begin on the first day after the Commission mails this Decision.

- This Decision is effective on its Mailed Date.

### ADOPTED IN COMMISSIONERS’ WEEKLY MEETING
April 29, 2015.

| (S E A L) ATTEST: A TRUE COPY Doug Dean, Director | - THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO JOSHUA B. EPEL ________________________________ PAMELA J. PATTON ________________________________ - GLENN A. VAAD ________________________________ - Commissioners |
| --- | --- |