# Decision No. C16-0742

BEFORE THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO

PROCEEDING NO. 16I-0605R

IN THE MATTER OF THE regional transportation district’s corrective action plan regarding the nine mile station derailment, rule 4 ccr 723-7-7346.

COMMISSION DECISION OPENING 
INVESTIGATION AND REQUIRING 
FILING OF CORRECTIVE ACTION PLAN

Mailed Date:  	August 11, 2016

Adopted Date: 	August 10, 2016

## by the commission

### Statement

- On June 13, 2016, a Regional Transportation District (RTD) light-rail vehicle derailed at the Nine Mile Station executing a crossover maneuver into the Nine Nile Station at Signal S156PR/Switch SE380B.  This maneuver was required as part of the new configuration and cut-in work happening in the Nine Mile Station tail tracks for the new R-Line.  The operator requested permission from Control to bypass a red signal, which was granted.  As the train attempted to make the cross-over maneuver, the switch began moving due to a loss of communication between the Supervisory Control and Data Acquisition (SCADA) system and the signal controller at the same time the train shunted the over-switch circuit.  This caused the switch to stop its movement mid-event and caused the LRV to derail as it passed the switch.  There were no injuries or fatalities as a result of this accident, and there was approximately $6,000 total damage to the light-rail vehicle.  This initial accident report was filed with the Commission on July 29, 2016, but has not yet been adopted by the Commission.

- RTD’s preliminary investigation of this incident indicates five contributory factors including: 1) Non-routine operations at Nine Mile Station to accommodate the R-Line tie-in work to the existing tail tracks; 2) Unstable Central Train Control (CTC) communication link to RCH1569PR, which allowed the switch control to drop from the CTC to an automatic mode; 3) SCADA software upgrades to the existing system from the R-Line, which resulted in the loss of the Control Center’s ability to select the train route via SCADA (the SCADA system could request a switch movement, but could not set a locked route movement); 4) the LRV operator failed to confirm switch alignment prior to the movement; and 5) the LRV operator entered the interlocking at a higher than authorized speed (14 miles per hour (MPH) as opposed to the maximum 10 MPH).  RTD has developed a hazard analysis that is numbered H1-06132016 and draft Corrective Action Plan (CAP) for this accident.

- Pursuant to 4 Code of Colorado Regulations 723-7-7346 of the Rules Regulating Railroads, Rail Fixed Guideways, Transportation by Rail, and Rail Crossings, we are opening this investigation proceeding.

- RTD is required to file the CAP it has developed and associated hazard analysis H1-06132016 in this investigation proceeding within 30 days of this Decision for Commission review and approval.

## ORDER

### The Commission Orders That:

- An investigatory proceeding is opened to review and approve the Regional Transportation District’s (RTD) hazard analysis H1-06132016 and Corrective Action Plan (CAP) for the Nine Mile Station derailment.

- RTD shall file its CAP and hazard analysis H1-06132016, in this investigation proceeding within 30 days of this Decision.

- This Decision is effective on its Mailed Date.

### ADOPTED IN COMMISSIONERS’ WEEKLY MEETING
August 10, 2016.

| (S E A L) ATTEST: A TRUE COPY Doug Dean, Director | - THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO JOSHUA B. EPEL ________________________________ GLENN A. VAAD ________________________________ FRANCES A. KONCILJA ________________________________ Commissioners |
| --- | --- |