THE

PUBLIC UTILITIES COMMISSION

OF THE

STATE OF COLORADO

(COPUC)

ELECTRIC

INTEGRATED RESOURCE PLANNING

RULES

COPUC 4 CCR 723-21

TABLE OF CONTENTS

Page

BASIS AND PURPOSE	  1

723-21-1.	APPLICABILITY	  2

723 21 2.	DEFINITIONS	  2

723 21 3.	FILING REQUIREMENTS	  5

723 21 4.	PUBLIC PARTICIPATION	  8

723 21 5.	ELECTRIC ENERGY AND DEMAND FORECASTS	 10

723 21 6.	EVALUATION OF EXISTING SUPPLY SIDE RESOURCES AND DEMAND SIDE SAVINGS	 13

723 21 7.	ASSESSMENT OF NEED FOR ADDITIONAL SUPPLY SIDE RESOURCES AND DEMAND SIDE SAVINGS	 20

723 21 8.	REQUEST(S) FOR PROPOSALS	 20

723 21 9.	THE COMPETITIVE RESOURCE ACQUISITION PROCESS.	 22

723 21 10.	COMMISSION REVIEW AND APPROVAL OF INTEGRATED RESOURCE PLANS	 30

723 21 11.	WAIVERS AND VARIANCES	 36

THE

PUBLIC UTILITIES COMMISSION

OF THE

STATE OF COLORADO

(COPUC)

ELECTRIC INTEGRATED RESOURCE PLANNING RULES

COPUC 4 CCR 723-21

BASIS AND PURPOSE.  The purpose of these rules is to establish a process for the development of integrated resource plans (IRPs) and the competitive acquisition of supply-side resources and demand-side savings by Commission jurisdictional electric utilities pursuant to the power to regulate public utilities delegated to the Commission by Article XXV of the Colorado Constitution and by §§ 40-3-102, 40-3-111, and 40-5-103, C.R.S.  This process is intended to benefit the people of Colorado by assuring that Commission jurisdictional electric utilities balance the following interests when planning to meet the current and future electricity needs of the state: 1) the attainment of fair, reasonable and low prices for electricity; 2) the preservation of adequate and reliable electric service; 3) the protection of public health and safety; 4) the preservation of environmental quality and sustainability; 5) the management of risks attributable to electric resource acquisition; 6) the development of diversified electric resource portfolios; 7) the impact of utility decision-making on Colorado and local economies; and 8) the maintenance of utility financial integrity.

COPUC RULE 4 CCR 723-21-1.	APPLICABILITY.

723-21-1	Applicability.  This rule shall apply to all jurisdictional electric utilities in the state of Colorado that are required to obtain a Certificate of Public Convenience and Necessity ("CPCN") from the Commission pursuant to C.R.S. § 40-5-101, or that are subject to the Commission's regulatory authority over rates and charges pursuant to C.R.S. § 40-3-101. Cooperative electric associations engaged in the distribution of electricity which are partially exempt from Commission jurisdiction pursuant to Article 9.5, Title 40, C.R.S. shall be subject only to Rule 3.3, infra.

COPUC RULE 4 CCR 723-21-2.	DEFINITIONS.

723-21-2.1	Availability Factor.  The ratio of the time a generating facility is available to produce energy at its rated capacity, to the total amount of time in the period being measured.

723-21-2.2	Capacity Factor.  The ratio of the net energy produced by a generating facility in a year, to the amount of energy that could have been produced if the facility operated continuously at full capacity year-round.

723-21-2.3	Commission.  Colorado Public Utilities Commission.

723-21-2.4	Demand-Side Savings or Demand-Side

Management.  Increases in energy efficiency, reduced demand or improved load factors resulting from hardware, equipment, devices, or practices that are installed or instituted at a customer facility.  Demand-side management measures can include fuel switching.

723-21-2.5	End-Use.  The light, heat, cooling, refrigeration, motor drive, or other useful work produced by equipment that uses electricity or its substitutes.

723-21-2.6	Energy Efficiency.  The decrease in electricity or gas requirements of participating customers during any selected time period, with end-use services held constant.

723-21-2.7	Heat Rate.  The ratio of energy inputs used by a generating facility expressed in BTUs (British Thermal Units), to the energy output of that facility expressed in kilowatt-hours.

723-21-2.8	Integrated Resource Plan.  A utility IRP consists of an electric demand and energy forecast; an evaluation of existing supply-side resources and demand-side savings; an assessment of need for additional supply-side resources and demand-side savings; and where applicable, the Request(s) for Proposals (RFP) used by the utility to solicit bids for the competitive acquisition of needed supply-side resources and demand-side savings; the utility's proposed resource acquisition and planning period portfolios; and alternative resource acquisition and planning period portfolios modeled by the utility.

723-21-2.9	Net Present Value of Revenue Requirements. The current worth of the expected stream of future revenue requirements associated with a particular resource portfolio.  The net present value of revenue requirements for a particular resource portfolio is obtained by applying a discount rate to the expected stream of future revenue requirements.

723-21-2.10	Planning Period.  The future period for which a utility develops its integrated resource plan.  For purposes of this rule, the planning period is twenty years and begins from the date the utility files its draft IRP with the Commission.

723-21-2.11	Planning Period Portfolio.  The specific combination of supply-side resources and demand-side savings proposed by the utility to meet electric system demand during the twenty-year planning period.

723-21-2.12	Rate Impact.  The impact on electric rates associated with a particular resource portfolio.  Rate impact shall be expressed on an average system-wide basis and for each electric customer rate class.

723-21-2.13	Renewable Resource.  Any facility, technology, measure, plan or action utilizing a renewable "fuel" source such as wind, solar, biomass, geothermal, waste, or small hydro.

723-21-2.14	Resource Acquisition Period.  The first six years of the twenty-year planning period.  The resource acquisition period begins from the date the utility files its draft IRP with the Commission.

723-21-2.15	Resource Acquisition Portfolio.  The specific combination of supply-side resources and demand-side savings the utility proposes to acquire during the six-year resource acquisition period to meet electric system demand.  The resource acquisition portfolio is a subset of the planning period portfolio.

723-21-2.16	Risk.  Potential changes in the net present value of revenue requirements and electric rates associated with a particular resource portfolio which result when the probability distribution associated with various planning assumptions is considered.

723-21-2.17	Supply-Side Resource.  A resource that can provide electrical energy or capacity to the utility. Supply-side resources include utility-owned generating facilities, and energy or capacity purchased from other utilities and non-utilities.

723-21-2.18	Thermal Efficiency.  With respect to supply-side resources, the total amount of electrical and other useful energy output divided by the total energy input.

723-21-2.19	Typical Day Load Pattern.  The electric demand placed on the utility's system for each hour of the day.

COPUC RULE 4 CCR 723-21-3.	FILING REQUIREMENTS.

723-21-3.1	Draft IRP Filing Requirement. Jurisdictional electric utilities, as described in Rule 1, shall file, for planning purposes, a draft IRP pursuant to these Rules on October 31, 1996, and every three years thereafter.  An original and fifteen copies of the draft IRP shall be filed with the Commission.

723-21-3.2	Request to Open the Public Participation

Docket.  Concurrently with the filing of a draft IRP pursuant to Rule 3.1, a utility shall file with the Commission a request to open a docket for the purpose of encouraging public participation in the IRP process and soliciting public commentary concerning resource planning and related resource acquisition issues.  The public participation docket shall be administered pursuant to Rule 4.1.

723-21-3.3	Cooperative Electric Associations.  Whenever cooperative electric associations engaged in the distribution of electricity develop IRPs for other jurisdictional authorities, such cooperative electric associations shall file, pursuant to C.R.S. §§ 40-9.5.-107 and 40-3-110, an original and 15 copies of such an IRP with the Commission no later than 30 days after its submission to such other jurisdictional authority.  The IRPs filed by such cooperative electric associations may be acted upon by the Commission in its sole discretion.

723-21-3.4	Contents of the Draft IRP.  A draft IRP filed pursuant to Rule 3.1 shall contain the following:

723-21-3.4.1	An electric demand and energy forecast developed pursuant to Rules 5.1 through 5.6;

723-21-3.4.2	An evaluation of existing supply-side resources and demand-side savings developed pursuant to Rules 6.1 through 6.9;

723-21-3.4.3	An assessment of need for additional supply-side resources and demand-side savings developed pursuant to Rule 7;

723-21-3.4.4	If the utility determines that it has a need for additional supply-side resources or demand-side savings pursuant to paragraph 723-21-3.4.3 of this subsection, the utility shall provide a statement indicating whether it prefers to acquire such additional supply-side resources and demand-side savings through all-source bidding or some form of segmented bidding and the justification for such preference.  The draft IRP shall include the RFP(s) which the utility proposes to use in order to facilitate the acquisition of additional supply-side resources and demand-side savings.  The RFP(s) shall comply with the requirements specified in Rules 8.1 through 8.4;

723-21-3.4.5	As part of its draft IRP, the utility shall disclose whether it, one of its affiliates, or one of its subsidiaries, desires to submit bids for supply-side resources or demand-side savings it will own and/or operate in the competitive resource acquisition process specified in Rules 9.2 through 9.5.  If this is the case, the utility shall nominate a third-party overseer to perform the functions specified in Rule 9.5, and shall provide the estimated cost to utilize the services of the third-party overseer, as part of its draft IRP.  The third-party overseer shall be paid by the utility, and prudently incurred costs shall be recoverable through rates.  The utility shall file a motion seeking Commission approval of the third-party overseer concurrent with the filing of its draft IRP filing pursuant to Rule 3.1 and its request to open the public participation docket pursuant to Rule 3.2. Intervenors in the public participation process may comment on the appropriateness of the third-party overseer nominated by the utility.  The Commission shall render a decision on the appropriateness of the third-party overseer prior to the issuance of RFP(s) by the utility pursuant to Rule 9.2.1; and

723-21-3.4.6	The document filed by electric utilities for the purpose of providing the information required pursuant to these Rules shall be organized in a manner that specifically references, and responds to, the requirements specified in each individual Rule contained herein.

723-21-3.5	Draft IRP Filing Requirements for Cooperative Electric Generation and Transmission Associations.  Cooperative electric generation and transmission associations shall be required to file only the information specified in Rules 3.4.1 through 3.4.3 as part of their draft IRP filing.  In addition, cooperative electric generation and transmission associations must comply with the public participation requirements specified in Rules 4.1 through 4.1.7.  Cooperative electric generation and transmission associations seeking a CPCN pursuant to C.R.S. § 40-5-101 shall also file proof of compliance with applicable federal requirements governing the competitive solicitation and acquisition of resources pursuant to Rule 10.6, and shall not be required to comply with the requirements of Rules 8.1 through 8.4, and 9.2 through 9.5.4.

COPUC RULE 4 CCR 723-21-4.	PUBLIC PARTICIPATION.

723-21-4.1	The Public Participation Process.

723-21-4.1.1  Concurrently with the filing of its draft IRP pursuant to Rule 3.1, the utility shall file a request to open a public participation docket with the Commission.  Parties shall have 30 days to intervene in this public participation docket.  Immediately upon the filing of an intervention with the Commission, the utility shall provide to such intervening party a copy of the draft IRP.  At the conclusion of the 30 day intervention period, a 60 day public participation process shall commence.  During the initial 30 day notice period, and the subsequent 60 day public participation process, formal discovery shall not be allowed.  Nothing in this Rule is intended to preclude Commission Staff from utilizing the Commission's statutory audit powers at any time, before, during, or after the commencement of the public participation process.

723-21-4.1.2	The utility shall afford all intervenors the opportunity to participate fully in the public participation process.  Unless otherwise ordered by the Commission, the utility or its designee shall chair the public participation process, schedule meetings, and develop agendas for these meetings.  With adequate notice to the utility, intervenors shall be allowed to place items on the agenda of public participation process meetings.

723-21-4.1.3	Meetings held as part of the public participation process shall be scheduled on a periodic basis and shall be open to members of the public, including those who have not intervened in the public participation docket.  Upon request, the utility shall provide an executive summary containing a non-technical description of its draft IRP to members of the public who have not intervened in the public participation docket.

723-21-4.1.4	The purposes of the public participation process are for the utility to provide information to the public regarding the contents of its draft IRP and to solicit public commentary concerning resource planning and related resource acquisition issues.  Topics to be discussed as part of the public participation process include, but are not limited to, the utility's load forecast; the evaluation of existing supply-side resources and demand-side savings; the assessment of need for additional supply-side resources and demand-side savings; the cost and general attributes of potential additional supply-side resources and demand-side savings; and the contents of the RFP(s) associated with the competitive acquisition of additional supply-side resources and demand-side savings, including bid evaluation criteria and the weights to be assigned to each criteria.

723-21-4.1.5	The utility shall have the right to request that the Commission require intervenors in the public participation docket to execute non-disclosure agreements governing the use of proprietary information before disclosing any such information.  The form and content of such non-disclosure agreements shall be consistent with those used in standard Commission practice.

723-21-4.1.6	Upon completion of the 60-day public participation process, the utility shall prepare, and submit to the Commission, a summary of the agendas and discussions held during the public participation process.  Intervenors in the public participation process may, within 45 days of the utility filing, comment on the contents of summary prepared by the utility.

723-21-4.1.7	Upon filing its IRP pursuant to Rule 10.1, the utility shall file a motion requesting the closure of the public participation docket.

COPUC RULE 4 CCR 723-21-5.	ELECTRIC ENERGY AND DEMAND FORECASTS.

723-21-5.1	Forecast Requirements.  The utility shall prepare the following energy and demand forecasts for each year within the planning period:

723-21-5.1.1	Annual sales of energy and coincident summer and winter peak demand disaggregated between Commission jurisdictional sales, FERC jurisdictional sales, and sales subject to the jurisdiction of other states;

723-21-5.1.2	Annual sales of energy and coincident summer and winter peak demand on a system-wide basis for each major customer class;

723-21-5.1.3	Annual energy and capacity sales to other utilities; and capacity sales to other utilities at the time of coincident summer and winter peak demand;

723-21-5.1.4	Annual intra-utility energy and capacity use at the time of coincident summer and winter peak demand;

723-21-5.1.5	Annual system losses and the allocation of such losses to the transmission and distribution components of the system.  Coincident summer and winter peak system losses and the allocation of such losses to the transmission and distribution components of the systems; and

723-21-5.1.6	Typical day load patterns on a system-wide basis for each major customer class.  This information shall be provided for peak-day, average-day, and representative off-peak days for each calendar month.

723-21-5.2	Modeling for Uncertainty.  The utility shall develop a range of forecasts of coincident summer and winter peak demand and energy sales that its system may reasonably be required to serve during the planning period.  The range shall include at least three levels of expected growth in coincident summer and winter peak demand and energy sales, based on alternative, internally-consistent assumptions about the determinants of coincident summer and winter peak demand and energy sales, as follows:

723-21-5.2.1	A base case scenario, which incorporates assumptions that the utility determines to be most likely;

723-21-5.2.2	A high-growth scenario; and

723-21-5.2.3	A low-growth scenario.

723-21-5.3	Required Detail.

723-21-5.3.1	In preparing forecasts, the utility shall develop forecasts of energy sales and coincident summer and winter peak demand for each major customer class.  The utility shall consider end-use, econometric or other supportable methodologies as the basis for these forecasts.  If the utility determines not to use end-use analysis, it shall explain the reason for its determination as well as the rationale for its chosen alternative methodology.

723-21-5.3.2	The utility shall explain the effect on its energy and coincident peak demand forecast of:  1) the demand-side savings and energy efficiency attributable to actions other than the utility's own demand-side management and energy efficiency programs for each major customer class; 2) the effect of the utility's regulated demand-side management and energy efficiency programs on energy sales to each major customer class (subject to the confidentiality provisions of Rule 5.3.3); and, 3) the effect of the utility's unregulated demand-side management and energy efficiency programs on energy sales to each major customer class (subject to the confidentiality provisions of Rule 5.3.3).

723-21-5.3.3	The utility shall maintain, as confidential, information reflecting historic and forecasted demand and energy use for individual customers in those cases when an individual customer is responsible for the majority of the demand and energy used by a particular rate class.  However, when necessary in the IRP proceedings, such information may be disclosed to parties who intervene in accordance with the terms of non-disclosure agreements approved by the Commission and executed by the parties seeking disclosure.

723-21-5.4	Historic Data.  The utility shall compare the annual forecast of coincident summer and winter peak demand and energy sales made by the utility to the actual coincident peak demand and energy sales experienced by the utility for the following three periods:  1) in the most recent IRP filed prior to the year in which the plan under consideration is filed; 2) five years preceding the year in which the plan under consideration is filed; and, 3) ten years preceding the year in which the plan under consideration is filed.

723-21-5.5	Description and Justification.  The utility shall fully explain, justify, and document the data, assumptions, methodologies, models, determinants, and any other inputs upon which it relied to develop its coincident peak demand and energy sales forecasts pursuant to this Rule, as well as the forecasts themselves.  The utility shall specifically discuss how, if at all, the potential impacts of increased electric industry competition and anticipated federal regulatory changes are reflected in its coincident peak demand and energy sales forecasts.  If such considerations are not reflected in the utility's load forecast, the utility shall explain why it deems such consideration unnecessary.

723-21-5.6	Format and Graphical Presentation of Data.  The utility shall include such graphical presentation of the data as shall make the data more understandable to the public, and shall make the data available to requesting parties in such electronic formats as the Commission shall reasonably require.

COPUC RULE 4 CCR 723-21-6.	EVALUATION OF EXISTING SUPPLY-SIDE RESOURCES AND DEMAND-SIDE SAVINGS.

723-21-6.1	Existing Supply-Side Resource Assessment.

723-21-6.1.1	The utility shall compile a comprehensive description of its existing supply-side resources which includes: all utility-owned generating facilities for which the utility has obtained a CPCN from the Commission pursuant to C.R.S. § 40-5-101 at the time the draft IRP is filed; existing or future purchases from other utilities or non-utilities pursuant to agreements effective at the time the plan under consideration is filed, regardless of the duration of such agreements; and existing or future supply-side resources to be obtained through pooling, wheeling and other coordination agreements effective at the time the plan under consideration is filed, regardless of the duration of such coordination agreements.

723-21-6.1.2	Information provided concerning existing supply-side resources shall include, at a minimum:

723-21-6.1.2.1		Name(s) and location(s) of utility-owned generation facilities;

723-21-6.1.2.2		Rated capacity and net dependable capacity of utility-owned generation facilities;

723-21-6.1.2.3		Fuel type, heat rates, capacity factors and availability factors projected for utility-owned generation facilities over the planning period;

723-21-6.1.2.4		Estimated in-service dates for utility-owned generation facilities for which a CPCN has been granted but which are not in-service at the time the plan under consideration is filed;

723-21-6.1.2.5		Estimated remaining useful lives of existing generation facilities;

723-21-6.1.2.6		The amount of capacity and/or energy purchased from utilities and non-utilities, the duration of such purchase contracts and a description of any contract provisions that allow for modification of the amount of capacity and energy purchased pursuant to such contracts; and

723-21-6.1.2.7		The amount of capacity and energy provided pursuant to wheeling or coordination agreements, the duration of such wheeling or coordination agreements, and a description of any contract provisions that allow for modification of the amount of capacity and energy provided pursuant to such wheeling or coordination agreements.

723-21-6.1.3	Utilities required to comply with these Rules shall coordinate their IRP filings such that the amount of electricity purchases and sales between utilities during the planning period is reflected uniformly in their respective IRPs.  Disputes regarding the amount, timing, price, or other terms and conditions of such purchases and sales shall be fully explained in each utility's IRP.

723-21-6.2	Assessment of Existing Demand-Side

Management Measures.

723-21-6.2.1	The utility shall compile a comprehensive description of: 1) demand-side management measures installed at the time the draft IRP under consideration is filed; and 2) demand-side management measures approved by the Commission, but not yet installed at the time the draft IRP under consideration is filed.  Agreements for interruptible service effective at the time the plan under consideration is filed shall also be included as a separately identified category in the description of demand-side management measures.

723-21-6.2.2	Information provided concerning existing demand-side management measures shall include, at a minimum:

723-21-6.2.2.1		The expected remaining useful life of the demand-side management measure;

723-21-6.2.2.2		The energy savings and reductions in coincident summer and winter peak demand made by the demand-side management measure pursuant to the agreement between the utility and the provider of the demand-side management measure; and

723-21-6.2.2.3		The customer class(es) affected by the demand-side management measure.

723-21-6.3	Reserve Margins.  The utility shall provide a description of, and justification for, the means (i.e., probablistic reliability indices as well as planning reserve margins) by which it assesses the desired level of reliability on its system throughout the planning period.  The description will also quantify the recommended or required reliability performance criteria for all groups specified in Rule 6.4.2., as well as power pools, to which the utility is a party.  A comparison, to at least include estimated economic costs or savings due to system resource reserves, between these performance criteria and the means used by the utility to assess reliability shall also be part of this description.

723-21-6.4	Existing Transmission Capabilities and

Future Needs.

723-21-6.4.1	The utility shall report its existing transmission capabilities, and future needs during the planning period, for facilities of 115 kilovolts and above, including associated substations and terminal facilities.  With respect to future needs, the utility shall explain the need for facilities based upon existing supply-side resources, demand-side savings, and load projections.  To the extent reasonably available, the utility shall include a description of the length and location of any additional facilities needed, their estimated costs, terminal points, megawatt rating, alternatives considered or under consideration, and other relevant information.  The utility shall specifically identify the location and extent of transfer capability limitations on its transmission network that may affect the future siting of supply-side resources.

723-21-6.4.2	The utility shall describe all transmission planning or coordination activities to which it is a party, including activities with entities inside and outside of Colorado, including state and regional transmission groups, transmission companies, and coordinating councils with which the utility may be associated.  The utility shall also describe all transmission additions required pursuant to § 211 of the Federal Power Act, any pending actions pursuant to § 211, and all transmission additions required pursuant to other federal open access transmission requirements.

723-21-6.4.3	In order to facilitate the evaluation of demand-side savings alternatives, siting alternatives, and generation alternatives in the planning and siting of additional transmission facilities, the utility shall utilize the information provided in Rules 6.4.1 and 6.4.2 above, as part of the bid evaluation criteria specified in Rule 8.2.

723-21-6.5	Environmental Impacts of Existing

Supply-Side Resources.

723-21-6.5.1	The utility shall provide the percentage of kilowatt-hours generated by each fuel used by the utility on its existing system, for the latest year for which such information is available.  To the extent feasible, the utility shall also estimate the percentage of total kilowatt-hours for which each fuel will be used on its system in each year of the planning period.

723-21-6.5.2	To the extent feasible, for each existing supply-side resource on its system, the utility shall present emission rates (expressed in pounds emitted per kilowatt-hour generated) of the following substances:

723-21-6.5.2.1		Oxides of sulphur;

723-21-6.5.2.2		Oxides of nitrogen;

723-21-6.5.2.3		Carbon dioxide;

723-21-6.5.2.4		Carbon monoxide;

723-21-6.5.2.5		Volatile organic compounds;

723-21-6.5.2.6		Particulate matter, including PM, PM10 and PM2.5; and

723-21-6.5.2.7		Air toxics, as defined in Title III of the Clean Air Act.

723-21-6.5.3	The utility shall present a comprehensive analysis of proposed compliance with Title IV of the Clean Air Act Amendments of 1990 including:

723-21-6.5.3.1		Planned operation levels of all facilities affected pursuant to either Phase I or II of Title IV;

723-21-6.5.3.2		The expected emission reductions required pursuant to both phases of Title IV; and

723-21-6.5.3.3		Whether the utility plans to seek emission credits attributable to its investments in conservation and renewable resources, pursuant to sections 404(f) and 404(g) of Title IV.

723-21-6.5.4	The utility shall also describe the impacts its existing supply-side resources have on land resources (including solid waste disposal) and water resources (including waste water treatment and discharge).

723-21-6.6	Economic Impacts of Existing Resources.  The utility shall describe the impacts its existing supply-side resources and demand-side savings have on the state's economy and employment, including, but not limited to, the effect on the mining, natural gas, energy efficiency, and renewable resource industries.

723-21-6.7	Small Business Impacts of Existing Resources.  The utility shall describe the impacts its existing supply-side resources and demand-side savings will have on small businesses, including but not limited to, the design, sale, supply, installation, or servicing of energy conservation, energy efficiency, and/or other demand-side management measures.

723-21-6.8	Load Building and Load Shaping Programs.  The utility shall provide a detailed description of all existing or planned programs that increase load (including industrial-development and other special rates, efforts to increase wholesale load, and other marketing efforts), including the costs of the programs and their actual or anticipated (in the case of planned programs) effects on energy sales and coincident summer and winter peak demand.  This requirement includes programs that encourage the substitution of the use of electricity for the use of gas, or programs created in response to federal open transmission access requirements.

723-21-6.9	Load Retention Programs.  The utility shall provide a detailed description of all existing or planned programs for load retention, including those intended to prevent bypass through the procedure specified in C.R.S. § 40-3-104.3.

COPUC RULE 4 CCR 723-21-7.	ASSESSMENT OF NEED FOR ADDITIONAL SUPPLY-SIDE RESOURCES AND DEMAND-SIDE SAVINGS.

723-21-7	Need Assessment Procedure.  By comparing the electric energy and demand forecast developed pursuant to Rules 5.1 through 5.6 with the existing level of supply-side resources and demand-side savings developed pursuant to Rules 6.1 through 6.9, the utility shall: 1) assess the need to acquire additional supply-side resources and demand-side savings during the resource acquisition period; and 2) describe the additional supply-side resources and demand-side savings required during the planning period.  For purposes of this Rule, additional supply-side resources shall include both new supply-side resources and modifications to existing supply-side resources.  Additional demand-side savings shall be provided by both new demand-side management measures and modifications to existing demand-side management measures.

COPUC RULE 4 CCR 723-21-8.	REQUEST(S) FOR PROPOSALS.

723-21-8.1	Purpose of the Request(s) for Proposals.  The RFP(s) filed by the utility pursuant to Rule 3.4.4 shall be designed to solicit competitive bids to acquire additional supply-side resources and demand-side savings pursuant to Rules 9.2 through 9.5.4.

723-21-8.2	Contents of the Requests(s) for Proposals - Bid Evaluation Criteria.  The RFP(s) shall contain sufficient information to apprise potential bidders of the utility's proposed criteria for the evaluation of bids received as part of the competitive resource acquisition process specified in Rules 9.2 through 9.5.4.  This information shall include the bid evaluation criteria, including the weights to be assigned to each criterion, that the utility plans to use in ranking the bids received.  Specific information associated with the bid evaluation criteria provided by the utility should include, but not be limited to, the following:

723-21-8.2.1	Preferred fuel types;

723-21-8.2.2	The extent to which additional supply-side resources or demand-side savings must be located in certain geographic areas due to transmission constraints, local load conditions, permitting constraints, or other factors;

723-21-8.2.3	Important transmission constraints on the utility's system and on adjoining utility systems, and reasonable estimates of transmission costs for supply-side resources located in different areas;

723-21-8.2.4	The extent and degree to which supply-side resources must be dispatchable, including the requirement, if any, that supply-side resources be able to operate under automatic dispatch control;

723-21-8.2.5	The customer class or classes from which the utility desires to acquire demand-side savings;

723-21-8.2.6	The methodology proposed by the utility to monitor and evaluate the benefits and costs of demand-side savings and the quantity of demand-side savings sought from each class;

723-21-8.2.7	Supply-side resource reliability requirements and objectives, and the method(s) that will be used to measure their achievement, including the contribution of individual supply-side resources;

723-21-8.2.8	The desirability of firm pricing and of contract terms of various durations;

723-21-8.2.9	Any price cap or cash flow constraints imposed by the utility;

723-21-8.2.10	Any other non-price factor of importance to the utility; and

723-21-8.2.11	The minimum bid evaluation criteria that must be met by a bidder for a bid to be considered responsive to the RFP(s).  The utility shall be reasonable in its specification of minimum bid evaluation criteria and shall not artificially limit the pool of bidders through unreasonable or excessively restrictive minimum criteria.

723-21-8.3	Contents of the Requests(s) for Proposals - Planning Assumptions.  The RFP(s) shall include the planning assumptions proposed by the utility in order to quantify risk as specified in Rules 9.3.8 and 9.4.  These planning assumptions shall include, but not be limited to, the following:  the demand for capacity and energy; general inflation rates; utility cost of capital; discount rates; fuel prices; reserve margins; transmission capacity constraints; the methodology used to account for the cost of compliance with existing environmental and public health and safety regulations; and the methodology used to account for environmental and public health and safety regulations that have been enacted but are not yet in effect.

723-21-8.4	Contents of the Request(s) for Proposals -

Other Information.  The RFP(s) shall also include the utility's proposed standard contract(s) for the acquisition of supply-side resources and demand-side savings.

COPUC RULE 4 CCR 723-21-9.	THE COMPETITIVE RESOURCE ACQUISITION PROCESS.

723-21-9.1	Competitive Resource Acquisition

Requirement.  The utility shall acquire all supply-side resources and demand-side savings including improvements to the utility's existing generation facilities and changes to existing power purchase contracts, pursuant to the procedures specified in Rules 9.2 through 9.5.4.  However, the following supply-side resources and demand-side savings need not be acquired using the competitive resource acquisition procedures specified in these Rules:

723-21-9.1.1	Emergency maintenance or repairs made to utility-owned generation facilities;

723-21-9.1.2	Capacity and/or energy from newly-constructed, utility-owned, supply-side resources with a nameplate rating of less than ten megawatts;

723-21-9.1.3	Capacity and/or energy from the generation facilities of other utilities or from non-utility generators pursuant to agreements for less than a one year term (including renewal terms) or for less than ten megawatts of

capacity;

723-21-9.1.4	Improvements or modifications to existing utility generation facilities that change the  production capability (capacity or energy) of the generation facility site in question, by less than 10 megawatts or 87,600 megawatt-hours per year, based on the utility's share of the total generation facility site output, and that have an estimated cost of less than $10 million.

723-21-9.1.5	New demand-side savings agreements associated with programs approved by the Commission on or before January 1, 1995; and

723-21-9.1.6	Interruptible service provided to the utility's electric customers.

723-21-9.1.7	Non-material modifications to, or amendments of, existing power purchase agreements, including the extension of agreement duration, that add less than 10 MW of incremental capacity to the utility's system and that reduce the system-wide net present value of revenue requirements as calculated over a twenty-year period.  The utility shall carry the burden of proof associated with demonstrating that such modified or amended power purchase agreements comply with the requirements contained in this Rule.

723-21-9.2	Competitive Resource Acquisition Procedures.  The utility shall use the following procedures to competitively acquire all supply-side resources and demand-side savings in which neither the utility, one of its affiliates, nor one of its subsidiaries, are bidders.  If the utility, one of its affiliates, or one of its subsidiaries, desires to submit bids for supply-side resources or demand-side savings it will own and/or operate, it shall adhere to the procedures specified in Rules 9.2 through 9.4.

723-21-9.2.1	Within 30-days of the conclusion of the public participation process pursuant to Rule 4.1.6, the utility shall publish and circulate, pursuant to standard industry practices, an RFP(s) that complies with the requirements of Rules 8.1 through 8.4.  Any person desiring to receive an RFP(s) issued pursuant to this Rule may submit a request to the utility and such person shall be provided with a copy of such requested RFP(s) issued by the utility.

723-21-9.2.2	Thirty to 45 days after publication of the RFP(s), the utility shall notice and convene a bid conference open to all potential bidders.  At the bid conference, the utility shall answer questions posed by bidders concerning the RFP(s).  Written responses to questions asked at the bid conference shall be provided within 10 days of the date of the bid conference at which the question was asked.  Bidders may request the utility to run planning models utilizing assumptions specified by the bidder.  The utility may charge a fee to recover the reasonable costs incurred for each modeling run.  The utility may schedule modeling requests to manage the burden of this requirement, but such scheduling shall not be allowed to disadvantage a bidder.  Bidders may petition the utility to hold more than one bid conference if needed.  However, in no event shall a bid conference be scheduled later than 45 days beyond the publication of the RFP(s) pursuant to Rule 9.2.1.

723-21-9.2.3	At the conclusion of the bid conference(s), the utility shall publish and circulate, pursuant to standard industry practices, the date for submission of sealed bids.  That date shall be no earlier than 30 days following the mailing of answers to questions raised at the last bid conference.  Bids submitted to the utility must be accompanied by an application fee in the amount of $1.00/kw with a minimum application fee of $1,000 and a maximum fee of $25,000.  The reasonable expenses incurred by the utility in conducting the IRP competitive resource acquisition process, less the application fees, shall be recoverable from customers.

723-21-9.2.4	All bidders shall be required to submit a firm price offer for each supply-side resource and demand-side management measure bid.  The firm price may be adjusted, according to specified terms, from year to year, from season to season, or during the hours of the day.  The firm bid shall specify prices for capacity and energy, the anticipated availability factor and capacity factor, and any limitations on the operation of the supply-side resource or demand-side management measure.  Each firm price bid shall specify the period of time for which the quoted bid price is guaranteed and the adjustment factors, if any, to be applied after the passage of the guaranty deadline.  Each bidder may also provide non-firm alternative bids to its firm price bid for each supply-side resource and demand-side management measure that specifically describe the risks that the bidder is passing on to customers of the utility.

723-21-9.2.5	Utility and non-utility bidders must include the cost of required transmission and distribution up-grades, if any, in their bid proposals.  Supply-side resources or demand-side management measures that relieve or delay planned transmission or distribution up-grades may be credited in the bid evaluation for the cost of the up-grades avoided, including avoided transmission and distribution line losses to the extent quantifiable.  These credits may take into account the length of time and the amount of transmission and distribution up-grades avoided.

723-21-9.2.6	After the close of the deadline for the submission of bids specified pursuant to Rule 9.2.3, the utility shall publicly open the sealed bids, and then shall commence a review to determine whether they are responsive to the minimum bid evaluation criteria specified in the RFP(s).  All responsive bids shall be ranked in accordance with the bid evaluation criteria and planning assumptions specified in the RFP(s).  All bids shall be kept confidential by the utility and shall be ranked and modeled on a consistent and uniform basis for all bid evaluation criteria and planning assumptions.  The utility shall not use any bid evaluation criteria to rank bids that are not specified in the RFP(s).  With proper justification, provided pursuant to Rule 9.3.7, the utility may elect to limit the number of bids subject to comprehensive modeling.  If in the course of its evaluation the utility determines that the planning assumptions specified in Rule 8.3 must be altered, all bidders shall be informed, in writing, of the altered planning assumptions and all bidders shall be given at least 30 days to amend their bids.

723-9-3	Contents of the Final IRP. Within 180 days of the conclusion of the public participation process pursuant to Rule 4.1.6, the utility shall file an IRP with the Commission that contains the information specified below.  Copies of the IRP shall be provided, immediately upon its filing with the Commission, to all intervenors in the public participation docket.  When required by the Commission, the utility shall provide work-papers to support the information contained in the IRP.

723-21-9.3.1	An annual electric demand and energy forecast developed pursuant to Rules 5.1 through 5.6;

723-21-9.3.2	An annual evaluation of existing supply-side resources and demand-side savings developed pursuant to Rules 6.1 through 6.9; and

723-21-9.3.3	An assessment of need for additional supply-side resources and demand-side savings developed pursuant to Rule 7.

723-21-9.3.4	The RFP(s) used by the utility to solicit bids for the competitive acquisition of supply-side resources and demand-side savings pursuant to the Rules 8.1 through 8.4.

723-21-9.3.5	A summary of the bids received;

723-21-9.3.6	A description of the process used to model and evaluate bids;

723-21-9.3.7	Where the utility has elected to limit the number of bids subject to comprehensive modeling, it shall provide justification for this course of action; and

723-21.9.3.8	A detailed description of the utility's proposed resource acquisition and planning period portfolios.  This description shall include: 1) the net present value of the revenue requirements for the resource acquisition period and the planning period associated with the portfolios.  In order to quantify risk, the net present value of the revenue requirements for the portfolios shall be calculated and reported for a range of potential outcomes associated with the planning assumptions identified in Rule 8.3; and 2) an analysis of the electric rate impacts associated with the portfolios for each year of the planning period.  In order to quantify risk, the electric rate impacts of the portfolios shall be calculated and reported for a range of potential outcomes associated with the planning assumptions identified in Rule 8.3.

723-9.3.9	  To the degree practicable, the utility shall describe any risks or impacts on the environment, sustainability, and public health and safety attributable to each resource included in its proposed resource acquisition and planning period portfolios.  In addition, the utility shall also describe the impacts the supply-side resources contained in its proposed resource acquisition and planning period portfolios have on land resources (including solid waste disposal) and water resources (including waste water treatment and discharge).

723-21-9.3.10	The utility shall describe the impacts the supply-side resources and demand-side savings contained in its proposed resource acquisition and planning period portfolios, will have on the state's economy and employment, including, but not limited to, the effect on the mining, natural gas, energy efficiency, and renewable resource industries.

723-21-9.3.11	The utility shall describe the impacts the supply-side resources and demand-side savings included in its proposed resource acquisition and planning period portfolios, will have on small businesses engaged in the design, sale, supply, installation, or servicing of energy conservation, energy efficiency, and/or other demand-side management measures.

723-21-9.3.12	The utility shall explain whether current rate designs for each major customer class are consistent with the contents of its IRP.  The utility shall also explain whether possible future changes in rate design will facilitate its proposed resource planning and resource acquisition goals.

723-21-9.3.13  The utility shall describe any steps it is taking to research, develop, demonstrate and commercialize renewable resources and other non-traditional technologies, and explain how such projects fit into its proposed resource acquisition and planning period portfolios.

723-21-9.4	Alternative Portfolio Modeling Requirement.  In addition to the detailed description of the utility's proposed resource acquisition and planning period portfolios, the utility shall perform, and include in its IRP, the modeling required pursuant to Rule 9.3.8 for the following alternative portfolios: 1) the portfolio with the lowest average real rates calculated over the planning period.  Real rates shall be calculated by applying a discount rate to the annual electric rate impacts developed pursuant to Rule 9.3.8; and 2) a diversified portfolio which utilizes increased amounts of DSM and renewable resources as compared to the utility's proposed resource acquisition and planning period portfolios.  The alternative portfolios developed pursuant to this Rule shall be presented in a comparable manner to the utility's proposed resource acquisition and planning period portfolios developed pursuant to Rule 9.3.8 for the range of potential outcomes associated with the planning assumptions identified in Rule 8.3.

723-21-9.5	Special Rules For Utility Bidding in the Competitive Resource Acquisition Process.  If the utility, one of its affiliates, or one of its subsidiaries, desires to submit bids for supply-side resources or demand-side management measures it will own and/or operate, the utility shall inform the Commission of this fact and nominate a third-party overseer upon the filing of its draft IRP pursuant to Rule 3.4.5.  The third-party overseer shall monitor the evaluation and modeling of bids conducted by the utility as part of the competitive resource acquisition process specified in these Rules and shall report its findings to the Commission in an independent and unbiased manner.  All bidders shall be required to submit their bids to the third-party overseer at the time specified for bid submission pursuant to Rule 9.2.3 and all bids shall be opened publicly by the third-party overseer.  All other procedures specified in Rules 9.2 through 9.4 shall be followed including the requirement that the utility file a report with the Commission that complies with Rules 9.3. and 9.4.  The third-party overseer shall file a separate report with the Commission within 180 days from the conclusion of the public participation process pursuant to Rule 4.1.6.  Upon the filing of this report with the Commission, the third-party overseer shall immediately provide copies to the utility and to all intervenors in the IRP and public participation dockets.  The report of the third-party overseer shall contain the following information:

723-21-9.5.1	A determination regarding whether the competitive resource acquisition process was conducted fairly, with any deficiencies specifically reported;

723-21-9.5.2	A description of the process used to model and evaluate bids;

723-21-9.5.3	Where the utility has elected to limit the number of bids subject to comprehensive modeling, a discussion of whether the limitation(s) were justified; and

723-21-9.5.4	A determination regarding whether the resource acquisition and planning period portfolios proposed by the utility pursuant to Rule 9.3.8 best balance the interests described in the Basis and Purpose section of these Rules.  If the third-party overseer disagrees with the resource acquisition and planning period portfolios provided by the utility, the third-party overseer shall specifically explain the items in dispute and, if practicable, provide the Commission with an alternative recommendation based on the alternative portfolios modeled by the utility pursuant to Rule 9.4.

COPUC RULE 4 CCR 723-21-10.	COMMISSION REVIEW AND APPROVAL OF INTEGRATED RESOURCE PLANS.

723-21-10.1	Review on the Merits.   The utility's IRP, as developed pursuant to Rules 9.3 and 9.4 will be filed in the form of an application administered pursuant to the Commission's Rules of Practice and Procedure.  The Commission may hold a hearing for the purpose of reviewing and rendering a decision regarding the contents of the utility's IRP upon its filing.

723-21-10.2	Basis for Commission Decision.  Based upon the evidence of record, the Commission shall issue a written decision approving, disapproving, or suggesting modifications, in whole or in part to the utility's IRP.  The Commission's decision shall be based on whether the IRP is consistent with, and appropriately balances, the interests identified in the Basis and Purpose section of these Rules and is otherwise in the public interest.  If the Commission declines to approve an IRP, either in whole or in part, the utility shall make changes to the IRP in response to the Commission's decision.  Within 30 days of the Commission's rejection of an IRP, the utility shall file an amended IRP application with the Commission, and provide copies to the parties.  All parties may participate in any hearings ordered regarding the amended IRP application.

723-21-10.3	Contents of the Commission IRP Decision.  The Commission IRP decision shall address the contents of the utility's IRP filed in accordance with Rules 9.3 and 9.4.  The Commission shall specifically render findings on the items described below.  If the Commission declines to approve any of these elements, the utility shall refile an amended IRP pursuant to Rule 10.2.

723-21-10.3.1	The electric demand and energy forecast developed pursuant to Rules 5.1 through 5.6;

723-21-10.3.2	The evaluation of existing supply-side resources and demand-side savings developed pursuant to Rules 6.1 through 6.9;

723-21-10.3.3	The assessment of need for additional supply-side resources and demand-side savings developed pursuant to Rule 7;

723-21-10.3.4	The adequacy of the contents of the RFP(s) used by the utility to solicit bids for the competitive acquisition of the supply-side resources and demand-side savings pursuant to Rules 8.1 through 8.4.

723-21-10.3.5	The utility's proposed resource acquisition and planning period portfolios provided pursuant to Rule 9.3.8.  The Commission may elect to approve an alternative to the utility's proposed resource acquisition and planning period portfolios based on: 1) the information contained in the report submitted by the utility pursuant to Rules 9.3 and 9.4; 2) the information contained in the report submitted by the third-party overseer pursuant to Rule 9.5 (if applicable); and 3) the rate impacts associated with acquiring various amounts of specific supply-side resource technologies and demand-side savings.

723-21-10.4	Cooperative Electric Generation and Transmission Associations.  IRP Applications.  Cooperative electric generation and transmission associations shall file an IRP application with the Commission within 180 days from the conclusion of the public participation process pursuant to Rule 4.1.6.  The IRP application shall include the following items:

723-21-10.4.1	The electric demand and energy forecast developed pursuant to Rules 5.1 through 5.6;

723-21-10.4.2	The evaluation of existing supply-side resources and demand-side savings developed pursuant to Rules 6.1 through 6.9; and

723-21-10.4.3	The assessment of need for additional supply-side resources and demand-side savings developed pursuant to Rule 7.

The Commission may hold a hearing for the purpose of reviewing and rendering a decision regarding the contents of the utility's IRP upon filing.

723-21-10.5	Commission Approval of Specific Supply-Side

Resources.

723-21-10.5.1	Utility-owned supply-side resources contained in the resource acquisition portfolio approved by the Commission pursuant to Rule 10.3.5 shall be granted a CPCN pursuant to C.R.S. § 40-5-101 if construction activity associated with such supply-side resources is scheduled to commence within two years from the filing of the utility IRP application specified in Rule 10.1.  If the CPCN is approved, the utility may not recover from ratepayers any amounts in excess of those specified in the successful bid.

723-21-10.5.2	Utility-owned supply-side resources contained in the resource acquisition portfolio approved by the Commission pursuant to Rule 10.3.5, for which construction activity is not scheduled to commence within two years, shall not be granted a CPCN as specified in Rule 10.5.1.  Commission approval of the resource acquisition portfolio pursuant to Rule 10.3.5 shall have the following effect on supply-side resources contained in that portfolio for which construction activity is not scheduled to commence within two years:

723-21-10.5.2.1	At the time of a proceeding on a utility's request to the Commission for recovery of investment and expenses incurred or approval of certain actions taken pursuant to the conditions set forth in its resource acquisition portfolio, whether in an advice letter, application, or certificate proceeding, the utility has the burden of going forward to submit prima facie evidence that its actions are or were consistent with the resource acquisition portfolio approved by the Commission.

723-21-10.5.2.2	Upon the utility meeting its initial burden of presenting prima facie evidence of action consistent with the approved resource acquisition portfolio, the burden of going forward shall shift to intervenors who wish to challenge the propriety of the utility's actions.

723-21-10.5.2.3	In order to overcome the utility prima facie evidence, an intervenor must present evidence that the utility's actions are not or were not consistent with the approved resource acquisition portfolio, or that due to changed circumstances or other factors, the utility's actions are or were improper, not in the public interest, or resulted or will result in the imposition of unjust or unreasonable rates.

723-21-10.5.2.4	Upon a showing by an intervenor of the matters set forth in subparagraph 723-21-10.5.2.3 of this subsection, the utility shall have the ultimate burden of proof to show that its actions are or were in compliance with its last approved resource acquisition portfolio and are or were appropriate and in the public interest.

723-21-10.5.2.5	The utility proposing certain actions which are inconsistent with its approved resource acquisition portfolio shall have the burden of explaining the reasons for the inconsistency and shall demonstrate that its proposed action is in the public interest.

723-21-10.5.2.6	The approval of the utility's resource acquisition portfolio pursuant to Rule 10.3.5 shall not preclude an independent finding by the Commission which differs from the utility's approved resource acquisition portfolio in a subsequent proceeding in which the utility seeks the recovery of revenue for actions taken consistent with its resource acquisition portfolio (e.g., rate case review, or application for cost recovery) pursuant to C.R.S. § 40-3-101 et seq., or the granting of a certificate of public convenience and necessity as required by C.R.S. § 40-5-101 et seq.

723-21-10.5.3	In order to insure that the final contracts for capacity and energy provided by third-party suppliers contained in the Commission-approved resource acquisition portfolio are consistent with the RFP(s) approved by the Commission pursuant to Rule 10.3.4, the Commission, in its sole discretion, may require that such contracts be submitted for Commission approval by the utility in the form of an application to be administered pursuant to the Commission's Rules of Practice and Procedure.

723-21-10.6	Commission Approval of Specific Supply-Side Resources by Cooperative Electric Generation and Transmission Associations.  Cooperative electric generation and transmission associations seeking a CPCN shall file proof of compliance with applicable federal requirements governing the competitive solicitation and acquisition of resources as part of their CPCN application.  The utility shall demonstrate that such requirements are an adequate substitute for the competitive resource acquisition procedures contained in Rules 8.1 through 8.4 and 9.1 through 9.5.4.

723-21-10.7  Commission Approval of Supply-Side Resources and Demand-Side Savings Outside the Competitive Resource Acquisition Process.  Utility expenditures made for the acquisition of additional supply-side resources and demand-side savings exempt from this Rule 9 are subject to Commission review for prudency.

723-21-10.8	Annual Progress Reports.  The utility shall file with the Commission, and provide copies to all parties to the most recent IRP docket, annual progress reports after submission of its IRP application.  The annual progress reports will inform the Commission of the utility's efforts to acquire the resources contained in the resource acquisition portfolio approved by the Commission pursuant to Rule 10.3.5.  An annual progress report shall not constitute an amendment to an approved IRP.  If the utility desires to amend its approved IRP, or its Commission-approved resource acquisition portfolio, it shall file an application to do so pursuant to the Commission's Rules of Practice and Procedure.  Interested parties can file comments with the Commission in response to annual progress reports filed by the utility.  The Commission may, in its sole discretion, initiate a proceeding or such other action it deems necessary in response to the information contained in an annual progress report.  Annual progress reports will also contain:

723-21-10.8.1	An annual electric demand and energy forecast developed pursuant to Rules 5.1 through 5.6;

723-21-10.8.2	An annual evaluation of existing supply-side resources and demand-side savings developed pursuant to Rules 6.1 through 6.11; and

723-21-10.8.3	An assessment of need for additional supply-side resources and demand-side savings developed pursuant to Rule 7.

723-21-10.8.4	An itemization of supply-side resources and demand-side savings acquired through the utilization of the competitive bidding exemptions pursuant to Rules 9.1.1 through 9.1.7.

723.21.10.8.5	Cooperative electric generation and transmission associations shall file annual progress reports containing the information required by Rules 10.8.1 through 10.8.3.

723-21-10.9	Amendment of an Approved IRP.  The utility may, at any time, file an application to amend the contents of an IRP approved pursuant to Rule 10.3 or 10.4.  Such an application shall be administered pursuant to the Commission's Rules of Practice and Procedure found at 4 CCR 723-1.  Any material deviation from the contents of an IRP application approved by the Commission pursuant to Rule 9.3 will require the filing of an application to amend as specified herein.

723-21-10.10	Resource Acquisition Outside the IRP

Competitive Resource Acquisition Process.  If a utility seeks to acquire supply-side resources or demand-side savings outside the IRP competitive resource acquisition process specified in Rules 9.2 through 9.5, it shall request a waiver pursuant to Rule 11.

COPUC RULE 4 CCR 723-21-11.	WAIVERS AND VARIANCES.

723-21-11.	General Waivers and Variances.  A utility may file an application for a waiver or variance from a provision of this Rule.  Any such application shall demonstrate the basis of the utility's contention that it should be granted a waiver or variance, including documentation regarding the costs and benefits of compliance.  For good cause shown, and if not contrary to law, the Commission may grant a waiver or variance if the Commission finds that compliance with such provision is impracticable or unreasonable.

G:\adpt\95R071E.RLS