THE
PUBLIC UTILITIES COMMISSION
OF THE
STATE OF COLORADO

RULES REGULATING 
TELECOMMUNICATIONS SERVICE PROVIDERS 
AND TELEPHONE UTILITIES

4 CODE OF COLORADO REGULATIONS (CCR) 723-2

. . . . . . . . .

RULE (4 ccr) 723-2-18.  CUSTOMER ACCESS LINES.

723-2-18.1	LECS shall construct and maintain all Local Access Lines used for Individual Line Service so that the following general parameters and characteristics are met:

723-2-18.1.1	Bandwidth for Voice Grade Access should be, at a minimum, 300 to 3,000 Hertz (see Rule 2-2.51).  The transmission insertion loss, as measured at the interface with the LEC network at the customer's location and including any losses in central office equipment, does not exceed 8.5 dB at 1004 + 20 Hertz (Hz).  All local access lines used for party line service shall be maintained so that the measured insertion loss under the previously described condition does not exceed 10 dB.  The frequency response performance of the Local Access Line (defined as the loss deviation from the reference frequency of 1004 Hz) shall not exceed -1 dB to +3.5 dB at 300 Hz nor -1 dB to +3.5 dB at 3,000 Hz.

723-2-18.1.2	Local Access Line current (defined as the metallic direct current flowing in the circuit at the customer interface during the talk and signaling states or during the idle line state) shall be greater than 20 milliamps (ma).

723-2-18.1.3	Power Influence, as measured as C-weighted Longitudinal Noise, shall be no greater than 80 dBrnC.

723-2-18.1.4	Longitudinal Balance (Circuit Balance), as measured as the difference between Power Influence (dBrnC) and Circuit Noise (dBrnC), shall be greater than 60 dB.

723-2-18.1.5	In addition, Local Access Lines used for Individual Line Service of less than 30,000 feet in length shall be constructed and maintained so that a measure of the message circuit noise from the network interface at the customer's premises to and including the central office termination shall not exceed 205 dBrnC.  All other Local Access Lines shall be maintained so that the measured message circuit noise does not exceed 30 dBrnC.  Circuit noise, as measured using the “3 KHz flat” weighting, shall not exceed 40 dBrn3 kHz on Local Access Lines of less than 30,000 ft. nor greater than 60 dBrn3 kHz on Local Access Lines of greater length.

723-2-18.2	The LEC shall routinely test each Local Access Line, and the records of these test results shall be  maintained pursuant to Rule 2-6.4.  This information shall be provided to the Commission upon request.

G:\YELLOW\input.doc