# Decision No. C15-0687

BEFORE THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO

PROCEEDING NO. 15A-0337R

IN THE MATTER OF THE APPLICATION OF THE TOWN OF WINDSOR AND THE COLORADO DEPARTMENT OF TRANSPORTATION FOR AUTHORITY TO ADD RAILROAD ENTRANCE AND EXIT GATES TO THE CROSSING OF THE GREAT WESTERN RAILWAY TRACKS AT EASTMAN PARK DRIVE IN THE TOWN OF WINDSOR, COUNTY OF WELD, COLORADO.

commission decision 
granting application

Mailed Date:  	July 16, 2015

Adopted Date: 	July 8, 2015

## by the commission

### Statement

- This matter comes before the Commission for consideration of an application (Application) filed by the Town of Windsor (Windsor) on May 14, 2015, requesting authority to install active warning equipment including flashing lights, four-quadrant gates, bells, constant warning time detection circuitry, new cabin, modifications to existing raised curb islands, vehicle detector loops, pavement markings, and associated W10-1 advance warning signs; and retaining existing interconnection and preemption at the existing at-grade highway-rail crossing of Eastman Park Drive with the tracks of the Great Western Railway (GWR), National Inventory No. 871917X, in Windsor, County of Weld, State of Colorado.

- The Commission gave notice of this Application (Notice) to all interested parties, including adjacent property owners pursuant to § 40-6-108(2), C.R.S.  The Notice was mailed May 18, 2015.

- On June 16, 2015, GWR filed an Entry of Appearance and Unopposed Motion for Extension of Time to Intervene (Motion).

- On June 29, 2015, GWR filed a Notice of Intervention.  GWR does not contest or oppose the Application.

- The Commission deemed the Application complete and granted the Motion by Decision No. C15-0622-I mailed July 2, 2015.

- Now being fully advised in the matter, we grant the Application.

### Findings of Fact

- The Commission gave notice to all interested parties, including the adjacent property owners.  No intervention was received opposing the Application.

- Windsor is requesting authority to install active warning at the crossing of Eastman Park Drive with the tracks of the GWR including flashing lights, four-quadrant gates, bells, constant warning time detection circuitry, new cabin, modifications to existing raised curb islands, vehicle detector loops, and retaining existing interconnection and preemption at the existing crossing.  Additionally, Windsor proposes to install railroad crossing pavement markings and W10-1 advance warning signs.  Windsor proposes to operate the four-quadrant gate system using timed exit gates with a delay between the start of the descent of the entrance gates and the start of the descent of the exit gates and with vehicle detection loops to hold the gates up in lanes where there is potential for vehicular traffic to queue onto the tracks.  Windsor proposes a 
14.5-second exit gate delay time, which would allow a 75 foot design vehicle just starting to enter the crossing from a stopped position to move through the crossing before the exit gate starts to descend unless the exit gate is held up by detection of a vehicle in the crossing.

- Windsor proposes that 35 seconds of total preemption time be provided at the crossing with 14 seconds of advance preemption time provided to the traffic signal controller.

- Windsor seeks this authority because it desires to obtain a quiet zone at this crossing from the Federal Railroad Administration (FRA).  When Windsor obtains the quiet zone, Windsor would post W10-9P “No Train Horn” placard signs with the W10-1 advance warning signs.

- Windsor states that there are currently two trains per day that use the crossing at a maximum timetable speed of 35 miles per hour (MPH) with no estimates of increases or decreases in these volumes in the future.  There are currently 7,400 vehicles per day (VPD) using the crossing at a posted speed limit of 45 MPH with traffic expected to increase to 8,900 VPD by 2030.

- Windsor estimates the cost of the proposed crossing changes at $437,547.  The costs of the project will be paid for through an FRA TIGER V grant.  Any costs greater than those covered by the TIGER V grant will be funded by Windsor.

- Windsor proposes starting construction in July 2015 with work being complete by February 2016.  We will require that Windsor file a copy of the signed Construction and Maintenance Agreement for this project by July 31, 2015 prior to starting work at the crossing.

- Windsor will be required to inform the Commission in writing when the crossing changes are complete and operational within ten days of completion.  The Commission will expect this letter by February 28, 2016.  However, the Commission does understand this letter may be provided earlier or later than this date depending on changes or delays to the construction schedule.

- We will require Windsor to update the crossing inventory form for the changed crossing conditions and to file a copy of the updated crossing inventory form with the Commission with completion of the crossing work by February 28, 2016.

### Conclusions

- The Commission has jurisdiction in this matter under §§ 40-4-106(2)(a) and (3)(a), C.R.S.

- No intervenor that filed a petition to intervene or other pleading contests or opposes the Application.

- Because the Application is unopposed, the Commission finds that it will determine this matter upon the record, without a formal hearing under § 40-6-109(5), C.R.S., and Rule 1403, Commission Rules of Practice and Procedure, 4 Code of Colorado Regulations 723-1.

- We find that good cause exists and that the requirements of public safety are met by granting the Application consistent with the above discussion.

## ORDER

### The Commission Orders That:

- The application filed by the Town of Windsor (Windsor) on May 14, 2015, requesting authority to install active warning equipment including flashing lights, four-quadrant gates, bells, constant warning time detection circuitry, new cabin, modifications to existing raised curb islands, vehicle detector loops, pavement markings, and associated W10-1 advance warning signs; and retaining existing interconnection and preemption at the existing at-grade highway-rail crossing of Eastman Park Drive with the tracks of the Great Western Railway (GWR), National Inventory No. 871917X, in Windsor, County of Weld, State of Colorado is granted.

- The intervention by GWR is granted.

- Windsor is authorized and ordered to proceed with the installation of active warning equipment consisting of flashing lights, four-quadrant gates, bells, constant warning time detection circuitry, new cabin, modifications to existing raised curb islands, vehicle detector loops, pavement markings and associated W10-1 advance warning signs at the crossing of Eastman Park Drive with the GWR in Windsor, Colorado.  The four-quadrant gate system will operate with timed exit gates with an exit gate delay time of 14.5 seconds with exit gates being held open if vehicles are detected by the detection loops.  A total preemption time of 35 seconds will be provided including 14 seconds of advance preemption time will be provided to the traffic signal controller.

- Windsor is required to file a signed copy of the Construction and Maintenance Agreement by July 30, 2015 with construction work not expected to begin until these signed agreements are filed.

- Windsor is required to inform the Commission in writing that the crossing changes are complete and operational within ten days after completion.  We shall expect this letter by February 28, 2016.  However, we understand this letter may be provided earlier or later than this date depending on changes or delays to the construction schedule.

- Windsor will be required to update the crossing inventory form showing the changed crossing conditions and filing a copy of that updated inventory form in this proceeding at the end of the project by February 28, 2016.

- The 20-day period provided for in § 40-6-114, C.R.S., within which to file applications for rehearing, reargument, or reconsideration, begins on the first day following the effective date of this Decision.

- The Commission retains jurisdiction to enter further decisions as necessary.

- This Decision is effective on its Mailed Date.

### ADOPTED IN COMMISSIONERS’ WEEKLY MEETING
July 8, 2015.

| (S E A L) ATTEST: A TRUE COPY Doug Dean, Director | - THE PUBLIC UTILITIES COMMISSION OF THE STATE OF COLORADO JOSHUA B. EPEL ________________________________ PAMELA J. PATTON ________________________________ - GLENN A. VAAD ________________________________ - Commissioners |
| --- | --- |